x

MATERIAL SAFETY DATA SHEET

AIRWICK INDUSTRIES, INC. CARLSTADT, NEW JERSEY 07072

Emergency Telephone Number.

Date:	September,   1985

Supersedes:       January,   1985

201-933-8200

| TRADE NAME A-33 LIQUID |  |  |  |
| --- | --- | --- | --- |
| PRODUCT TYPE Detergent/Disinfectant/Odor Counteractant. |  |  |  |
| 1. PHYSICAL DATA |  |  |  |
| BOIUNG POINT (°F) | 212°F | FREEZING POINT (°F) | 31°F |
| SPECIFIC GRAVITY (H2O«1) | 1.0353 | SOLUBILITY IN WATER, % by wt. at 20°C | Complete |
| PERCENT VOLATILES BY VOLUME | ""89% | pH , f i 1: 64 | 10.0-11.5 |
| APPEARANCE AND ODOR | Green liauid with non-descript odor |  |  |
| OTHER | N/A * |  |  |

II. HAZARDOUS INGREDIENTS

| CHEMICAL OR COMMON NAME | CAS# | % | PEL OR TLV | PRIMARY HAZARD |
| --- | --- | --- | --- | --- |
| Quaternary ammonium chlorides-n-alkyl (60Z Cu> 30% Cie, | 68391-01-5 | 3.0 | N/A | Corrosive |
| 5% C12. 5Z C18.) dimethyl benzyl ammonium chlorides |  | • |  |  |
| • | * |  |  |  |

III. FIRE AND EXPLOSION HAZARD DATA

| FLASH POINT (°F) (test method) | iHl^nrn FLAME EXTENS.ON N/A |
| --- | --- |
| FLAMMABLE LIMITS IN AIR. °/o by volume LOWER N/A UPPER N/A |  |
| EXTINGUISHING MEDIA | Non-flammable; Select media that is suitable for surrounding |
| SPECIAL FIRE FIGHTING PROCEDURES | Npne |
| UNUSUAL FIRE AND EXPLOSION HAZARDS | . None |

*N/A=NOT APPLICABLE

IV. HEALTH HAZARD DATA

EFFECTS OF OVEREXPOSURE

Ingestion: Local gastrointestinal irritation with nausea, vomiting, diarrhea, convulsions and respiratory distress. Skin: Moderate irritant. Eyes: Severe irritant.

EMERGENCY AND FIRST AID PROCEDURES

-

Ingestion: Give water. Induce vomiting.

Skin: Wash with soap and water.

Eyes: Flush with water for 15 minutes. Call physician.

•

V.	REACTIVITY DATA

STABILITY        Unstable   Q       Conditions

Stable      E      To Avoid     None

INCOMPATIBILITY

(Materials To Avoid)	Anionic soap  and detergents.

HAZARDOUS DECOMPOSITION

PRODUCTS	Nsne

_	May Occur         D       Conditions

ZATION         Will Not Occur   El       To Avoid	None

VI.	SPILL OR LEAK PROCEDURES

| STEPS TO BE TAKEN IF MATERIAL IS RELEASED OR SPILLED | Pick up excess with' absorbent-type the remaining with water. | material and flush |
| --- | --- | --- |
| WASTE DISPOSAL METHOD | Discard according to local, state and label directions. | and federal regulations |

VII. SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION

(Specify Type)	N/A

VENTILATION                   Local            M/4          Mechanical                   Special    „,.                Other      „,.
	 Exhaust        N/A         (General)Sufficient	N/A	N/A

PROTECTIVE	EYE

GLOVES	Gloves recommended	PROTECTION	Goggles recommended

OTHER PROTECTIVE

CLOTHING OR EQUIPMENT	N/A

VIII. SPECIAL PRECAUTIONS

PRECAUTIONS TO BE TAKEN Store at ambient temperature. Keep container tightly closed when IN HANDLING AND STORAGE not in use>  Do not freeze.  Do not reuse empty container.

OTHER        N/A

PLEASE NOTE:  BEFORE USING THIS OR ANY PRODUCT, READ AND FOLLOW THE DIRECTIONS AND PRECAUTIONS ON T' "= PRODUCT LABEL.