MATERIAL

r

A,.

SAFETY

DATA

SHEET

The information contained herein is based on data available at the time of preparation of this data sheet and which The Gltdden Company believes to be reliable. However., no warranty is expressed or implied regarding the accuracy of this data. The Glidden Company shall not be responsible for the use of this information, or of any product, method or apparatus mentioned and you must make your own determination of its suitability and completeness for your own use, for the protection of the environment, and the health and safety of your employees and users of this material.

Complies with OSHA hazard communication standard 29CFR1910.1200.

| 1225 Series, 86, HMIS Rating | 87 |
| --- | --- |
| Health | 2 |
| Flammability | 4 |
| Reactivity | 0 |

1245 HMIS Rating

Health	*3

Flammability	4

Reactivity	0

'See Health Hazard Data for chronic health information.

Prepared November, 1988

THE GLIDDEN COMPANY

925 Euclid Avenue Cleveland. Ohio 44115

Emergency Telephone No. (216)826-5566

)

f

a

RUSTMASTER® Enamel No. 1225 Series; WOODMASTER® Polyurethane Nos. 86,87

FIRE & EXPLOSION HAZARD DATA

DOT Proper Shipping Name: Consumer Commodity

Hazard Class: ORM-D

Extinguishing Media: Dry chemical or foam

Unusual Fire and Explosion Hazards: Do not spray near open flame. Keep at room temperature as exposure to direct sunlight or other heat may cause bursting.

Special Fire Fighting Procedures: Water may be used to cool and protect exposed

containers.

HEALTH HAZARD DATA

Primary Route(s) of Exposure: Inhalation

Skin Contact

Effects of Overexposure:

Inhalation: Irritation of respiratory tract. Prolonged inhalation may lead to fatigue, drowsiness, dizziness and/or lightheadedness, headache, uncoordination, nausea, vomiting, coughing, central nervous system depression, respiratory problems, intoxication, confusion, anesthetic effect or narcosis, difficulty of breathing.

Skin Contact: I rritation of skin. Prolonged or repeated contact can cause dermatitis, defatting. May be absorbed through skin.

Eye Contact: I rritation of eyes. Prolonged or repeated contact can cause blurred vision, tearing of eyes, redness of eyes, severe eye irritation.

Ingestion: Amounts ingested incidental to consumer and industrial handling are not likeiy to cause injury; however, ingestion of larger amounts may cause lung inflammation and damage due to aspiration of material into lungs.

Notice: Reports have associated repeated and prolonged occupational over-exposure to solvents with permanent brain and nervous system damage. Intentional misuse by deliberately concentrating and inhaling the contents may be harmful or fatal.

Emergency and First Aid Procedures:

Inhalation: Remove to fresh air. Restore and support continued breathing. Get

emergency medical attention. Have trained person give oxygen if necessary.

Get medical help for any breathing difficulty. Skin Contact: Wash off quickly with plenty of water, then soap and water;

remove contaminated clothing. Wash contaminated clothing before reuse. Eye Contact: Flush immediately with large amounts of water, especially under

lids, for at least 1 5 minutes. Obtain emergency medical treatment. Ingestion: If swallowed, obtain medical treatment immediately.

Medical Conditions Aggravated By Exposure: This product is not expected to aggravate existing medical conditions; however, ingredients contained in this product have been reported to aggravate preexisting eye, skin, respiratory disorders, lung disorders, asthma-like conditions, cardiac abnormalities, kidney disorders, liver disorders, nervous system disorders.

Supplemental Health Information: This product contains crystalline silica, which is considered a hazard by inhalation. The International Agency for Research on Cancer (IARC) has determined that there is sufficient evidence for the car-cinogenicity of crystalline silica to experimental animals, and limited evidence for the carcinogenicity of crystalline silica to humans. Crystalline silica is also a known cause of silicosis, a noncancerous lung disease. Exposure to materials in this product have been associated with possible cardiac abnormalities, blood abnormalities, liver damage, kidney damage. Excessive inhalation of solvent vapors under uncontrolled conditions may lead to unconsciousness, respiratory failure, asphyxiation, and even death.

)

REACTIVITY DATA

Stability: Stable

Incompatibility: Oxidizers Acids Bases

Aldehydes Alkylhalides Halogens Amines

Conditions to Avoid: Elevated temperatures

Contact with oxidizing agent

Hazardous Decomposition Products: Carbon Monoxide

Carbon Dioxide

Hazardous Polymerization: Will not occur

SPILL OR LEAK PROCEDURES

Steps to be taken in case material is released or spilled: Comply with all applicable

health and environmental regulations.

Eliminate all sources of ignition.

Ventilate area.

Spills may be collected with non-combustible absorbent materials.

Waste Disposal: Dispose in accordance with all applicable regulations. Avoid discharge to natural waters. Do not incinerate closed containers.

SPECIAL PROTECTION INFORMATION

Respiratory Protection: Control environmental concentrations below applicable standards. Where respiratory protection is required, use only NIOSH MSHA approved respirators in accordance with OSHA Standard 29 CFR 1 91 0.1 34.

Ventilation: Provide dilution ventilation or local exhaust to prevent build-up of vapors.

Personal Protective Equipment: Eye Wash

Safety Shower

Safety Glasses or Goggles

Impervious Gloves

SPECIAL PRECAUTIONS

Handling and Storage: Store below 1 00°F. Keep containers closed and away from heat and open flame or storage in direct sunlight.

Other Precautions: Use only with adequate ventilation. Do not take internally. Keep out of reach of children. Avoid contact with skin and eyes, and breathing of vapors. Wash hands thoroughly after handling, especially before eating or smoking. Keep containers tightly closed and upright when not in use. Avoid conditions which result in formation of inhalable particles such as spraying or abrading (sanding) painted surfaces. If such conditions cannot be avoided, use appropriate respiratory protection as directed under Special Protection Information. Empty containers may contain hazardous residue. Do not puncture or incinerate containers. Do not spray near open flame or fire.