MATERIAL SAFETY DATA SHEET

Revised: 16/07/04

PHENOLPHTHALEIN, 0.5%	Replaces: 09/05/00

CAROLINA BIOLOGICAL	Printed: 20/05/08

1.	PRODUCT DESCRIPTION

Product Name:      Phenolphthalein, 0.5%

Product Code(s):   87-9963, 87-9965, 84-0935

Size:	120 ml, 500 ml

Chemical Name:     Chemical is mixture.

CAS Number:        None assigned to this mixture

Formula:	See section 2

Synonyms:         None known

Distributor:       Carolina Biological Supply Company

2700 York Road

Burlington, NC 27215

Chemical Information:  800-227-1150 (8am-5pm (ET) M-F) Chemtrec (Transportation Spill Response 24 hours): 800-424-9300

2.	COMPOSITION/INFORMATION ON JNGREDIENTS

Principle Hazardous Components:

Ethyl alcohol (CAS#64-17-5)-48%

Methyl isobutyl ketone(Denaturant)(CAS#108-10-1)-2% Hexane(Denaturant)CAS#110-54-3)-1% TLV and PEL units: Ethyl alcohol- ACGIH-TLV -1000 ppm(TWA)

OSHA-PEL -1000 ppm(TWA)

Methyl isobutyl ketone-ACGIH-TLV- 50 ppm(TWA),STEL-75 ppm

OSHA-PEL -50 ppm(TWA), STEL-75 ppm Phenolphthalein- None established Hexane- ACGIH-TLV -50 ppm(TWA) OSHA-PEL - 50 ppm(TWA)

3.	HAZARD IDENTIFICATION

Emergency Overview:  Clear colorless liquid with characteristic alcohol

odor.

May cause irritation to eyes and skin.

Potential Health Effects:

Eyes:  May cause irritation.

Skin:  May cause irritation.

Ingestion:  May cause gastrointestinal discomfort.

Inhalation:  May cause irritation to respiratory tract.

4.	FIRST AID MEASURES

Emergency and First Aid Procedures:

Eyes - Flush with water for at least 15 minutes, raising and lowering

eyelids occasionally.  Get medical attention if irritation persists.

Skin - Thoroughly wash exposed area for at least 15 minutes.  Remove

contaminated clothing.  Launder contaminated clothing before reuse.  Get

medical attention if irritation persists.

Ingestion - If swallowed, if conscious, give plenty of water.

Immediately call a physician or poison control center.  Never give

anything by mouth to an unconscious person.

Inhalation - Remove to fresh air.  Give oxygen if breathing is

difficult; give artificial respiration if breathing has stopped.  Keep

person warm and quiet, and get medical attention.

5.	FIREFIGHTING PROCEDURES

Flash Point(Method Used):  54F NFPA Rating:      Health:  2

Page 1 of 4

MATERIAL SAFETY DATA SHEET

PHENOLPHTHALEIN, 0.5% CAROLINA BIOLOGICAL

Revised: 16/07/04 Replaces: 09/05/00 Printed: 20/05/08

Fire:  3

Reactivity:  0 Extinguisher Media:

Use media suitable to extinguish surrounding fire.

Flammable Limits in Air % by Volume:  Ethyl alcohol:LEL-3.3%,UEL-19% Autoignition Temperature:  Ethyl alcohol - 793F Special Firefighting Procedures:

Firefighters should wear full protective equipment and NIOSH approved self-contained breathing apparatus.

Unusual Fire and Explosion Hazards: Vapors are heavier than air and can travel to a source of ignition and flash back.

6.	SPILL OR LEAK PROCEDURES

Steps to be Taken in Case Material is Released or Spilled:

Ventilate area of spill.  Eliminate all sources of ignition. Remove all non-essential personnel from area.  Clean-up personnel should wear proper protective equipment and clothing.  Absorb material with suitable absorbent and containerize for disposal.

7.	SPECIAL PRECAUTIONS

Precautions to be Taken in Handling or Storing:  Material should be kept

in an

area suitable for the storage of highly flammable liquids.  Bond and

ground

containers when transferring liquid.

Other Precautions:  Keep oxidizing materials and strong acids away.

8.	SPECIAL PROTECTION INFORMATION

Respiratory Protection(Specify Type):

A NIOSH/MSHA chemical cartridge respirator should be worn if PEL or TLV is exceeded. Ventilation:

Local Exhaust:       Preferred Mechanical(General): Yes

Special:	Generally not needed

Other:	No

Protective Gloves:

Rubber, neoprene, PVC, or equivalent. Eye Protection:

Splash proof chemical safety goggles should be worn at all times. Other Protective Clothing or Equipment:

Lab apron, eye wash, and safety shower.

9.	PHYSICAL DATA

Molecular Weight: Melting Point: Boiling Point: Vapor Pressure: Vapor Density(Air=l): Specific Gravity(H2O=1): Percent Volatile by Volume: Evaporation Rate(H20=l): Solubility in Water: Appearance and Odor:

Does not apply

No information available

173 .3-176.4F

40 mmHg at 68F

1 . 5

.815 at 60F

95%

1. 70

Miscible

Clear, colorless liquid

Page 2 of 4

MATERIAL SAFETY DATA SHEET

Revised: 16/07/04

PHENOLPHTHALEIN, 0.5%	Replaces: 09/05/00

CAROLINA BIOLOGICAL	Printed: 20/05/08

10. REACTIVITY DATA

Stability:  Stable

Conditions to Avoid:  Heat, sparks, and open flame.

Incompatibility(Materials to Avoid):  Oxidizers

Hazardous Decomposition Products:  Thermal decomposition or burning may

produce carbon monoxide and/or carbon dioxide.

Hazardous Polymerization:  Will not occur

11.TOXICITYDATA

Toxicity Data:   Mixture has not been thoroughly evaluated.  Data is

listed

for individual components.

Ethyl alcohol -LD50(oral-rat)-7,060 mg/kg,

LC50(ihl-rat)- 20,000 ppm/lOH Methyl isobutyl ketone- LD50(oral-rat)- 5,080 mg/kg,

LC50(ihl-rat)- 8/000/4H Hexane- LD50(oral-rat)- 28,710 mg/kg,

TCL(ihl-human)- 5,000 ppm/lOM Phenolphthalein - LDLo(ipr-rat) 500 mg/kg Effects of Overexposure:

Acute:  See section 3 Chronic:

Ethyl alcohol: Mutation data cited.  Not listed as causing cancer by I ARC,

NTP, or OSHA

Methyl isobutyl ketone: No information available Hexane:  Not listed as causing cancer by IARC, NTP, or OSHA Conditions Aggravated by Overexposure:  Pre-existing conditions of the skin or

liver may be aggravated. Target Organs:  Skin and liver Primary Route(s) of Entry:  Inhalation, skin absorption

12.	ECOLOGICAL DATA

EPA Waste Numbers:  None

13.	DISPOSAL INFORMATION

Waste Disposal Methods:  Dispose in accordance with all applicable

Federal,

State and Local regulations.

Always contact a permitted waste disposer (TSD) to assure compliance.

14.	TRANSPORT INFORMATION

Description: Flammable liquid, n.o.s. (Contains Ethanol, Methyl isobutyl ketone, Hexane), 3, UN1993, II

15.	REGULATORY INFORMATION

EPA TSCA Status:  On TSCA Inventory

Hazard Category for SARA Section 311/312 Reporting:  Acute, Fire

SARA Sec. 313

SARA EHS      Chemicals	CERCLA      RCRA

Product or      Sec. 302      Name  Chemical	Sec. 103    Sec.

Components        TPQ        List  Category         RQ Ibs. 261.33

Page 3 of 4

MATERIAL SAFETY DATA SHEET

Revised: 16/07/04

PHENOLPHTHALEIN, 0.5%	Replaces: 09/05/00

CAROLINA BIOLOGICAL	Printed: 20/05/08

Phenolphthalein	No	No	No	No         No

Ethyl alcohol	No	No	No	No          No

Methyl isobutyl

ketone	No	Yes	No	5000

Yes(Ignitable)

Hexane	No	No	No	No           No

16. ADDITIONAL INFORMATION

The information provided in this Material Safety Data Sheet represents a compilation of data drawn directly from various sources available to us. Carolina Biological Supply makes no representation or guarantee as to the suitability of this information to a particular application of the substance covered in the Material Safety Data Sheet.  Any^employer must carefully assess the applicability of any information contained herein in regards to the particular use to which the employer puts the material. Glossary

ACGIH	American Conference of Governmental Industrial Hygienists

CAS Number..Chemical Abstracts Service Number

CERCLA	Comprehensive Environmental Response, Compensation, and

Liability Act

DOT	U.S. Department of Transportation

IARC	International Agency of Research on Cancer

mppcf	million particles per cubic foot

N/A	Not Available

NTP	National Toxicology Program

OSHA	Occupational Safety and Health Administration

PEL	Permissible Exposure Limit

ppm	parts per million

RCRA	Resource Conservation and Recovery Act

SARA	Superfund Amendments and Reauthorization Act

TLV	Threshold Limit Value

TSCA	Toxic Substances Control Act

Page 4 of 4