TEXACO INC.

INDUSTRIAL HYGIENE, TOXICOLOGY, AND MATERIAL SAFETY DATA SHEET

NOTE: NO REPRESENTATION  IS MADE AS TO THE ACCURACY OF THE INFORMATION

HEREIN.    SEE PAGE 5 FOR CONDITIONS UNDER WHICH DATA    ARE FURNISHED.

TEXACO

Trade Name and Synonyms 00365  TEXACO  UNLEADED

Manufacturer's Name

Texaco  Inc

Address P.O.   Box   509  Beacon,   NY   12508

Emergency Telephone No.

(914)   831-3400  ext.   406

Chemical Name and/or Family or Description

Automotive Lead-Free Gasoline

THIS PRODUCT IS CLASSIFIED AS:

X

HAZARDOUS BY DEFINITION NO.(S)

1,2,7

NOT HAZARDOUS: _ ON ATTACHED EXPLANATION SHEET 4

WARNING STATEMENT:

DANGER: EXTREMELY FLAMMABLE

HARMFUL OR FATAL IF SWALLOWED

MAY BE HARMFUL IF INHALED; MAY CAUSE IRRITATION

MAY BE HARMFUL IF ABSORBED THROUGH SKIN

Protective Equipment (Type)

Eyes:        Chemical type goggles or face shield optional,

Skin:

Protective clothing such as uniforms, coveralls or lab coats should be worn.  Launder or dry clean when soiled.  Gloves impervious to chemicals and petroleum distillates required.

Inhalation:

SCBA or supplied air respiratory protection required for entry into tanks,vessels, or other confined spaces containing gasoline.

Ventilation Required:   Adequate   to meet   recommended  concentrations.

Permissible Concentrations:

Air:         The ACGIH (1982) TWA for gasoline is 300 ppm; Texaco recommends a TWA of 100 ppm.

First Aid Eyes:

Skin:

Ingestion:

Flush with water for fifteen minutes. Wash exposed areas with soap and water.

Do NOT induce vomiting.  May cause chemical pneumonitis. Call a physician.

Inhalation:

Should symptoms noted under physiological effects occur, remove to fresh air.  If not breathing, apply artificial respiration.

Other Instructions:  Remove gasoline-soaked clothing.

PHYSIOLOGICAt: EFFEGTSr

Effects of Exposure
Acute:
Eyes:	Causes   slight-moderate  eye  irritation.

Skin:

Moderately irritating; causes redness, edema, or drying of the skin.

Respiratory System: May cause dizziness, irritation of eyes, nose and throat, vomiting, bluish color of the skin, and CNS effects. See A.C., p.4.

Chronic:

Recent studies with laboratory animals have shown that gasoline vapors caused kidney damage and kidney cancer in rats and liver cancer in mice.

Other:

Sensitization Properties:

Skin:    Yes

No    X       Unknown

Respiratory:   Yes

No

Unknown

Median Lethal Dose (LD soLC so ((Species)

Oral 	LD50  =  18.75  ml/kg   (rat)

Inhalation   	N.D.

G.T.   5  ml/kg   (rabbit)

Dermal

Other	N.   D ...

Irritation Index. Estimation of Irritation (Species)

Skin	0.98/8.0   (rabbit)

Eyes

0/110   (rabbit)

Symptoms of Exposure   See   above .

FIRE PROTECTION INFORMATION

Ignition Temp. F.

850  F

Flammable Limits%        Lower 1.4%

Flash Point    F. (Method) Upper   7.6%

-40F   (COC)

Products Evolved When Subjected to Heat or Combustion:

Carbon monoxide and carbon dioxide may be formed on burning in limited air supply.

Recommended Fire Extinguishing Agents And Special Procedures:

According to the National Fire Protection Association Guide 325M, use dry chemical, foam or carbon dioxide.  Water may be ineffective on the flames, but water should be used to keep fire-exposed containers cool.  If a leak or spill has not ignited, use water spray to disperse the vapors and to provide protection for the persons attempting to stop the leak.

Unusual or Explosive Hazards:

Flowing gasoline can be ignited by self-generated static electricity; use adequate grounding.

ENVIRONMENTAL. PROTECTION

00365'

Waste Disposal Method:

Re-evaluation of the product may be required by the user at the time of disposal, since the product uses, transformations, mixtures and processes may change classification to non-hazardous or hazardous for reasons other than, or in addition to ignitability. (See Remarks for Waste Classification.) Procedures in Case of Breakage or Leakage:

Eliminate all ignition sources including internal combustion engines and power tools. Ventilate area. Avoid breathing vapor. Use SCBA or supplied-air mask for large spills in confined areas. Contain spill if possible. Remove with inert absorbant.

Remarks:

Waste Classification: Product (as presently constituted) has the RCRA characteristic of ignitability and if discarded in its purchased form would have the hazardous waste number D001.

PRECAUTiGM/W LABEL

DANGER: EXTREMELY FLAMMABLE

HARMFUL OR FATAL IF SWALLOWED

MAY BE HARMFUL IF INHALED; MAY CAUSE IRRITATION

MAY BE HARMFUL IF ABSORBED THROUGH SKIN

Long term exposure to vapors has caused cancer in laboratory

animals.    Keep away from heat, sparks and flame.

Avoid breathing vapor.Use "only in well-ventilated locations.

Avoid contact with eyes and prolonged contact with skin.

Keep container closed.   Wash thoroughly after handling.

FOR USE AS MOTOR FUEL ONLY

Requirements for Transportation, Handling and Storage:

Transport, handle and store in accordance with OSHA Regulation 1910.106, and applicable D.O.T. regulations.

DOT Proper Shipping Name: dot Hazard Class (if applicable):

Gasoline

Flammable liquid, UN 1203

Boiling Point (°F)

Specific Gravity         Q . 7- . 77

Appearance and Odor      Light   straw   to   light   rpd   liquid

(H20=1)

GT9Q

Vapor Pressure _SHr33H_  (mmHg)
Vapor Dc-nsity    3-4.0	(Air=1)

pH of undiluted product      N.A.

Percent Volatile by Volume    100 Viscosity        L.T.    1.4   rSt   0   1 OOF

Solubility    slight

Evaporation     N.D.
Other      -

COMPOSITION

Components Presenting a Significant Hazard

Lead-free gasoline

Gasoline consists mainly of highly branched paraffinic hydrocarbons, branched olefins, cycloparaffins amd aromatics.   The benzene content normally varies from 0.2-3.5% with a typical value of 1.4%.

100

Other Components

None

TEXACO INTENDS TO COMPLY FULLY WITH PROVISIONS OF THE TOXIC SUBSTANCES CONTROL ACT STATE OF MICHIGAN CRITICAL MATERIALS ACT (REVISED 1982)

1.4% benzene; conversion factor 6.7 pounds per gallon

Other effects of inhalation include central nervous system effects such as contracted pupils, loss of reflexes, convulsions, seizures, sudden loss of consciousness, coma and sudden death. Other indications of overexposure are headaches, flushing of the face, nausea, mental confusion and depression, loss of appetite, blurred speech and difficulty in swallowing. This product is intended for motor fuel only.

To determine applicability or effect of any law or regulation with respect to this product, user should consult his legal advisor or the appropriate government agency.   Texaco does not undertake to furnish advice on such matters.

By

Date:

R.   T.   Richards

08-10-83

New

Title Mqr.   Env.   Conservation  & Toxicology U    Revised, Supersedes

N.D. - Not Determined        N.A. - Not Applicable < Less Than       > Greater Than

Hazardous Polymerizations

Do not occur

Occur

The Material Reacts Violently With: (If Others is checked below, see Additional Comments on.Page 4 for further details)
Air	Water	Heat	Strong Oxidizers	Others	None of These

X	X