y

(OOODOO-OOOOOO-	-4063

MATERIAL SAFETY DATA SHEET: BEAUTI-TRAC YELLOW

DATE   OF   ISSUE 5/08/1998

Page:    1

SUPERSEDES 12/04/1996

SECTION  I   -   GENERAL   INFORMATION

Chemical Name   £   Synonyms

-anical   Family: EPOXY

Trade  Name   fc   Synonyms BEAUTI-TRAC  YELLOW

Formula Mixture   —> X

Manufacturer's   Name:

MANTEK,   DIVISION  OF  NCH  CORP.

Address:

BOX 152170

IRVING,	TEXAS   75015

Prepared By:

C  KERINS/CHEMIST

Product   Code  Number 4063

Emergency  Phone  Number 800-424-9300

SECTION II - HAZARDOUS INGREDIENTS THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS

Chemical  Name   (Ingredients)

PART  A

DIGLYCIDYL ETHER OF BISPHENOL A

LEAD CHROMATE

BARIUM SULFATE

SILICON DIOXIDE

PART B: POLYETHYLENE POLYAMINE ADDUCT

ETHYLEHE GLYCOL MONOPROPYL ETHER

ACETIC ACID

Hazard

IRR./SENS.

CARC.

IRRITANT

CARC.

IRR./SENS.

IRRITANT

IRRITANT

TLV

N/E     1.

.05MG/M3 1 5MG/M3  1.

.1MG/M3 1. N/E     1. 25PPM  1. 10PPM   1.

PEL

N/E	2.

NOT EST	2

5MG/M3	2.

10MG/M3	2 .

N/E	2.
25PPM  2.

10PPM	2.

N/E N/E N/E N/E N/E N/E

CAS *

25068-38-6

7753-97-6

7727-43-7

14803-60-7

H/E

2807-30-9

64-19-7

SECTION III - PHYSICAL DATA

Boiling Point (F) :	200-270	Specific Gravity	(H20=l) :	A:1.91  B:1.07

Vapor Pressure (MM HG) :	18	Color:	A. YELLOW B. WHITE

or Density (Aj_r=l) :	0.6	Odor:	AMMONIA

_.. @ 100% :	N/A	Clarity:	A. OPAQUE B. CLEAR

% Volatile by Volume:	A+B 40%	Evaporation Rate	(BU A/C=1) :	0.1

H20 Solubility:	SLIGHT	Viscosity:	SEMI-VISCOUS

SECTION IV - FIRE AND EXPLOSION HAZARD

Flash Point	Flammable Limits	LEL	UEL

NON-FLAM / T. C. C.	N/A	N/A	N/A

Extinguishing Media

X <—Foam	X <-~Alcohol Foam        X <—CO2       X <—Dry Chemical         X <—Water Spray      < — Other

Special Fire Fighting Procedures:

FIREFIGHTERS SHOULD WEAR A SELF-CONTAINED BREATHING APPARATUS AND FULL PROTECTIVE GEAR.

Unusual Fire and Explosion Hazards:

OXIDES OF CARBON AND ACRID SMOKE  WILL BE FORMED IF HEATED TO DECOMPOSITION.

Aerosol Level (NFPA 30B) :    N/A

NFPA 704 Hazard Rating       (0= Insignificant l=Slight  2=Moderate  3-Hj.gh  4=Extreme) 2  <--Health      1  <--Flammability  0  <—Instability      <--Special

SECTION V - HEALTH HAZARD DATA

Threshold  Limit  Value: NOT  ESTABLISHED   FOR MIXTURE.       SEE   SECTION   II.

Effects  of Overexposure:

-Acute    (Short   Term  Exposure)

SKIN CONTACT:   MAY   CAUSE   IRRITATION.   MAY   CAUSE  AN  ALLERGIC   SKIN  REACTION  AS   EVIDENCED  BY   HIVES,    SWELLING  AND   ITCHING.       EYE   CONTACT:    CAUSES   IRRITATION SUCH AS   REDNESS   OR  STINGING.   MAY   CAUSE   CORNEAL  DAMAGE.    THE   GRIT   IN  THE   PRODUCT  MAY   CAUSE  MECHANICAL  DAMAGE    (SCRATCHES)    TO   THE   SURFACE   OF   THE  EYZ. INHALATION:   VAPORS  MAY   CAUSE   IRRITATION   OF  THE  RESPIRATORY   TRACT   SUCH  AS      COUGHING,    SNEEZING  OR  TROUBLE   BREATHING."       INGESTION:    UNCURED  MATERIAL   CAN CAUSE   IRRITATION  TO  THE  DIGESTIVE  TRACT,   NAUSEA,   VOMITING,    DIARRHEA AND  POSSIBLE     DAMAGE  TO   THE  BLOOD AND  BLOOD   PRODUCING  ORGANS.

-Chronic    (Long  Term  Exposure)

^ATED  OR  PROLONGED   SKIN  CONTACT  MAY   CAUSE   SENSITIZATION.    SANDING   THIS   MATERIAL   CAN  REALEASE   PARTICLES   OF   SILICA AND   LEAD   CHROMATE   WHICH  HAVE CAUSED  CANCER   IN   LABORATORY  ANIMALS.    RESPIRATORY   PROTECTION   SHOULD  BE   USED   IF   SANDING   THIS   MATERIAL.      MEDICAL   CONDITIONS   AGGRAVATED  BY   EXPOSURE  ARE PRE-EXISTING RESPIRATORY  AND   SKIN  CONDITIONS   SUCH  AS  ASTHMA,    EMPHYSEMA,   AND  DERMATITIS.      TARGET   ORGANS:    LIVER,   KIDNEYS,    BLOOD,    LUNGS.

Primary Routes   of   Entry:	X   <--Inhalation	<--Ingestion	X   <--Absorption

Emergency  and  First  Aid   Procedures: -Inhalation:

MATERIAL SAFETY DATA SHEET: BEAUTI-TRAC YELLOW         e^ge: 2 SECTION V - HEALTH HAZARD DATA (Continued)

REMOVE FROM THE AREA TO FRESH AIR. SEEK MEDICAL ATTENTION IF RESPIRATORY IRRITATION DEVELOPS OR IF BREATHING BECOMES DIFFICULT.

-Eye Contact:

RINSE THE EYES WITH WATER. REMOVE ANY CONTACT LENSES AND CONTINUE FLUSHING WITH PLENTY OF WATER FOR SEVERAL MINUTES. SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS.

-Skin Contact: WASH AFFECTED AREAS WITH PLENTY OF SOAP AND WATER FOR SEVERAL MINUTES. SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS.

-Ingestion:

GIVE 3 TO 4 GLASSES OF WATER, BUT DO NOT INDUCE VOMITING. IF VOMITING OCCURS, GIVE FLUIDS AGAIN. GET IMMEDIATE MEDICAL ATTENTION. DO NOT GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS OR CONVULSING PERSON.

-Notes to Physician: DUE TO THE TOXICITY OF LEAD, IF PART A OR A MIXTURE OF PART A AND B IS SWALLOWED, VOMITING MAY NEED TO BE INDUCED UNDER MEDICAL SUPERVISION-

SECTION VI - TOXICITY INFORMATION

Product Contains Chemicals Listed as Carcinogen or Potential Carcinogen By:

IARC—> Yes         NTE—> Yes	OSHA—> No	AC6IH—> Yes         OTHER—> No

ETHYLENE GLYCOL MONOPROPYL ETHER     CRYSTALLINE SILICA

ORL-RAT LD50: 3089MG/KG  3.         IHL-HMN TCLO: 16MPPCF/8H/17
IHL-RBT LCLO: 200QPPM    3.	Y-I TFX-PUL 3.

SKN-RBT LD50: 960MG/KG  3.	IHL-HMN LCLO: 300UG/M3/10Y-I 3.

EYE-RBT: 100MG SEVERE  3.

DIGLYCIDYL ETHER OF BISPHENOL A

LEAD CHROMATE	ORL-RAT LD50: >20G/KG 3.

SCU-RAT TDLO: 135MG/KG TFX:HEO  3.     ORL-RAT LD50: 11.4 G/KG 3. CYT-HMN: LYM 13 UMOL/L  3.

POLYETHYLENE POLYAMINE ADDUCT NO TOXIC DATA FOUND ACETIC ACID

ORL-RAT LD50: 3310MG/KG  3. ORL-HMN TDLO: 1470MG/KG: GIT  3. SKN-HMN 50MG/24H MLD  3. IHL-HMN TCLO: 316PPM/M3 IRR  3.

FOR TEXAS: VOLATILE ORGANIC COMPOUND (VOC) : 51.6G/L

CHEWING OF PAINT CHIPS CONTAINING LEAD BY SMALL CHILDREN HAS BEEN SHOW IN SEVERAL STUDIES  TO CAUSE LEARNING DISABILITIES AND OTHER HEALTH PROBLEMS. THIS PAINT SHOULD NOT BE USED IN AREAS EXPECTED TO BE USED BY SMALL CHILDREN.

SECTION  VI^   -REACTIVITY   DATA^

Stability:	X   <--Stable	<--Unstable

Conditions   to Avoid:

N/A

Incompatibility   (Materials   to Avoid) : OTHER EPOXY   RESINS,   ORGANIC  ACIDS

Hazardous  Decomposition Products:

OXIDES   OF   CARBON,    OXIDES   OF   NITROGEN,    LEAD   FUMES,    LEAD   OXIDE,   OXIDES   OF   CHROMIUM.

Hazardous   Polymerisation:	<--May Occur	X  <--Will   Not  Occur

Conditions   to  Avoid:

N/A

SECTION VIII - SPILL OR LEAK PROCEDURES

Steps to be Taken if Material is Released or Spilled:

DIKE AND CONTAIN SPILL IF SAFE TO DO SO. ABSORB WITH AN INERT MATERIAL AND TRANSFER ALL MATERIAL INTO A PROPERLY LABELED CONTAINER FOR DISPOSAL. WEAR

APPROPRIATE PROTECTIVE CLOTHING.

Waste Disposal Method(s):

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL, STATE AND LOCAL REGULATIONS.

Neutralizing Agent: N/A

SECTION IX - SPECIAL PROTECTION INFORMATION

Required  Ventilation:

GENERAL  VENTILATION   IS   NORMALLY   ADEQUATE.      LOCAL  VENTILATION   (A  FAN)    IS   RECOMMENDED   IF  MATERIAL   IS   APPLIED   IN   POORLY   VENTILATED  AREAS.

Respirator-y  Protection:

A NIOSH/MSHA APPROVED  RESPIRATOR   IN   POORLY   VENTILATED  AREAS   AND/OR  FOR EXPOSURE  ABOVE   THE  ACGIH  TLV  OR  OSHA   PEL   OR  WHERE  MISTING  EXISTS.

Glove   Protec tion:

NEOPRENE  RUBBER  GLOVES   SHOULD   BE  WORN.

Eye  Protection:

SAFETY   GLASSES  WITH  SIDE   SHIELDS   IF   THE  METHOD   OF  USE   PRESENTS   THE   LIKELIHOOD   OF  EYE   CONTACT.

Other  Protection:

PROTECTIVE   CLOTHING  TO   PREVENT   SKIN   CONTACT.

MATERIAL SAFETY DATA SHEET: BEAUTI-TRAC YELLOW         Pa(3e:

SECTION IX - SPECIAL PROTECTION INFORMATION (Continued)

SECTIOtT X - STORAGE AND HANDLING INFORMATION

borage Temperature:     Indoors—> X	Outdoors—>	Heated-->	Refrigerated—>

inimum Temperature:  35°F.  Maximum Temperature: 120°F

•••^cautions to be Taken in Handling and Storing;

P CONTAINER CLOSED WHEN NOT IN USE. DO NOT ALLOW TO FREEZE.

ther Precautions:

EEP OUT OF REACH OF CHILDREN- READ THE ENTIRE LABEL BEFORE USING THE PRODUCT. FOLLOW THE LABEL DIRECTIONS.

SECTION XI - REGULATORY INFORMATION

^lemical   Name	CAS   Hnmher	Upjjer   %   Limit

;lycol ether	n/a	is

MROMIUM COMPOUNDS	N/A	20

'hose ingredients listed above are subject to the reporting requirements of 313 of Title III of the Superfund Amendments and Reauthorization Ac1 .986 and 40 CFR part 372.

3lease call 1-800-527-9919 for additional information if you are a California customer. ?his MSDS is not intended for users in the state of California.

SECTION XII - REFERENCES

I.  THRESHOLD LIMIT VALUES FOR CHEMICAL SUBSTANCES AND PHYSICAL AGENTS AND

BIOLOGICAL EXPOSURE INDICES, ACGIH, 1997. > .  OSHAL PEL. 3.  SAX'S DANGEROUS PROPERTIES OF INDUSTRIAL MATERIALS, EIGHTH EDITION,

RICHARD J. LEWIS, SR.

ALL OF THE COMPONENTS WITHIN THIS PRODUCT CAN BE FOUND IN THE CURRENT TSCA INVENTORY.

IRR: IRRITANT, FLAM/FLAMM: FLAMMABLE, COMB: COMBUST IBLE , CORR: CORROSIVE CARC:CARCINOGENIC, TOXiTOXIC, N/A:NOT APPLICABLE, N/E:NOT ESTABLISHED, COC: CLEVELAND OPEN CUP, PMCC: PENSKY-MARTIN CLOSED CUP, TCC: TAGLIABUE CLOSED CUP, LEL:LOWER EXPLOSION LIMIT, UEL:UPPER EXPLOSION LIMIT, NFPA:NATIONAL FIRE PROTECTION ASSOCIATION, IARC: INTERNATIONAL AGENCY FOR THE RESEARCH ON CANCER, NTP:NATIONAL TOXICOLOGY PROGRAM, OSHA:OCCUPATIONAL SAFETY 5 HEALTH ADMINISTRATION, ACHGIH:AMERICAN CONFERENCE OF GOVERNMENTAL INDUSTRIAL HYGIENISTS, TLViTHRESHOLD LIMIT VALUE, PEL: PERMISSIBLE EXPOSURE LEVEL, STEL: SHORT-TERM EXPOSURE LIMIT, MLDzMILD, MOD: MODERATE, SEViSEVERE, MUT:MUTAGENIC, AS PHYX: ASPHYXIANT

• INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORMULATION.  HOWEVER, NO WARRANTY IS EXPRESSED OR •-.Ki>ING THE ACCURACY OF TI3B3E DAIA uk Tiffi RESULTS TO biE. OBTAINED FROM Thh. USt: THHREOif. S

MANTEK, DIVISION OF NCH CORP. assumes no responsibility for personal injury or property damage caused by the use, storage, or disposal of: the in a manner not recommended on the product label. Users assume all risks associated with such unrecommended use, storage, or disposal of the product.