MSDSNo.:	815

Revision No.:      008
Date:	5/13/97

MATERIAL SAFETY DATA SHEET

HILTI, INC.:	P.O. Box 21148, Tulsa, OK 74121; Ph: 1 800 879 6000; Emergency No.: 1 800 879 4444

PRODUCT:	RM 700 EP USDA Epoxy Mortar, Part C

DESCRIPTION:   Aggregate for RM 700 EP epoxy mortar

INGREDIENTS:

Silicon Dioxide (sand)

INGREDIENTS AND EXPOSURE LIMITS
CAS NUMBER:	PEL:	TLV:

14808-60-7	0.1mg/m3t	0.1mg/m3t

STEL:

NE

PEL = OSHA Permissible Exposure Limit. TLV = ACGIH Threshold Limit Value. These are 8 hour time-weighted averages unless otherwise indicated by "C" (Ceiling) or "STEL" (Short Term Exposure Limit). NE = None Established, f indicates "as respirable dust".

PHYSICAL DATA

VOC CONTENT:  None MELTING POINT: Not Determined VAPOR DENSITY: Not Applicable SOLUBILITY IN WATER:  Nil

pH: Not Determined

APPEARANCE AND ODOR: White or tan sand granules. No odor. BOILING POINT: Not Applicable VAPOR PRESSURE: Not Applicable EVAPORATION RATE: Not Applicable SPECIFIC GRAVITY: 2.6

FIRE AND EXPLOSION HAZARD DATA

EXTINGUISHING MEDIA: Not Applicable	FLAMMABLE LIMITS: Not Applicable

SPECIAL FIRE FIGHTING PROCEDURES: None known	FLASHPOINT:  Not Applicable

UNUSUAL FIRE AND EXPLOSION HAZARDS: None known

REACTIVITY DATA

INCOMPATIBILITY: Strong oxidizing agents HAZARDOUS DECOMPOSITION PRODUCTS: None expected HAZARDOUS POLYMERIZATION: Will not occur

STABILITY: Stable.

CONDITIONS TO AVOID: Contact with powerful oxidizing agents such as fluorine, chlorine trifluoride, manganese tricxide, oxygen trifluoride.

HEALTH HAZARD DATA

KNOWN HAZARDS: None expected from normal use of this product. Excess exposure to dusts by inhalation for extended periods of time can lead to the development of pulmonary diseases including pneumoconiosis such as silicosis.

CARCINOGENICITY: IARC has classified respirable silica (sand, CAS No. 14808-60-7) as Group 2A based on studies of quarry and foundry workers. The use of this product does not pose a cancer risk; however, wearing a NIOSH-approved dust mask in any dusty situations will reduce the potential for exposure to silica.

PRIMARY ROUTES OF EXPOSURE: Dermal. Inhalation.

SIGNS AND SYMPTOMS OF EXPOSURE: Eyes - Mechanical inhury can occur from rubbing the eyes due to the abrasive nature of sand. Skin - Prolonged and repeated exposure to the hands can lead to drying of the skin and dermatitis. Inhalation - Excessive exposure to dusts can lead to breathing difficulty.

MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE: Eye, skin, and respiratory conditions.

EMERGENCY AND FIRST AID PROCEDURES

EYES: Immediately flush with plenty of water. Call a physician if irritation persists. SKIN: Wash with soap and water. INHALATION: Move victim to fresh air. Call a physician if symptoms persist.

INGESTION:   Not normally a route of exposure.  Give plenty of water to drink.   Seek medical attention.   Do not induce vomiting unless directed to by a physician.

OTHER:     Referral to a physician is recommended if there is any question about the seriousness of the injury/exposure.

CONTROL MEASURES AND PERSONAL PROTECTIVE EQUIPMENT VENTILATION: General, i.e. natural or mechanically induced fresh air movements.

EYE  PROTECTION:     Safety  glasses with  side  shields as  a  minimum.     Splash-proof chemical  goggles recommended while mixing parts A and B.

SKIN PROTECTION: Cotton gloves recommended to keep hands from defatting (drying out).

RESPIRATORY PROTECTION: None normally required with adequate ventilation as noted above. Use a NIOSH-approved dust mask when working in areas where dust is being generated at or near the PEL/TLV.

PRECAUTIONS FOR SAFE HANDLING AND USE

HANDLING AND STORING PRECAUTIONS:   Avoid generating dust.  Avoid contact with eyes, skin, or clothing. Wash hands after handling. Store in a dry location.

SPILL PROCEDURES: Sweep up and discard.

REGULATORY INFORMATION

OSHA HAZARD COMMUNICATION: This MSDS has been prepared in accordance with the federal OSHA Hazard Communication Standard.

HMIS CODES: Health 2, Flammability 0, Reactivity 0, PPEA

DOT SHIPPING NAME: Non-regulated.

TSCA INVENTORY STATUS: Chemical components listed on TSCA inventory.

SARA TITLE III, SECTION 313:  This product does not contain any toxic chemicals which are subject to reporting under Section 313 of SARA Title III (40 CFR Part 372).

WASTE DISPOSAL METHODS: Consult with regulatory agencies or corporate personnel for disposal methods that comply with local, state, and federal health and environmental regulations.

HEALTH / SAFETY CONTACTS:    Steve Gerrard (x6309)

Jerry Metcalf   (x6704)

The information and recommendations contained herein are based upon data believed to be correct; however, no guarantee or warranty of any kind expressed or implied is made with respect to the information provided.