MATERIAL SAFETY DATA SHEET

(SIMILAR TO OSHA FORM 174)

N.A. = Not Applicable Not Est. = Not Established Prop. = Proprietary

SECTION 1 -  MANUFACTURER / DISTRIBUTOR INFORMATION

Product Name: EARTH SENSE #1 Glass and Multi-Surface Cleaner Super Concentrate

Product #:   4001

Supplier's Name: National Chemical Laboratories of PA, Inc.

Supplier's Address: 401 N. 10m Street • Philadelphia PA 19123

Emergency Phone #: CHEM-TEL (800) 255-3924 Chemical Name: N.A. Proprietary Blend Formula: N.A. Blended Product Hazard Class: ORM-D   D.O.T. ID#:   N/A

HMIS Hazard Code:    Health: 2   Flammability: 3   Reactivity: 0   Other: 0

Chemical Family: Concentrated Glass Cleaner

D.O.T. Shipping Name:   Consumer Commodity

| SECTION 2 - HAZARDOUS INGREDIENTS |  |  |
| --- | --- | --- |
| CHEMICAL NAME | CAS NUMBER % Concentration Range | Hazard Ratinqs (TLV/PEL) |
| Isopropyl Alcohol 2-Amino Ethanol Dipropylene Glycol Monomethyl Ether | 67-63-0 PROP. 141-43-5 PROP. 34590-94-8 PROP. | 400 ppm 3 ppm Not Est. |

SECTION 3  -  PHYSICAL DATA

Boiling Point (°F):   180 Solubility in Water: Complete Appearance: Thin Clear Blue Liquid Odor: Alcohol

Specific Gravity:   0.840 Evaporation Rate: = To Water pH:  11.4

SECTION 4 -  FIRE AND EXPLOSION DATA

Flash Point (°F):   70 (TCC)

Flammable Limits:   Not Est.

Unusual Fire & Explosion Hazards: Respiratory protection required for fire fighting personnel. Stay upwind if possible. Cool exposed tanks with

water. Water spray may be ineffective in fire fighting.

SECTION 5 -  HEALTH HAZARD DATA

Route(s) of Entry:     Inhalation: Yes    Skin:    Yes    Ingestion:   Yes	Threshold Limit Value:    Not Est.

Effects of Overexposure: Vapors irritate eyes, nose, throat. Liquid will damage eye tissue. If ingested may cause gastric distress and systemic

intoxication.

Emergency & First Aid Procedures: Remove to fresh air and if breathing has stopped give artificial respiration. Keep individual calm. Seek medical

attention. For Eyes: Flush eyes with water for at least fifteen minutes. If irritation persists seek medical attention. For Skin: Flush skin thoroughly wit

water. If Ingested: Do not induce vomiting. Give large quantities of water. Seek medical attention. Do not give anything by mouth to an unconscious

person.

Conditions Aggravated by Exposure: May aggravate pre-existing pulmonary condition.

Listed Carcinogen, Mutagen, Teratogen (Ingredient, Source, Amount):   None

SECTION 6 -  REACTIVITY DATA

Stability:   Stable

Conditions to Avoid:   None

Incompatibility: Avoid contact with alkylene oxides, acid anhydrides, inorganic acids, organic acids, halogens, phosphorus trichloride, aldehydes,

monomers, polymerizable esters, hydrogen with palladium.

Hazardous Decomposition Products: Nitrogen Oxides, Carbon Monoxide.

Hazardous Polymerization: Will not occur.

SECTION 7  -  SPILL OR LEAK PROCEDURES

Steps to be taken if material is released or spilled: Shut off source if possible to do so without hazard. Eliminate sources of ignition. Prevent liquid

from entering sewers, water sources or low areas.

Waste disposal method: Follow local, state, and federal regulations.

SECTION 8  -  SPECIAL PROTECTION INFORMATION

Respiratory Protection: Not required

Ventilation: Local

Protective Gloves: Latex or rubber

Eye Protection: Goggles or safety glasses with side shields.

Other Protective Equipment: Not required.

SECTION 9 - SPECIAL PRECAUTIONS

Precautions to be taken in handling and storage: Store away from heat, sparks, or other sources of ignition. Do not block entrances. Store betweei

40°Fand120°F

Other Precautions: Keep out of reach of children. For professional use only.

The information contained in this MSDS was obtained from current and reliable sources, however the data is provided without any warranty, expressed or implied, regarding its correctness or accuracy. Since the conditions or handling, storage and disposal of this product are beyond the control of this company, it is not responsible for loss, injury and expense arising out of the product's improper use. No warranty, expressed or inferred, regarding the product described in this MSDS shall be created or inferred by any statement in this MSDS. Various government agencies may have specific regulations regarding the transportation, handling, storage, use or disposal of this product which may be covered by this MSDS. The user is responsible for full compliance.

DaTePrepa'red^ulToe/uT"    By: ~G.C.