MAINTENANCE SUPPLY COMPANY, INC.   :

POST OFFICE BOX 498 HUNTERSVILLE, NORTH CAROLINA 28078

.> '

*•

- material safety data sheet -  '

these material safety data sheets are sent to you in compliance with 29 cfr 1910.12qo which became effective november 25, 1985.

emergency phone: 1-800-255-3924 phone number for information: 1-422-2071 ' manufacturer's msds number: r 342-3 cr179) distributor's msos reference:  8254-0 revision date:  2/20/92

SECTION 1 -. IDENTIFICATION

TRADE NAME:        GOODBYE GRAFFITI » 2 (LIQUID}

',-• • ••••* **. • . ,• .; • •

TYPE OF PRODUCT:   GRAFFITI REMOVER

nfpa rating: health 3 flajwability 3 reactivity o hmis rating: health 3 flammability 3 reactivity o

PERSONAL PROTECTION 8 DOT HAZARD CLASSIFICATION: FLAMMABLE/CORROSIVE MATERIAL

MANUFACTURER'S NAME:  AMREP, INC. ,

ADDRESS:	990 INDUSTRIAL PARK DRIVE

MARIETTA, GA 30062

NOTICE:  JUDGEMENT BASED ON INDIRECT TEST DATA

| SECTION 2 - MATERIAL IDENTIFICATION |  | AND INFORMATION |  |
| --- | --- | --- | --- |
| CHEMICAL NAMES | APPROX. % (WT) | OSHA PEL (PPM} TLV | ACGIH (PPM) |
| AMYL ACETATE (CAS # 628637) | < 5 | 100 | 100 |
| 2-BUTOXYETHANOL (CAS # 111-76-2} | 33-38 | 50 | 25 |
| METHYL AMYL KETONE (CAS if 110-43-0) | 5-10 | 50 | 50 |
| ORTHOOICHLOR08ENZENE (CAS # 95-50-1) | 8-12 | 50 | 50 |
| AROMATIC HYDROCARBON (CAS if 647-95-6) | SOLVENT 2-5 | 100 | 100 |
| *METHYLENE CHLORIDE (CAS # 75-09-2) | 10-15 | 200 | 50 |
| XYLEf^E | 5-10 | 100 | 100 |

(CAS # 1330-20-7)

*METHYLENE CHLORIDE IS LISTED AS A CARCINOGEN OR POTENTIAL

CARCINOGEN 3Y THE IARC MONOGRAPH.

NOTICE: THIS PRODUCT CONTAINS THE FOLLOWING LISTED SARA TITLE III

SECTION 313 CHEMICALS:

METHYLENE CHLORIDE  CAS # 75-09-2  APPROX. %, 10-15

•1-

SECTIQN 3 -_PHYSICAL/CH£MICAL CHARACTERISTICS

SOILING POINT. DEGREES F.: 104-340

VAPOR PRESSURE: 1.2 MH HG a 20 DEGREES C.

VAPOR DENSITY (AIR =1): <1.0	"

SOLUBILITY IN WATER; MODERATE	.

APPEARANCE AND OOOR: LIGHT YELLOWS/SWEET FRAGRANCE  ,

SPECIFIC GRAVITY (H20=l): 0.973 G/ML

EVAPORATION RATE (BUTYL ACETATE =1): >1

WATER REACTIVE: HO

SECTION 4 - FIRE ANO EXPLOSION HAZARD DATA

FLAMMABILITY AS PER USA FLAHE PROJECTION TEST (AEROSOLS): N/A

AUTO IGNITION TEMPERATURE- DEGREES F.: N/A FLAMMABLE LIMITS IN AIR. VOLUME %: LOWER (LED: l.O;

UPPER <UEL): 7,0

FLASH POINT AND METHOD USED: <100 DEGREES F., (TCC) EXTINGUISHER MEDIA: MATER SPRAY, FOAM, CARBON DIOXIDE,

DRY CHEMICAL SPECIAL FIREFIGHTING PROCEDURES* DO NOT ENTER CONFINED AREAS

WITHOUT FULL BUNKER SEAR (HELMET, FACE SHIELD. COATS, BOOTS, GLOVES)*  USE SELF-CONTAINED NIOSH APPROVED BREATHING. APPARATUS*  COOL FIRE CONTAINERS WITH WATER, UNUSUAL FIRE £ EXPLOSION HAZARDS: COOL CONTAINERS TO PREVENT VAPOR PRESSURE BUILD-UP AMD POTENTIAL RUPTURE.

_SECTION 5 ~_REACTIVITY_HAZARD_DATA

STABILITY: STABLE

CONDITIONS TO AVOID: EXPOSURE TO TEMPERATURES IN EXCESS OF 120 DEGREES F.. HEAT, SPARKS, OPEN FLAMEy OR OTHER

SOURCES OF IGNITION.

INCOMPATIBILITY (MATERIALS TO AVOID): PLASTICS

HAZARDOUS DECOMPOSITION PRODUCTS:

HYDROStN CHLORIDE, CARBON DIOXIDE AND OTHER UNIDENTIFIED COMPOUNDS DURINS COMBUSTION.

HAZARDOUS POLYMERIZATION: WILL NOT OCCUR

SECTION 6 - HEALTH HAZARD DATA

primary routes of entry: inhalation, skin absorption, ingestion

acute effects:

inhalation: irritant eye contact: irritant skin contact: irritant insestion: irritant

chronic effects:

2-8utoxyethanol ahq other components cause liver, kidney, and toxic effects in test animals. also 2-8utoxyethanol is associated with damage to red blood cells,

medical conditions generally aggravated by exposure: preexisting skin, eye and lung disorders may be aggravated by exposure to this product.

eye contact: flush with copious amounts of water for a minimum of 15 minutes and get immediate medical attention*

skin contact: flush with copious amounts of water. wash mith soap aw yarm water. get immediate medical

attention.

-2-

inhalation: remove to fresh air, if mot breathimg give cpr or oxygen. set immediate medical attention* •-

ingestiqn: 00 not induce vomiting as aspiration into lungs pfay result, call poison control center inmeoiately and/or get immediate medical attention,

f

SECTION 7 - CONTROL AND PROTECTIVE MEASURES

RESPIRATORY PROTECTION: IF EXPOSURE MAY EXCEED RECOMMENDED TLV»S USE A NIOSH APPROVED SELF-BREATHING APPARATUS.

PROTECTIVE GLOVES: CHEMICAL RESISTANT GLOVES,

EYE PROTECTION: CHEMICAL RESISTANT GOGGLES IF EYE CONTACT

IS LIKELY,

VENTILATION REQUIREMENTS: USE OiMLY WITH ADEQUATE VENTILATION.

OTHER PROTECTIVE CLOTHING AND EQUIPMENT: CHEMICAL RESISTANT

CLOTHING, MASH CONTAMINATED CLOTHING BEFORE REUSING,

HYGIENIC WORK PRACTICES: WASH WITH SOAP 'AND WATER BEFORE EATING, SMOKING, DRINKING OR USING TOILET FACILITIES,

SECTION 8 - PRECAUTIONS FOR SAFE HANDLING AND USE/LEAK PROCEDURE

STEPS TO BE TAKEN IF MATERIAL IS SPILLED OR RELEASED: TURN OFF ALL SOURCES OF IGNITION.  PROVIDE IMMEDIATE VENTILATION.  DIKE AREA,  APPLY AN ABSORBENT TG SOAK UP.  TRANSFER ABSORBED MATERIAL INTO CONTAINERS FOR DISPOSAL.

WASTE DISPOSAL METHODS: DISPOSE OF ALL WASTE IN ACCORDANCE

WITH FEDERAL, STATE AMD LOCAL REGULATIONS.

PRECAUTIONS TO BE TAKEN IN HANDLING AMD STORAGE: STORE IN TEMPERATURES BELOW 120 DEGREES F.  FOLLOW ALL LISTED RECOMMENDATIONS AND PRODUCT LABELING,

OTHER PRECAUTIONS AND/OR SPECIAL HAZARDS: FOLLOW ALL LISTED

RECOMMENDATIONS.

WE BELIEVE THE STATEMENTS, TECHNICAL INFORMATION ANO

RECOMMENDATIONS CONTAINED HEREIN ARE RELIABLE. BUT THEY ARE GIVEN WITHOUT WARRANTY OR GUARANTEE OF ANY KIND, EXPRESSED OR IMPLIED,7

-3-