^.T^ 'MATERIAL SAFETY (AMOCO) DATA SHEET

AMOCO 300 MOTOR OIL SAE 15W-40

MANUFACTURER/SUPPLIER: Amoco Oi1 Company 200 East Randolph Drive Chicago, Illinois 60601

EMERGENCY HEALTH INFORMATION:  (800) 447-8735 EMERGENCY SPILL INFORMATION:  (800) 424-9300 OTHER PRODUCT SAFETY INFORMATION:  (312) 856-3907

IMPORTANT COMPONENTS: Solvent refined paraffinic petroleum oil (CAS 64741-88-4).

Zinc dialkyl dithiophosphate (ZDDP).

WARNING STATEMENT: Continuous long-term contact with used motor oils has caused skin

cancer in animal tests. Avoid prolonged skin contact with used motor oils.

HMIS/NFPA CODES:(HEALTH;0)(FLAMMABILITY;1)(REACTIVITY;0) APPEARANCE AND ODOR: Oily liquid.

EFFECT: FIRST AID: PROTECTION:

EFFECT: FIRST AID: PROTECTION:

HEALTH HAZARD INFORMATION

EYE

No significant irritation expected. Flush eyes with plenty of water. None required, however, use of safety glasses is good industrial practice.

SKIN

None expected for single short-term exposures.  Prolonged or repeated contact may produce some irritation.

None required for unused motor oil.  Contact with used motor oil—wash exposed skin thoroughly with soap and water.

Wear protective clothing and impervious gloves when working with used motor oils.

INHALATION

EFFECT:     None expected under usual conditions of use. FIRST AID:  None required. PROTECTION:  None required for usual conditions of use.

INGESTION

EFFECT:     Expected to be relatively non-toxic. FIRST AID:  If a large amount is swallowed, induce vomiting, get medical attention.

FIRE AND EXPLOSION INFORMATION

ASHPOINT:     420°F, Minimum (COC)

TINGUISHING MEDIA: Agents approved for Class B hazards (e.g., dry chemical, carbon

dioxide, halogenated agents, foam, steam) or water fog.

REACTIVITY INFORMATION

ABILITY: Stable.

CHEMICAL AND PHYSICAL PROPERTIES

-UBILITY IN WATER:  Negligible,	below 0.1%

ECIFIC GRAVITY (WATER =1):	0.88

>COSITY:  72-76 SUS P 210°F	VISCOSITY INDEX: 120 minimum

JR POINT: -10°F Maximum

STORAGE AND ENVIRONMENTAL PROTECTION ...

)RAGE REQUIREMENTS: No special requirements.

ILLS AND LEAKS: Treat as an oil spill.  Contain and remove by mechanical means.

>TE DISPOSAL:  Disposal must be in accordance with applicable federal, state, or

local regulations.  Enclosed-controlled incineration is recommended unless directed otherwise by applicable ordinances.

TOXICOLOGICAL INFORMATION

ITION!

itinuous  long-term contact with  used motor oils  has caused  cancer  in  animal

;ts.

case of contact, wash exposed skin thoroughly with soap and water or use .erless hand cleaners to remove used motor oils from skin.  Do not use oline, thinners, or solvents.

r protective clothing and impervious gloves when working with used motor s.  Remove oil-soaked clothing, including shoes, and thoroughly clean and before reuse.

s product contains a zinc dithiophosphate (ZDDP) component.  The ZDDP in s product is judged not to present a significant risk to human health when d personal hygiene is observed.

eated dermal exposure to ZDDPs have produced severe skin irritation in bits which resulted in reduced food consumption and substantial body weight s. Testicular atrophy was seen in these rabbits, and appeared to be a sequence of the substantial body weight loss.  ZDDPs have also been found to mutagenic in some tests: however, this activity has been attributed to the c constituent.  Zinc is ubiquitous in the environment and an essential rient.  Further, it is generally accepted that zinc does not present a agenic or carcinogenic risk to humans.

s product contains a small amount of an organic compound of a type which

AMOCO 300 MOTOR OIL SAE 15W-40 PAGE 03 OF 03

TOXICOLOGICAL INFORMATION - CONTINUED

produced testicular effects in rabbits following prolonged and repeated skin exposure in high dosages. The concentration of this component is below that which produced no effect in the animal studies. The high levels of exposure that produced adverse effects in rabbits are not expected to result from normal use of the product. No adverse health effects are expected to occur when good personal hygiene is observed.

REGULATORY INFORMATION

OSHA HAZARD COMMUNICATION STANDARD: Not hazardous per 29 CFR 1910.1200(d),

DOT PROPER SHIPPING NAME (BULK, LAND):  Not regulated. Truck/Rail Shipping Class: Petroleum Lubricating Oil.

ISSUE INFORMATION

BY:

Stephen A. Elbert	ISSUED:  November 25, 1985

Mgr., Product Safety & Toxicology	SUPERSEDES: August 07, 1985

This material safety data sheet and the information it contains is offered to you in good faith as accurate. We have reviewed any information contained in this data sheet which we received from sources outside our company. We believe that information to be correct but cannot guarantee its accuracy or completeness. Health and safety precautions in this data sheet may not be adequate for all individuals and/or situations. It is the user's obligation to evaluate and use this product safely and to comply with all applicable laws and regulations. No statement made in this data sheet shall be construed as a permission or recommendation for the use of any product in a manner that might infringe existing patents. No warranty is made, either express or implied.