ISSUE DATE:      11/07/00

ZEP MANUFACTURING COMPANY	supersedes: 09/16/92	date printed: 03/22/02

P.O. BOX 2015

ZEP PLS LIQUID

Product Number:   1043	Penetrating Fluid

ATLANTA, GA 30301

NSI Chemicals Group 1-877-I-BUY-ZEP

SECTIONI-EMERGENCY CONTACTS

CABARRUS COUNTY OF	For Addiliorial ™

NSI Chemicals Group 1-877-I-BUY-ZEP

BOARD OF EDUCATION PO BOX 388 CONCORD, NC 28025

For a Medical Emergency: INFOTRAC

(877) 541-2016 Toll Free - All Calls Recorded

For a Transportation Emergency: CHEMTREC

(800)424-9300

In the District of Columbia: (202) 483-7616

SECTION II - HAZARDOUS INGREDIENTS	TLV	EFFECTS	%IN

DESIGNATIONS	(PPM)	(SEE NOTICE)       PROD.

** ALIPHATIC NAPHTHA ** ligroin; CAS# 8052-41-3; RTECS#	100	CNS CBL	50-60

WJ8952000; OSHA PEL - 100 ppm

** LIGHT AROMATIC NAPHTHA** aromatic hydrocarbon solvent;	N/D	CBL CNS IRR	20-30

solvent naph(ha(petroleum); CAS# 64742-95-6; RTECS#NONE; OSHA

PEL-N/D

** PROPRIETARY BLENDED SALTS OF OXYGENATED AND SULFONATED       N/D	IRR	10-20

HYDROCARBONS ** CAS# PROPRIETARY; RTECS# - NONE; OSHA/ACGIH OIL

MIST LIMIT = 5 mg/mS

@**DIETHYLENEGLYCOLMONOBUTYL ETHER**	N/D	IRR	<5

2-(2-butoxyethoxy)-ethanol; butyl carbitol; CAS# 112-34-5;

RTECS#KJ9100000; OSHA PEL-N/D

** PARAFFIN OIL** blend of heavy and light naphthenic	N/D	IRR	10-20

letroleum distillate; CAS# 64742-52-5; RTECS* NONE; OSHA PEL -

N/D; ACGIH OIL MIST LTMIT= 5mgAn3

@ IDENTIFIES CHEMICALS LISTED UNDER SARA-SECTION 313 FOR RELEASE REPORTING.

SECTION III - HEALTH HAZARD DATA

SPECIAL NOTE: MSDS data pertains to the product as dispensed fiom the container. Adverse health effects

would not be expected under recommended conditions of use (diluted) so long as prescribed safety precautions

are practiced.

ACUTE EFFECTS OF OVEREXPOSURE:

Overexposure to the vapors from this product may produce mucous membrane irritation, particularly of the eye

and respiratory tract. Overexposure to vapors may also produce mild central nervous system depression

characterized by headache, dizziness, nausea, and stupor, leading to unconsciousness in extreme cases.

Introduction of solvents, as in aspiration of vomitus fluid, may produce chemical pneumonia. Existing

respiratory disorders and lung diseases may be aggravated by inhalation of vapors.

CHRONIC EFFECTS OF OVEREXPOSURE:

Repeated or prolonged inhalation exposure may produce reversible lung damage. Skin which is repeatedly

defatted by contact with solvents may be more susceptible to irritation, infection, and dermatitis.

None of the hazardous ingredients are listed as carcinogens by IARC, NTP, & OSHA

EST'D PEL/TLV: Not established    PRIMARY ROUTES OF ENTRY: Inh, Skin.

HMIS CODES: HEALTH 1; FLAM. 2; REACT. 0; PERS. PROTECT. B ; CHRONIC HAZ. YES

FIRST AID PROCEDURES:

SKIN: Wash contaminated skin with soap or a mild detergent. Get medical attention if irritation develops.

EYES: Immediately flush eyes with plenty of water for at least 15 minutes, occasionally lining upper and

lower lids. Get medical attention at once.

INHALE: If symptoms occur, move affected person to fresh air. If symptoms persist, get medical attention

promptly.

INGEST: Aspiration hazard - do not induce vomiting. If vomiting occurs, keep head below hip level. Get

emergency medical attention immediately.

SECTION IV - SPECIAL PROTECTION INFORMATION

ROTECTIVE CLOTHING: Wear neoprene, nitrile, or natural rubber gloves or gloves with proven resistance

Page:     1 of 2

[ to the ingredients listed.

I EYE PROTECTION: Wear tight-fitting safety glasses especially if contact lenses are worn.

RESPIRATORY PROTECTION: Keep face away from spray mist and do not breathe vapors. J^VENTILATTON: If vapors are detected, ventilate work area by opening windows and using exhaust fans.

SECTION V - PHYSICAL DATA

BOILING POINT (F):        310-600	SPECIFIC GRAVITY:	0.82

VAPOR PRESSURE(mmHg):     - 2MM @70F         EVAPORATION RATE (BUTYL ACETATE=1):  < 0.5

VAPOR DENSrTY(AIR=l):     > AIR	pH(CONCENTRATE):	N/A

pH(USE DILUTION OF ):	N/A

VOC CONTENT (CONCENTRATE): 75.0% 5.15 Ib/gl

APPEARANCE AND ODOR: A CLEAR, DARK BROWN LIQUID WITH A STRONG, PLEASANT FRAGRANCE.

SECTION VI - FIRE AND EXPLOSION DATA

FLASH POINT(F) (METHOD USED): 115	(TCC )

FLAMMABLE LIMITS: LEL: 0.7    UEL: 6.5

EXTINGUISHING MEDIA: Carbon dioxide, dry chemical and foam.

SPECIAL FIRE FIGHTING: Wear self-contained positive pres. breathing apparatus.

UNUSUAL FIRE HAZARDS; Fire exposed drums should be cooled with stream of water.

SECTION VII - REACTIVITY DATA

STABILITY: Stable

INCOMPATIBLILITY(AVOID): Heat, open flame, spark, and oxidizing agents.

POLYMERIZATION: Will not occur.

HAZARDOUS DECOMPOSITION: Carbon dioxide, carbon monoxide, and other unidentified organic compounds.

SECTION VIII - SPILL AND DISPOSAL PROCEDURES

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED:

Immediately eliminate all flame, ignition and high-heat sources. Absorb spill on inert absorbent material.

Pick up and place residue in a clean, D.O.T. specification container for disposal. Wash area thoroughly with a

detergent solution and rinse well with water.

WASTE DISPOSAL METHOD:

Liquids cannot be sent to landfills unless solidified Unusable product and collected, spent material may

require disposal as a hazardous waste at a permitted rreatment/storage/disposal facility. Solvent wastes may

require treatment to meet the appropriate standards before disposal in a chemical or industrial waste

ndfill. Consult local, state, and federal agencies for proper disposal method in your area.

.CRA HAZ. WASTE NOS.: D001

SECTION IX - SPECIAL PRECAUTIONS

PRECAUTIONS TO BE TAKEN WHEN HANDLING AND STORING:

Combustible! Store and use away from heat, sparks, open flame, or any source of ignition.

Keep product away from skin and eyes.

Keep away from food and food products.

Clothing or shoes which become contaminated with substance should be removed promptly and not reworn until

thoroughly cleaned

Keep out of the reach of children.

SECTION X - REGULATORY INFORMATION

DOT PROPER SHIPPING NAME: INDUSTRIAL CLEANERS N.O.I..LIQUID KEEP FROM

FREEZING

NOTE: DOT information applies to larger package sizes of affected products. For some products, DOT may

require alternate names and labeling in accordance with packaging group requirements.

DOT HAZARD CLASS: NA	DOT PACKING GROUP:

DOT I.D. NUMBER:	DOT LABEL/PLACARD:

EPA TSCA CHEMICAL INVENTORY - ALL INGREDIENTS ARE LISTED

EPA CWA 40CFR PART 117 SUBSTANCE(RQ IN A SINGLE CONTAINER): NONE

Page:     2 of 2

NOTICE

Thank you for your interest in, and use of, NSI products. NSI Chemicals Group, is pleased to be of service to you by supplying this Material Safety Data Sheet for your files. NSI Chemicals Group is concerned for your health and safety. NSI products can be used safely with proper protective equipment and proper handling practices consistent with label instructions and the MSDS. Before using any NSI product, be sure to read the complete label and the Material Safety Data Sheet.

As a further word of caution, NSI wishes to advise that serious accidents have resulted from the misuse of "emptied" containers. "Empty" containers retain residue (liquid and/or vapor) and can be dangerous. DO NOT pressurize, cut, weld, braze, solder, drill, grind or expose such containers to heat, flame, or other sources of ignition; they may explode or develop harmful vapors and possibly cause injury or death. Clean empty containers by triple rinsing with water or an appropriate solvent. Empty containers must be sent to a drum reconoitioner before reuse.

TERMS AND ABBREVIATIONS LISTED ALPHABETICALLY BY SECTION

SECTION II: HAZARDOUS INGREDIENTS

CAR: Carcinogen - A chemical listed by the National Toxicology

Program (NTP), the International Agency for Research on Cancer

(IARC) or OS HA as a definite or possible human cancer causing

agent.

CAS #: Chemical Abstract Services Registry Number - A univers

ally accepted numbering system for chemical substances.

CBL: Combustible - At temperatures between 100F and 200F

chemical gives off enough vapor to ignite if a source of

ignition is present as tested with a closed cup tester.

CNS: Central Nervous System depressant which reduces the activity

of the brain and spinal cord.

COR: Corrosive - Causes irreversible injury to living

tissue (e.g. bums).

DESIGNATIONS: Chemical and common names of hazardous

ingredients.

EIR: Eye Irritant Only - Causes reversible reddening and/or

inflammation of eye tissues.

EXPOSURE LIMrrS: The time weighted average (TWA) airborne

concentration at which most workers can be exposed without any

expected adverse effects. Primary sources include ACGIH TLVs,

and OSHA PELs (TWA, STEL and ceiling limits).

ACGIH: American Conference of Governmental Industrial Hygienists.

CEILING: The concentration that should not be exceeded in the workplace during any part of the working exposure. OSHA: Occupational Safety and Health Administration PEL: Permissible Exposure Limit - A set of time weighted average exposure values, established by OSHA, for a normal 8-hour day and a 40-hour work week. PPM: Partsper million - unit of measure for exposure limits. (S) SKIN: Skin contact with substance can contribute to overall exposure.

STEL: Short Term Exposure Limit - Maximum concentration for a continuous 15-minute exposure period. TLV: Threshold Limit Value - A set of time weighted average exposure limits, established by the ACGIH, for a normal 8-hour day and a 40-hour work week.

FBL: Flammable - At temperatures under 100F, chemical gives off enough vapor to ignite if a source of ignition is present as tested with a closed cup tester.

HAZARDOUS INGREDIENTS: Chemical substances determined to be potential health or physical hazards based on the criteria established in the OSHA Hazard Communication Standard - 29 CFR 1910.1200

HTX: Highly toxic - the probable lethal dose for a 70kg (150 Ib.) man and may be approximated as less than 6 teaspoons (2 tablespoons).

IRR: Irritant - Causes reversible effects in living tissues (e.g. inflammation) - primarily skin and eyes. N/A: Not Applicable - Category is not appropriate for this product.

N/D: Not Determined - Insufficient information to make a determination for this item.

RTECS#: Registry of Toxic Effects of Chemical Substances - an unreviewed listing of published toxicology data on chemical substances.

SARA: Superfund Amendment and Reauthorization Act - Section 313 designates chemicals for possible reporting for the Toxics Release Inventory.

SEN: Sensitizer - Causes allergic reaction after repeated exposure,

TOX: Toxic - The probable lethal dose for a 70 kg (150 Ib.) man is one ounce (2 tablespoons) or more.

(rev. 1/98)

SECTION III: HEALTH HAZARD DATA

ACUTE EFFECT: An adverse effect on the human body from a single

exposure with symptoms developing almost immediately after

exposure or within a relatively short time.

CHRONIC EFFECT: Adverse effects that are most likely to occur

from repeated exposure over a long period of time.

EST D PEL/TLV: This estimated, time-weighted average, exposure

limit, developed by using a formula provided by the ACGIH,

pertains to airborne concentrations from the product as a

whole. This value should serve as guide for providing safe

workplace conditions to nearly all workers.

HMIS CODES: Hazardous Material Identification System - a rating

system developed by the National Paint and Coating Association

for estimating the hazard potential of a chemical under normal

workplace conditions. These risk estimates are indicated by a

numerical rating given in each of three hazard areas

(Health/Flarnrnability/Reactivity) ranging from a low of zero to

a high of 4. The presence of a chronic hazard is indicated with a yes.

Consult HMIS training guides for Perspnal Protection letter

codes which indicate necessary protective equipment.

PRIMARY ROUTE OF ENTRY: The way one or more hazardous

ingredients may enter the body and cause a generalized-systemic

or specific-organ toxic effect.

ING: Ingestion - A primary route of exposure through

swallowing of material

INH: Inhalation - A primary route of exposure through

breathing of vapors.

SKIN: A primary route of exposure through contact with

the skin.

SECTION IV: SPECIAL PROTECTION INFORMATION

ended, use only MSHA and

; Safety and Health Administration NIOSH: National Institute for Occupational Safety and Health

SECTION V: PHYSICAL DATA

EVAPORATION RATE: Refers to the rate of change from the
liquid state to the vapor state at ambient temperature and
pressure in comparison to a given substance (e.g. water).
_tt. a ___,	^_.L_     'JJty or alkalinity of an

•Jeutral pH = 7; Alkaline pH = 14)

: The percentage or amount in pounds per

gallon of the product that is regulated as a Volatile Organic

Compound under the Clean Air Act of 1990 and various state

jurisdictions.

SOLUBILITY IN WATER: A description of the ability of the

product to dissolve in water.

SECTION VII: REACTIVITY DATA

HAZARDOUS DECOMPOSITION: Breakdown products expected to be produced upon product decomposition by extreme heat or fire. INCOMPATIBILITY: Material contact by extreme heat and the conditions to avoid to prevent hazardous reactions. POLYMERIZATION: Indicates the tendency of the product's molecules to combine with themselves in a chemical reaction, releasing excess pressure and heat.

STABILITY: Indicates the susceptibility of the product to spontaneously and dangerously decompose.

SECTION VIII: SPILL AND DISPOSAL PROCEDURES

RCRA WASTE NOS: RCRA (Resource Conservation and Recovery Act) waste codes (40 CFR 261) applicable to the disposal of spilled or unusable product from the original container.

SECTION X: TRANSPORTATION DATA

CWA: Clean Water Act- Federal Law which regulates chemical

releases to bodies of water.

RQ: Reportable Quantity - The amount of the specific ingredient

that, when spilled to the ground and can enter a storm sewer or

natural watershed, must be reported to the National Response

Center, and other regulatory agencies.

TSCA: Toxic Substances Control Act - a federal law requiring

all commercial chemical substances to appear on an inventory

maintained by the EPA.

DISCLAIMER

All statements, technical information and recommendations contained herein are based on available scientific tests or data which we believe to be reliable. The accuracy and completeness of such data are not warranted or guaranteed. We cannot anticipate all conditions under which this information and our products, or the products of other manufacturers in combination with out products, may be used. NSI assumes no liability or responsibility for loss or damage resulting from the improper use or handling of our products, from incompatible product combinations, or from the failure to follow instructions, warnings, and advisories in the products label and Material Safety Data Sheet.