FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Material Safety Data Sheet (MSDS)

msds #•

Revision Date:   November 25, 2002

Section 1 — Chemical Product and Company Identification

Potassium Bitartrate

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition. Information on Ingredients

Potassium Bitartrate

Synonyms: cream of tarter, potassium acid tartrate

CAS#: 868-14-4

Section 3 — Hazards Identification

White crystals or powder. Odorless.

Substance not considered hazardous. However, not all health aspects of this substance have been

thoroughly investigated.

flinn Ai-A-GLANCE

Health-0

Flammability-0

Reactivity-0

Exposure-0

Storage-0

0 is low hazard, 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 1 5 minutes.

External: Wash continuously with fresh water for 1 5 minutes.

Internal: Give large quantities of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Non flammable, non combustible solid.

NFPA code

None Established

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #2. Store with acetates, halides, sulfates, sulfites, thiosulfates and phosphates.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron.

© 2002 Flinn Scientific, Inc. All Rights Reserved.

page 1 of 2

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Material Safety Data Sheet (MSDS)

Potassium Bitartrate

MSDS#:    614.00

Revision Date:   November 25, 2002

Section 9 — Physical and Chemical Properties

White crystals or powder. Odorless.

Solubility: Soluble in boiling water; insoluble in alcohol.

Formula: KHC4H4O6

Formula Weight: 188.18

Specific Gravity: 1.984

Section 10 — Stability and Reactivity

Shelf life: Indefinite.

Section 11 —Toxicoloqical Information

Acute effects: N.A. Chronic effects: N.A. Target organs: N.A.

ORL-RAT LD50: N.A. IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

N.A. = Not available, not all health aspects of this substance have been fully investigated.

Section 12— Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option.

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (212-769-1).

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Reduce Science Accidents-Use Flinn Chemicals

flinn@flinnsci.com   www.flinnsci.com

P.O. Box 219   BatavialL 60510

(800) 452-1261 Fax (866) 452-1436

© 2002 Flinn Scientific, Inc. All Rights Reserved.

page 2 of 2