(000000-000000-

-5620

DATE   OF   ISSUE 5/07/1997

SUPERSEDES 4/20/1994

SECTION   I   -   GENERAL   INFORMATION

Chemical  Name   &   Synonyms N/A

Trade Name   &   Synonyms PRINTER'S   CHOICE  AEROSOL

o.cal   Family: 1LEUM DISTILLATES

Formula Mixture   —> X

ba^nuf ac turer ' s  Name:

MANTEK,   DIVISION  OF  NCH  CORP.

Address:

BOX 152170

IRVING,         TEXAS 75015

Prepared By:

B MALKE*   /  CHEMIST

Product  Code Number 5620

Emergency Phone  Number 800-424-9300

SECTION II - HAZARDOUS INGREDIENTS THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS

Ch^fm cal Name (Ingredients)

V M £ P NAPHTHA

XYLENE

PROPANE

ISOBUTANE

N-BUTANE

Hazard

FLAMM/IRR.

FLAMM/IRR.

ASPHYX.

FLAMM/IRR.

FLAM/IRR

TLV

300PPM 1. 100PPM 1. NOT EST.1. NOT EST.l. 800PPM 1.

PEL

1350MG/M32 100PPM 2. 1000PPM 2. NOT EST.2. 800PPM  2.

STEL NOT EST. 150PPM NOT EST. NOT EST. NOT EST.

CAS *

64742-89-8

1330-20-7

74-98-6

75-28-5

106-97-8

SECTION III - PHYSICAL DATA

Boiling Point (F):	245-300	Specific Gravity	(H20=l):	0.75-0.77

Vapor Pressure (MM HG) :	40	Color:	WHITE

Vapor Densi ty (Air=l) :	4.0	Odor:	LEMON

PH @ 100% :	4	Clarity:	OPAQUE

% Volatile by Volume:	90	Evaporation Rate	(BU A/C=1) :	XL

Solubility:	NEGLIGABLE	Viscosity:	NON^VISc6u~S~ ~

SECTION IV - FIRE AND EXPLOSION HAZARD

Flash Point	Flammable Limits	LEL	UEL

<73°F.   I SETA	ISOBUTANE	1.8	7.0

Extinguishing Media

X <--Foam	<—Alcohol Foam       X <—C02       X <--Dry Chemical	<—Water Spray      <—Other

Special Fire Fighting Procedures:

AS WITH ALL FIRES INVOLVING CHEMICALS, FIREFIGHTERS SHOULD WEAR FULL PRO- TECTIVE GEAR INCLUDING A SELF CONTAINED BREATHING APPARATUS.  USE WATER

SPRAY TO COOL EXPOSED CONTAINERS TO PREVENT RUPTURING OF CONTAINERS.

Unusual Fire and Explosion Hazards:

DUE TO AEROSOL PACKAGING, DO NOT EXPOSE CANS TO HIGH HEAT OR OPEN FLAME AS EXPLOSION CAN OCCUR, FLAME EXTENSION - 30+ INCHES;  BURNBACK - 6 INCHES

Aerosol Level (NFPA 30B):     3

NFPA 704 Hazard Rating       {0=Insignificant l=Slight  2=Moderate  3=High  4=Extreme) 3  <—Health       4  <—Flammability  0  <—Instability      <—Special

„_____	 SECTION V - HEALTH HAZARD DATA

Threshold Limit Value:

NOT ESTABLISHED FOR MIXTURE.  SEE SECTION II.

Effects of Overexposure:

-Acute (Short Term Exposure)

INHALATION; MAY CAUSE HEADACHE, NASAL AND RESPIRATORY IRRITATION, NAUSEA DROWSINESS, FATIGUE, PNEUMONITIS, PULMONARY EDEMA, CENTRAL NERVOUS SYSTEM DEPRESSION.  PROPELLANT IS A SIMPLE ASHPHYXIANT AND MAY CAUSE HEADACHE, DIZZINESS, DISORIENTATION AND ANETHESIA.  EYE CONTACT: MAY CAUSE IRRITATION SEEN AS ITCHING AND REDNESS.  SKIN CONTACT: MAY CAUSE IRRITATION SEEN AS ITCHING AND REDNESS.  INGESTION: MAY CAUSE GASTROINTESTINAL IRRITATION SEEN AS NAUSEA, VOMITING AND DIARRHEA. INGESTION AND SUBSEQUENT VOMITING OF THIS PRODUCT CAN LEAD TO ASPIRATION OF THE PRODUCT INTO THE LUNGS WHICH CAN CAUSE DAMAGE AND MAY BE FATAL.

-Chronic (Long Term Exposure) MARKED IRRITATION OF SKIN AND EYES UPON CHRONIC OVEREXPOSURE.  TARGET ORGANS: LIVER, KIDNEYS CENTRAL NERVOUS SYSTEM.

f^' ' lary Routes of Entry:     X <--Inhalation       O-Ingestion        <--Absorption

,rgency and First Aid Procedures:

-Inhalation: IF INHALED, REMOVE FROM AREA TO FRESH AIR.  SEEK MEDICAL ATTENTION IF RESP>r-ATORY IRRITATION DEVELOPS OR IF BREATHING BECOMES DIFFICULT.

-Eye Contact:

SECTION V - HEALTH HAZARD DATA (Continued)

FOR EYE CONTACT, RINSE EYES WITH WATER.  REMOVE ANY CONTACT LENSES, AND CONTINUE FLUSHING WITH PLENTY OF WATER FOR SEVERAL MINUTES.  SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS.

-Skin Contact:	^

WASH AFFECTED AREAS WITH PLENTY OF SOAP AND WATER FOR SEVERAL MINUTES. SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS.

-Ingestion: GIVE 3 TO 4 GLASSES OF WATER, BUT DO NOT INDUCE VOMITING. IF VOMITING OCCURS, GIVE FLUIDS AGAIN. SEEK MEDICAL ATTENTION IF DISCOMFORT OCCURS.

-Notes to Physician: INGESTION AND SUBSEQUENT VOMITING OF THIS PRODUCT CAN LEAD TO ASPIRATION OF THE PRODUCT INTO THE LUNGS WHICH CAN CAUSE DAMAGE AND MAY BE FATAL.

SECTION VI - TOXICITY INFORMATION

Product Contains  Chemicals Listed as Carcinogen or Potential  Carcinogen By:

IARC-->  No	NTP—>  No	OSHA—>  No	ACGIH—>  No	OTHER—> No

V M  S  p  NAPHTHA	ISOBUTANE

1HL-RAT   LCiiO:	3400PPM/4H   3.	NO  DATA AVAILALBE

IPR-RAT   LDL) :	8560MG/M3   3.

IHL-MU3   LCLO:	500000M3/M3   3.
XYLENE

3RL-RAT   LD50:	4300MG/KS   3.

EYE-flMN   200   PPM  3.	PROPANE

IHL-RAT   LC50:	5000PPfc/4H   3.	NO  DATA AVAILABLE

IHL-MAN   LCLO:	10000PPM/6H     3.

SCU-RAT   LD50:	1700MG/M3      3.
S- BUTANE

IHL-RAT   LC50:	658G/M3/4H  3.

iHL-MUS   LC50:	680G/M3/2H   3.

SECTION VII - REACTIVITY DATA

Stability:         X <--Stable      <—Unstable Conditions to Avoid: HEAT,HOT SURFACES, SPARKS, OPEN FLAME.

Incompatibility (Materials to Avoid) :

STRONG OXIDIZERS (SUCH AS CHLORINE, LIQUID OXYGEN) .

Hazardous Decomposition Products: OXIDES OF SILICON AND CARBON.

Hazardous Polymerization:	<--May Occur      X <—Will Not Occur

Conditions to Avoid: K/A

SECTION VIII - SPILL OR LEAK PROCEDURES

Steps to be Taken if Material is Released or Spilled:

DUE TO AEROSOL PACKAGING, LARGE SPILLS ARE UNLIKELY.  IN THE EVENT OF A SMALL SPILL, ELIMINATE IGNITION SOURCES AND PICK UP MATERIAL WITH A CLOTH AND

WASH AREA WITH A DETERGENT.

Waste Disposal Method(s):

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL, STATE, AND LOCAL REGULATIONS. TYPICAL DISPOSAL IS TO WRAP THE EMPTY AEROSOL CONTAINER IN SEVERAL LAYERS OF

NEWSPAPER AND DISPOSE OF IN THE TRASH.

Neutralizing Agent: N/A

SECTION IX - SPECIAL PROTECTION INFORMATION

Required Ventilation:

GENERAL VENTILATION IS NORMALLY ADEQUATE.

Respiratory Protection:

NONE REQUIRED UNDER NORMAL CONDITIONS OF USE.

Glove Protection:

NEOPRENE RUBBER IF REPEATED OR PROLONGED SKIN CONTACT IS LIKELY.

Eye Protection:

SAFETY GLASSES IF THE METHOD OF USE PRESENTS THE LIKELIHOOD OF EYE CONTACT.

Other Protection: N/A

SECTION X - STORAGE AND HANDLING INFORMATION

Storage Temperature:     Indoors —> X	Outdoors-->	Heated-->	Refrigerated—>

Minimum Temperature: 20°F.   Maximum Temperature: 120°F.

Precautions to be Taken in Handling and Storing:

DO NOT USE OR STORE NEAR HEAT, HOT SURFACES, SPARKS OR OPEN FLAME. STORE IN A COOL AREA.

Other Precautions:	-f.

KEEP OUT OF REACH OF CHILDREN. READ THE ENTIRE LABEL BEFORE USING THIS PRODUCT-

SECTION X - STORAGE AND HANDLING INFORMATION (Continued)

SECTION XI - REGULATORY INFORMATION

Chemical Name	CAS Number	'Opper % Limit

XYLENE    ~	1330-20-7	5

TJJs^e ingredients listed above are subject to the reporting requirements of 313 of Title III of the Superfund Amendments and Reauthorization Act of md 40 CFR part 372.

Please call 1-800-527-9919 for additional information if you are a California customer. This MSDS is not intended for users in the state of California.

SECTION XII - REFERENCES

- THRESHOLD LIMIT VALUES FOR CHEMICAL SUBSTANCES AND PHYSICAL AGENTS
AND BIOLOGICAL EXPOSURE INDICES, ACGIH, 1996-1997.

- OSHA PEL.

- SAX'S DANGEROUS PROPERTIES OF INDUSTRIAL MATERIALS, EIGHTH EDITION,
RICHARD J. LEWIS, SR.

IRR: IRRITANT, FLAM/FLAMM: FLAMiABLE, COMB: COMBUST IBLE, CORR: CORROSIVE, CARC:CARCINOGENIC, TOXrTOXIC, N/A:NOT APPLICABLE, N/E:NOT ESTABLISHED, COC:CLEVELAND OPEN CUP, PMCC: PENSKY-MARTIN CLOSED CUP, TCC: TAGLIABUE CLOSED CUP, LEL:LOWER EXPLOSION LIMIT, UELlUPPER EXPLOSION LIMIT, NFPA:NATIONAL FIRE PROTECTION ASSOCIATION, IARC: INTERNATIONAL AGENCY FOR THE RESEARCH ON CANCER, NTP:NATIONAL TOXICOLOGY PROGRAM, OSHA: OCCUPATIONAL SAFETY £ HEALTH ADMINISTRATION, ACGIH:AMERICAN CONFERENCEOF GOVERNMENTAL INDUSTRIAL HYGIENISTS, TLV:THRESHOLD LIMIT VALUE, PEL: PERMISSIBLE EXPOSURE LEVEL, STEL:SHORT TERM EXPOSURE LIMIT, MLD:MILD, MOD:MODERATE, SEV:SEVERE, MUT:MUTAGENIC, ASPHYX:ASPHYXIANT

THE INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORMULATION.  HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THESE DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF.

MANTEK, DIVISION OF NCH CORP. assumes no responsibility for personal injury or property damage caused by the use, storage, or disposal of the product in a manner not recommended on the product label.  Users assume all risks associated with such unrecommended use, storage, or disposal of the product.