MATERIAL SAFETY DATA SHEET

Revised: 05/15/08

FLYNAP	Replaces: 04/10/08

CAROLINA BIOLOGICAL	Printed: 05/20/08

1.	PRODUCT DESCRIPTION

Product Name:      FlyNap

Product Code(s):   17-3010, 17-3015, 17-3025, 17-3048,.17-3050, 17-3052,

17-3060, 74-6530, 17-1950, 17-1951, 17-1958, 17-1959,

17-1960, 17-1964, 17-1966, 17-1984, 17-1987, 17-1990,

17-1993

Size:	10 mL, 100 mL

Chemical Name:     Product is a mixture
CAS Number:        See section 2
Formula:	See section 2

Synonyms:         None known Distributor:       Carolina Biological Supply Company

2700 York Road

Burlington, NC 27215

Chemical Information:  800-227-1150 (8am-5pm (ET) M-F) Chemtrec (Transportation Spill Response 24 hours): 800-424-9300

2.	COMPOSITION/INFORMATION ON INGREDIENTS

Principal Hazardous Components:  Triethylamine (CAS# 121-44-8) 50%;

Ethanol(CAS# 64-17-5) 25%; Fragrance (neutralizer) 25%

TLV and PEL units:

Triethylamine:  ACGIH-TLV 1 ppm (TWA) skin, 3 ppm (STEL) skin;

OSHA-PEL 25 ppm (TWA). Ethanol:  ACGIH-TLV  lOOOppm (TWA); OSHA-PEL lOOOppm (TWA).

3.	HAZARD IDENTIFICATION

Triethylamine is a severe irritant to eyes and mucous membranes.

Prolonged exposure to triethylamine may cause damage to heart, liver

and/or kidneys.

Emergency Overview:

Potential Health Effects:

Eyes:  Corrosive.  Liquid contact with eyes may cause extreme irritation

and corneal damage.  Vapors may cause irritation.

Skin:  Corrosive.  May cause irritation including redness, pain, and

skin burns.  Triethylamine is absorbed through the skin.

Ingestion:  Corrosive.  May cause gastrointestinal discomfort.

Triethylamine may cause burns to the mouth, throat and stomach.

Aspiration into lungs may cause lung damage.

Inhalation:  Corrosive.  May cause irritation to the respiratory tract.

4.	FIRST AID MEASURES

Emergency and First Aid Procedures:

Eyes - Flush with water for at least 15 minutes, raising and lowering

eyelids occasionally.  Get medical attention at once.

Skin - Thoroughly wash exposed area for at least 15 minutes.  Remove

contaminated clothing.  Launder contaminated clothing before reuse.  Get

medical attention at once.

Ingestion - Do not induce vomiting.  If swallowed, if conscious, give

water immediately and get medical attention at once.  Never give

anything by mouth to an unconscious person.

Inhalation - Remove to fresh air.  Give oxygen if breathing is

difficult;  give artificial respiration if breathing has stopped.  Keep

person warm and quiet, and get medical attention at once.

5.	FIREFIGHTING PROCEDURES

Page   1   of   4

MATERIAL SAFETY DATA SHEET

Revised: 05/15/08

FLYNAP	Replaces: 04/10/08

CAROLINA BIOLOGICAL	Printed: 05/20/08

Flash Point(Method Used):     -9°C (closed cup) for triethylamine

13°C (closed cup) for ethanol NFPA Rating:      Health: 3 Fire:  3 Reactivity:  0 Extinguisher Media:

Use dry chemical, CO2 or appropriate foam. Flammable Limits in Air % by Volume: Triethylamine: LEL 1.2, UEL 8.0 Ethanol:  LEL 3.3 (®60°C) , UEL 19.0 (@60°C) Autoignition Temperature: No information available Special Firefighting Procedures:

Firefighters should wear full protective equipment and NIOSH approved self-contained breathing apparatus.

Unusual Fire and Explosion Hazards:  Triethylamine vapors are heavier than air and may travel along the ground to a distant source of ignition and flash back.

6.	SPILL OR LEAK PROCEDURES

Steps to be Taken in Case Material is Released or Spilled:

Ventilate area of spill.  Eliminate all sources of ignition. Remove all non-essential personnel from area.  Clean-up personnel should wear proper protective equipment and clothing.  Absorb material with suitable absorbent and containerize for disposal.

7.	SPECIAL PRECAUTIONS

Precautions to be Taken in Handling or Storing:  Store tightly closed in a cool, dry, well ventilated area suitable for the storage of flammable liquids. Other Precautions:  Keep all oxidizing materials and acids away.

8.	SPECIAL PROTECTION INFORMATION

Respiratory Protection(Specify Type):

A NIOSH/MSHA chemical cartridge respirator should be worn if PEL or TLV is exceeded. Ventilation:

Local Exhaust:  Yes Mechanical(General):  Yes Special:  No Other:   No Protective Gloves:

Rubber, neoprene, PVC, or equivalent. Eye Protection:

Splash proof chemical safety goggles should be worn at all times. Other Protective Clothing or Equipment:

Lab apron, eye wash, and safety shower.

9.	PHYSICAL DATA

Molecular Weight: N/A; material is a mixture.

Melting Point: Triethylamine: -115°C; Ethanol: -114.5°C

Boiling Point: Triethylamine: 89.3°C; Ethanol: 78°C

Vapor Pressure: Triethylamine: 54mmHg®20°C; Ethanol: 59.3mmHg@25°C

Vapor Density(Air=l): 3.5 for triethylamine; 1.59 for ethanol

Specific Gravity(H2O=1): .73 for triethylamine; .789 for ethanol

Percent Volatile by Volume:  100%

Page 2 of 4

MATERIAL SAFETY DATA SHEET

Revised: 05/15/08

FLYNAP	Replaces: 04/10/08

CAROLINA BIOLOGICAL	Printed: 05/20/08

Evaporation Rate(H20=l):  Greater than 1

Solubility in Water:  Completely miscible

Appearance and Odor:  Yellowish solution with strong fragrant odor.

10. REACTIVITY DATA

Stability:  Stable in ordinary conditions of use and storage.

Triethylamine is strongly alkaline, and will react strongly with acids,

producing great heat.

Conditions to Avoid:  Heat, open flame, sources of ignition,

incompatible materials.

Incompatibility(Materials to Avoid):  Acids, oxidizers, chlorine,

hypochlorite, halogenated compounds, and nitrogen tetraoxide.

Hazardous Decomposition Products: Carbon dioxide, carbon monoxide,

nitrogen oxides.

Hazardous Polymerization:  Will not occur

11.TOXICITYDATA

Toxicity Data:  Triethylamine  ihl-rat LClo:  1000ppm/4H

orl-rat LD50:  460mg/kg Ethanol  orl-rat LD50:  7060 mg/kg

ihl-rat LC50:  20000 ppm/lOH Effects of Overexposure:

Acute:  See section 3

Chronic:  Triethylamine:  Mutation data cited.  Reproductive Effects data cited.  Not listed as causing cancer by IARC, NTP, or OSHA. Ethanol:  Mutation data cited.  Tumorigenic data cited.  Not listed as causing cancer by IARC, NTP ,or OSHA.

Conditions Aggravated by Overexposure: Preexisting conditions of the skin, eyes, liver and/or kidneys.

Target Organs:  Prolonged Overexposure to triethylamine may cause damage to the heart, liver, and/or kidneys. Primary Route(s) of Entry:  Ingestion and skin

12.	ECOLOGICAL DATA

EPA Waste Numbers:  Triethylamine  D001,  Ethanol  D001

13.	DISPOSAL INFORMATION

Waste Disposal Methods:  Dispose in accordance with all applicable

Federal, State and Local regulations.

Always contact a permitted waste disposer (TSD) to assure compliance.

14.	TRANSPORT INFORMATION

Flammable Liquid, corrosive, n.o.s. (contains Ethyl Alcohol,

Triethylamine), 3, UN2924, II

Label (s) Required:  FLAMMABLE LIQUID, CORROSIVE

15.	REGULATORY INFORMATION

EPA TSCA Status:  On TSCA Inventory

Hazard Category for SARA Section 311/312 Reporting: Acute, Chronic, Fire

| SARA Sec. 313 SARA EHS Chemicals Product or Sec. 302 Name Chemical Components TPQ List Category | CERCLA RCRA Sec. 103 Sec. RQ Ibs. 261.33 |
| --- | --- |
| Triethylamine No No No | No No |
| Page 3 of 4 |  |

MATERIAL SAFETY DATA SHEET

Revised: 05/15/08

FLYNAP	Replaces: 04/10/08

CAROLINA BIOLOGICAL	Printed: 05/20/08

Ethanol	No         No     No	No        No

16. ADDITIONAL INFORMATION

The information provided in this Material Safety Data Sheet represents e compilation of data drawn directly from various sources available to us. Carolina Biological Supply makes no representation or guarantee as to the suitability of this information to a particular application of the substance covered in the Material Safety Data Sheet.  Any employer must carefully assess the applicability of any information contained herein in regards to the particular use to which the employer puts the material. Glossary

ACGIH	American Conference of Governmental Industrial Hygienists

CAS Number..Chemical Abstracts Service Number

CERCLA	Comprehensive Environmental Response, Compensation, and

Liability Act

DOT. ....'... .U.S. Department of Transportation

IARC	International Agency of Research on Cancer

mppcf	million particles per cubic foot

N/A	Not Available

NTP	National Toxicology Program

OSHA	Occupational Safety and Health Administration

PEL	Permissible Exposure Limit

ppm	parts per million

RCRA	Resource Conservation and Recovery Act

SARA	Superfund Amendments and Reauthorization Act

TLV	Threshold Limit Value

TSCA	Toxic Substances Control Act