flinn scientific, inc.

r   Material Safety Data Sheet (MSDS)	MSDS #: 53LO°

Revision Date;May 20, 2008

SECTION 1 — CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

Naphthalene

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

SECTION 2 — COMPOSITION. INFORMATION ON INGREDIENTS

Naphthalene CAS#: 91-20-3

SECTION 3 — HAZARDS IDENTIFICATION

White, volatile flakes, cubes, spheres or powder. Odor of moth balls.	flinn Ar-A-GLANCE

Moderately toxic by inhalation, ingestion, and skin contact. Possible carcinogen.	Health-2

Irritating to body tissues. Avoid body tissue contact.	Flammability-1

Reactivity-0 Exposure-1 Storage-0

0 is low hazard, 3 is high hazard

SECTION 4 — FIRST AID MEASURES

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Rinse out mouth, give 1 to 2 cups of water or milk, induce vomiting. After vomiting, give mixture of 2 Tbs. of activated

charcoal mixed with one cup of water. Call a physician or poison control at once.

SECTION 5 — FIRE FIGHTING MEASURES

Combustible solid.	.	NFPA code

Flash Point: 174 F    Upper: 5.9%   Lower: 0.9%    Autoignition Temperature: 978 F	H-2

When heated to decomposition, emits acrid smoke and irritating fumes.	F-2

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE	R-0

and SCBA with full facepiece operated in positive pressure mode.

SECTION 6 — ACCIDENTAL RELEASE MEASURES

Restrict unprotected personnel from area. Remove all ignition sources and water . Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

SECTION 7 — HANDLING AND STORAGE

Flinn Suggested Chemical Storage Pattern: Organic #3. Store with hydrocarbons, esters and aldehydes. Store in a cool dry place. Use and dispense in a hood.

SECTION 8 — EXPOSURE CONTROLS. PERSONAL PROTECTION

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. ^~~    Use ventilation to keep airborne concentrations below exposure limits. Always wear a NIOSH-approved respirator with proper cartridges or a positive pressure, air-supplied respirator when handling this material in emergency situations (spill or fire). Exposure guidelines: TWA 10 ppm, STEL 15 ppm (OSHA, ACGIH)

©Flinn Scientific, Inc. All Rights Reserved.	PAGE 1 OF 2

flinn scientific, inc.

Material Safety Data Sheet     Naphthalene

MSDS#:   531.00

Revision Date: May 20, 2008

^

SECTION 9 — PHYSICAL AND CHEMICAL PROPERTIES

White, volatile flakes, cubes, sphere or powder.

Odor of moth balls.

Solubility: Soluble in benzene, absolute alcohol and ether;

insoluble in water.

Formula: C10H8   Formula Weight: 128.17

Specific Gravity: 1.145 Melting Point: 80.2 C Boiling Point: 217.7 C Vapor Pressure: 1mm (53 C) Vapor Density: 4.4

SECTION 10 — STABILITY AND REACTIVITY

Shelf life: Volatile flakes, keep tightly closed.

SECTION 11 — TOXICOLOGICAL INFORMATION

Acute effects: Toxic, irritant Chronic effects: Possible carcinogen. Target organs: Eyes, blood, kidneys

ORL-RAT LD50: 490 mg/kg IHL-RAT LC50: N.A. SKN-RAT LD50: >2500 mg/kg

N.A. = Not available, not all health aspects of this substance have been fully investigated.

SECTION 12 — ECOLOGICAL INFORMATION

Data not yet available.

SECTION 13 — DISPOSAL CONSIDERATIONS

~

Please consult with state and local regulations. Flinn Suggested Disposal Method 18b is one option.

SECTION 14 — TRANSPORT INFORMATION

Shipping Name: Napthalene, refined Hazard Class: 4.1, Flammable solid UN Number: UN 1334

N/A = Not applicable

SECTION 15 — REGULATORY INFORMATION

TSCA-listed, EINECS-listed (202-049-5), RCRA code U165.

SECTION 16 — OTHER INFORMATION

This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific, Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. The data should not be confused with local, state, federal or insurance mandates, regulations, or requirements and CONSTITUTE NO WARRANTY. Any use of this data and information must be determined by the science instructor to be in accordance with applicable local, state or federal laws and regulations. The conditions or methods of handling, storage, use and disposal of the product(s) described are beyond the control of Flinn Scientific, Inc. and may be beyond our knowledge. FOR THIS AND OTHER REASONS, WE DO NOT ASSUME RESPONSIBILITY AND EXPRESSLY DISCLAIM LIABILITY FOR LOSS, DAMAGE OR EXPENSE ARISING OUT OF OR IN ANY WAY CONNECTED WITH THE HANDLING, STORAGE, USE OR DISPOSAL OF THIS PRODUCT(S).

Consult your copy of the Flinn Chemical Catalog/Reference Manual for additional information about laboratory chemicals

XFlinn Scientific, Inc. All Rights Reserved.

page 2 of 2