Funn scientific

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

MSDS #:     475.00

Revision Date:    November 25, 2002

FLINN

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

Magnesium Hydroxide

MSDS #:     475.00

Revision Date:   November 25, 2002

Section 1 — Chemical Product and Company Identification

Section 9 — Physical and Chemical Properties

Magnesium Hydroxide

Flinn Scientific, Inc. P.O. Box2l9 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

White, odorless powder.

Solubility: Almost insoluble in water and alcohol. Soluble in

solutions of ammonium salts and dilute acids.

Formula: Mg(OH)2

Formula Weight: 58.34

Specific Gravity: 2.36

| Section 2 — Composition. Information on Inaredients |  |
| --- | --- |
| Magnesium Hydroxide Synonym: magnesium hydrate CAS#: 1309-42-8 |  |
| Section 3 — Hazards Identification |  |
| White, odorless powder. Substance not considered hazardous. However, not all health aspects of this substance have been thoroughly investigated. | flinn at-a-glance Healih-O FIammabiiitv-0 Reactivity-O' Exposure-0 Storage-0 0 is low hazard, 3 is high hazard |
| Section 4 — First Aid Measures |  |

Gail a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye; Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give large quantities of water. Call a physician or poison control at once.

Section 5 — Fire FiqhtincLMeasures

Not combustible solid.

NFPA code

None Established

Section 10 — Stability and Reactivity

Avoid contact with strong acids. Shelf Life: Indefinite.

Section 11 — Toxicological Information

Acute effects: Dust may be irritant

Chronic effects: Diarrhea and abdominal pain

Target organs: N.A.

N.A = Not available, not all health aspects of this substance have been fully investigated

gectjon 12 —_Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option.

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A

N/A = Not applicable

ORL-RAT LD50; N.A. IHL-RAT LC50: N.A.

SKN-RBT LD50: N.A.

Section 15 — Regulatory Information

TSCA-listed!ElNECS-iisted(215-I70-3).

Section 6 — Accidental Release Measures

Section 16 — Other information

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #4. Store with hydroxides, oxides, silicates and carbonates. Store in a cool dry place. Use and dispense in a hood.

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FUNN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Section 8 —-JExposure Controls . Personal Protection	.

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron.

Flinn Chemicals—Your Best Choice

flinnfoiflinnsci.com   www.flinnsci.com

P.O. Box 219   Batavia IL 60510 (800)452-1261 Fax (866) 452-1436

© 2002 Flinn Scientific, Inc.  Al! Rights Reserved.

page 1 of 2

C 2002 Flinn Scientific. Inc.  All Rights Reserved.