MATERIAL SAFETY DATA SHEET  :       M/M CHILL CLEAN	PAGE : O1

(O82833-O1 15S> -UO1257O6-SO93     ) ! DATE OF ISSUE	] SUPERSEDES	!

|    O4/14/95	| _ 12/16/93

SECTION I - GENERAL INFORMATION

CHEMICAL NAME & SYNONYMS	TRADE NAME & SYNONYMS

N/A	M/M CHILL CLEAN

CHEMICAL FAMILY	FORMULA

ALKALINE AQUEOUS SOLUTION	X<--MIXTURE_

MANUFACTURERS  NAME:

CERTIFIED LABS, DIV. OF NCH CORP.

ADDRESS (NUMBER, STREET, CITY, STATE & ZIP CODE)

BOX 152170

IRVING, TEXAS 75O15

PREPARED BY:	! PRODUCT CODE NUMBER | EMERGENCY TELEPHONE NUMBER

M JENKINS/CHEMIST	5O93	214-438-1381

SECTION II- HAZARDOUS INGREDIENTS

THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS.

CHEMICAL NAME (INGREDIENTS) : ISOBUTANE

HAZARD	>FLAMM       | TLV	>NOT EST.  1    PEL-->NOT EST. 2

STEL(TWA)* ->N/E	| CAS#-->75-28-5     :

CHEMICAL NAME (INGREDIENTS) : TRIETHANOLAMINE

HAZARD	>IRRITANT      TLV	>NOT EST.  1    PEL-->NOT EST. 2

STEL(TWA)* ->N/E	| CAS#-->1O2-71-6    |	;

CHEMICAL NAME (INGREDIENTS) :

POLY (OXYETHYLENE/OXYPROPYLENE)	GLYCOL

HAZARD	>IRR/MUT     )	TLV	>NOT EST.  1    PEL-->NOT EST. 2

STEL(TWA)* ->N/E	CAS#-->9OO3-11-6	,

M/M CHILL CLEAN

SECTION III - PHYSICAL DATA

PAGE : O2

BOILING PT.(FAHRENHEIT)

VAPOR PRESSURE (MM HG). VAPOR DENSITY (AIR=1)

212:

N/A

N/A

SPEC GRAVITY (H2.0-1)

WHITE

COLOR

:1.OO5

PH

10O%

CLARITY  TRANSPARENT

PERCENT, VOLATILE BY VOLUME (%)

SOLUBILITY IN WATER VISCOSITY

91

COMPLETE FOAMY

EVAPORATION RATE (BU AC = 1 )

N/A

SECTION IV - FIRE AND EXPLOSION HAZARD

FLASH POINT (METHOD USED) NON-FLAM  T.C.C.

flammable limits in/a

in/a

in/a

UEL

EXTINGUISHING   MEDIA       "ALCOHOL"
<--FOAM	X<--FOAM	XO-C02

DRY X<--CHEMICAL

WATER X<--SPRAY

<--OTHER

SPECIAL FIRE FIGHTING PROCEDURES

COOL FIRE EXPOSED AEROSOL CONTAINERS TO PREVENT RUPTURE.

FIREFIGHTERS SHOULD WEAR A SELF CONTAINED BREATHING APPARATUS AND FULL

PROTECTIVE GEAR.

UNUSUAL FIRE & EXPLOSION HAZARDS

DO NOT EXPOSE CONTAINER TO TEMPERATURES ABOVE 12O:F TO

PREVENT RUPTURING. OXIDES OF CARBON/NITROGEN MAY BE EVOLVED DURING

A FIRE.

LEVEL 1 AEROSOL (NFPA SOB)

NFPA HAZARD RATING (O=INSIGNIF ICANT ; 1 =SLIGHT;2 = MODERATE;3 = HIGH;4 = EXTREME) : 1 <--HEALTH    1 <--FLAMMABILITY   O <--REACT IVITY      <--SPECIAL

SECTION V - HEALTH HAZARD DATA

THRESHOLD LIMIT VALUE :

NOT ESTABLISHED FOR THE MIXURE.

SEE SECTION II.

SECTION IX - SPECIAL PROTECTION INFORMATIOPAGE : O5

REQUIRED VENTILATION

GENERAL MECHANICAL VENTILATION IS NORMALLY ADEQUATE.

RESPIRATORY PROTECTION

NIOSH/MSHA APPROVEP MASK FOR SPRAY MISTS.

PROTECTIVE GLOVES

NEOPRENE RUBBER GLOVES IF REPEATED OR PROLONGED

SKIN CONTACT IS LIKELY.

EYE PROTECTION

SAFETY GLASSES WITH SIDE SHIELDS IF THE METHOD OF

USE PRESENTS THE LIKELIHOOD OF EYE CONTACT.

OTHER PROTECTION

WEAR GENERAL DUTY WORK CLOTHING AND SHOES.

SECTION X - STORAGE AND HANDLING INFORMATION

STORAGE TEMPERATURE	INDOOR      HEATED      REFRIGERATED  OUTDOOR

I        J.INLJUUK	I  F1G.M  1  CU	iKtrKliJlCt^AICUi

I        V	'	'	'

12O:F <--MAX   35:F <--MIN     X

PRECAUTIONS TO BE TAKEN IN HANDLING 8. STORING STORE UNDER MODERATE TEMPERATURES.

OTHER PRECAUTIONS

KEEP OUT OF REACH OF CHILDREN.

READ ENTIRE LABEL BEFORE USING PRODUCT.

FOLLOW LABEL DIRECTIONS.

SECTION XI - REGULATORY INFORMATION

CHEMICAL NAME	C.A.S. NUMBER         UPPER % LIMIT

N/A

THOSE INGREDIENTS LISTED ABOVE ARE SUBJECT TO THE REPORTING REQUIREMENTS OF 313 OF TITLE III OF THE SUPERFUND AMENDMENTS AND REAUTHORIZATION ACT OF 1986 AND 4O CFR PART 372.

IF UE (USE EXEMPTION) APPEARS UNDER UPPER % LIMIT, END USERS ARE EXEMPT FROM NOTIFICATION BECAUSE THE PRODUCT IS USED AND LABELED FOR ROUTINE JANITORIAL WORK, OR THE PRODUCT IS USED AND LABELED FOR FACILITY GROUNDS MAINTENANCE (SUCH AS FERTILIZERS AND HERBICIDES), OR THE PRODUCT IS USED AND LABELED FOR MAINTAINING MOTOR VEHICLES.

CALIFORNIA PROPOSITION 65

M/M CHILL CLEAN

(CONTINUED)	SECTION XI - REGULATORY INFORMATION	PAGE : 06

WARNING: THIS PRODUCT CONTAINS THE FOLLOWING CHEMICAL(S) KNOWN TO THE STATE OF CALIFORNIA TO CAUSE (1) CANCER OR (2) BIRTH DEFECTS OR OTHER REPRODUCTIVE HARM: ARSENIC (1), CADMIUM (1), LEAD (2), NICKEL (1), ETHANOL (2), NITRILTRI ACETIC ACID (1) AND GLYCINE, N,N BIS(CARBOXYMETHYL)- (l) AS TRACE CONTAMINANTS.

SECTION XII - TRANSPORTATION * (FOR FUTURE USE)

APPLICABLE REGULATIONS

<--49 CFR     <--IMCO      <--TARIFF 6 D     <--IATA   <--MILITARY AIR (APR 71-4)

SHIPPING NAME

HAZARD CLASS	| ID NUMBER | REPORT QTY

LABELS	| LIMITED QTY

UNIT CONTAINER

DOT  SPS CONTAINER	] NET EXPLOSIVE WT.

AEROSOL PROPELLANT(S)

SECTION XIII - REFERENCES

- THRESHOLD LIMIT VALUES FOR CHEMICAL SUBSTANCES AND PHYSICAL AGENTS AND
BIOLOGICAL EXPOSURE INDICES, ACGIH,  1994-1995.

- OSHA PEL.

- SAX'S DANGEROUS PROPERTIES OF INDUSTRIAL MATERIALS, EIGHTH EDITION,
RICHARD J. LEWIS, SR.

- VENDOR'S MSDS.

*SHORT TERM EXPOSURE LIMIT (TWA) LISTED AS FINAL RULE LIMITS PUBLISHED IN THE FEDERAL REGISTER/VOL.54 NO . 1 2 ,  1-19-89

riNUED)	SECTION! XIII - REFERENCES	PAGE : O7

THE INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORMULATION, HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF.

CERTIFIED LABS, DIV. OF NCH CORP.         ASSUMES NO RESPONSIBILITY FOR PERSONAL INJURY OR PROPERTY DAMAGE CAUSED BY THE USE , STORAGE OR DISPOSAL OF THE PRODUCT IN A MANNER NOT RECOMMENDED ON THE PRODUCT LABEL. USERS ASSUME ALL RISKS ASSOCIATED WITH SUCH UNRECOMMENDED USE, STORAGE, OR DISPOSAL OF THE PRODUCT.