i

MATERIAL SAFETY DATA SHEET

HMIS Rating:    Health  1 Flammability   0 Reactivity   0

| IDENTITY (As Used on Late! and List) Manufacturer's Name | Distributed By: Handi-Clean Products, Inc | Emergency Telephone Number (910) 292-3083 After 5:00 D.m. (910) 454-3662 |  |
| --- | --- | --- | --- |
| Address: | 301 Swing Road Greensboro, NC 27409 | Telephone Number for Information (910) 292-3083 | Date Prepared 1-10-93 |

SAF-T-STEP Slip Resistant Floor Cleaner and Treatment

Section I

FSP1

Section II • Hazardous Ingredients/Identity Information

Hydroxyacetic acid

Citric Acid

2-propanol-1-butoxy

Note:   Blank Spaces are not permitted.    If any item is not applicable,   or no

information  is available, the space must be marked to indicate that.

CAS NO.     OSHA PEL     ACGIH TLV     % (optional)

1 - 4%

79-14-1

2 ppm

77-92-9

1 - 3%

N.D.

5131-66-8       100 ppm

1 - 2%

Section III . Ptwiical Ctemical Characterltt)c$

| OtMlmm fEMraft BQnlfly POml | 212°F | Specific Gravity (H,0 - 1) | 1.05 | Appearancel Blue Grn Liquid |  |
| --- | --- | --- | --- | --- | --- |
| Vapor Pressure (mm Ha) | N/A | Evaporation Rate | N/A | Odor | Mild |
| Vapor Density (AIR * 1) | N/A | (Butyl Acetate « 1) |  | Solubility In Water | Complete |

Section IV • Rre and Explosion Hazard Data

Rash Point (Method Used)      None @ ASTM D92

| Flammable Limits

LEL       N/A |UEL     N/A

Extinguishing Media:   Foam, CO* Dry Chemical. Water Fog

Special Fire Fighting Procedures: Normal fire fighting procedure.

Unusual Fire and Explosion Hazards: None known. Heat of fire may melt containers.

Section V • Reactivity Data

Stability:! Stable I     X   I Unstable I

I  Conditions to Avoid - N/A

Incompatibility (Materials to Avoid)   Strong alkalis, bleach, cyanides, sulfides, or chlorine of any kind. Hazardous Decomposition or Byproducts   None known

| Hazardous Polymerization | May Occur |  | Will Not Occur | X | Conditions to Avoid N/A |
| --- | --- | --- | --- | --- | --- |

Section VI • Health Hazard Data

Ingestion?   X

Route(s) of Entry:

Skin?    X

Inhalation?

Health Hazards (Acute and Chronic)   Direct contact with skin and eves can cause irritation.   Prolonged or repeated contact with skin and eyes may cause redness or burns.   INHALATION: of mist or vapor may cause some individuals to cough or cause irritation of

nose, throat and lungs. Harmful if swallowed.

Emergency and First Aid Procedures   INGESTION:   Give large amounts of water or milk.   Call physician.   EYES/SKIN: Immediately (lush with water for at least 15 minutes. If pain, irritation or redness persists, see physician.   INHALATION: Remove to refresh air. See

physician If irritation persists.	Section VII • Precautions for Safe Handling and Use

Steps Steps to Be Taken in Case Material is Released or Spilled: Absorb with diatomaceous earth or other inert material. Sweep

or scrape up.   Rinse area with water down drain.

Waste Disposal Method:   Dispose in accordance with local, state, & federal regulations

Precautions to Be Taken In Handling and Storing:   Store in a cool dry area away from heat or flame (containers may melt). Keep from freezing as containers may rupture.      Other Precautions     KEEP OUT OF REACH OF CHILDREN

|  |  | Section VIII | - Control | Measures |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Respirator' Protection (Specify Type) |  |  | None Reguired for normal use |  | Ventilation | Normal |
| Protective uloves None |  | Reguired for | normal use |  | Eye Protection | Safety Glasses |
| Other Protective Clothing | or | Equipment | None | Reguired |  |  |
| Work/Hygienic Practices |  |  | Wash | hands thoroughly after using product |  |  |