(

MATERIAL

SAFETY

DATA

SHEET

The information contained herein is based on data available at the time of preparation of this data sheet and which The Glidden Company believes to be reliable. However, no warranty is expressed or implied regarding the accuracy of this data. The Glidden Company shall not be responsible for the use of this information, or of any product, method or apparatus mentioned and you must make your own determination of its suitability and completeness for your own use, for the protection of the environment, and the health and safety of your employees and users of this material.

Complies with OSHA hazard communication standard 29CFR1910.1200.

| 4501 Series, 5539, HMIS Rating | 5540 |  |
| --- | --- | --- |
| Health Flammability Reactivity | *2 2 0 |  |
| 4570 HMIS Rating |  |  |
| Health Flammability Reactivity | *3 2 0 |  |

*See Health Hazard Data for chronic health information.

Prepared November. 1988

THE GLIDDEN COMPANY

925 Euclid Avenue Cleveland. Ohio 44115

Emergency Telephone No. (216)826-5566

GLID-GUARD® Alkyd

Industrial Enamel No. 4501 Series;

GLID-GUARD® Silicone

Alkyd Enamel Nos. 5539, 5540

FIRE & EXPLOSION HAZARD DATA

DOT Proper Shipping Name: Paint, UN 1263 Hazard Class: Combustible Liquid Extinguishing Media: Dry chemical or foam

Unusual Fire and Explosion Hazards: Closed containers may explode when exposed to extreme heat or fire. Vapors can form explosive mixtures in air at elevated temperatures. May decompose under fire conditions emitting irritant and/or toxic gases.

Special Fire Fighting Procedures: Water may be used to cool and protect exposed containers.

HEALTH HAZARD DATA

Primary Route(s) of Exposure: Inhalation

Skin Contact

Effects of Overexposure:

Inhalation: Irritation of respiratory tract. Prolonged inhalation may lead to fatigue, drowsiness, dizziness and/or lightheadedness, headache, uncoordination, nausea, vomiting, central nervous system depression, anesthetic effect or narcosis.

Skin Contact: Irritation of skin. Prolonged or repeated contact can cause dermatitis, defatting.

Eye Contact: Irritation of eyes. Prolonged or repeated contact can cause blurred vision, redness of eyes, tearing of eyes, severe eye irritation.

Ingestion: Amounts ingested incidental to consumer and industrial handling are not likely to cause injury; however, ingestion of larger amounts may cause lung inflammation and damage due to aspiration of material into lungs.

Notice: Reports have associated repeated and prolonged occupational overex-posure to solvents with permanent brain and nervous system damage. Intentional misuse by deliberately concentrating and inhaling the contents may be harmful or fatal.

Emergency and First Aid Procedures:

Inhalation: Remove to fresh air. Restore and support continued breathing. Get

emergency medical attention. Have trained person give oxygen if necessary.

Get medical help for any breathing difficulty. Skin Contact: Wash off quickly with plenty of water, then soap and water;

remove contaminated clothing. Wash contaminated clothing before reuse. Eye Contact: Flush immediately with large amounts of water, especially under

lids, for at least 1 5 minutes. Obtain emergency medical treatment. Ingestion: If swallowed, obtain medical treatment immediately.

Medical Conditions Aggravated By Exposure: This product is not expected to aggravate existing medical conditions; however, ingredients contained in this product have been reported to aggravate preexisting eye, skin, respiratory disorders, lung disorders, asthma-like conditions.

Supplemental Health Information: This product contains crystalline silica, which is considered a hazard by inhalation. The International Agency for Research on Cancer (IARC) has determined that there is sufficient evidence for the car-cinogenicity of crystalline silica to experimental animals, and limited evidence for the carcinogenicity of crystalline silica to humans. Crystalline silica is also a known cause of silicosis, a noncancerous lung disease. Exposure to materials in this product have been associated with possible blood abnormalities, liver damage, kidney damage. Excessive inhalation of solvent vapors under uncontrolled conditions may lead to unconsciousness, respiratory failure, asphyxiation, and even death.

Attention: Products in this series contain ethylene glyycol when tinted with Dramatone. Ethylene glycol can cause severe kidney damage when ingested and has been shown to cause birth defects in laboratory animals.

REACTIVITY DATA

Stability: Stable

Incompatibility: Oxidizers Acids Bases Amines

Conditions to Avoid: Elevated temperatures

Contact with oxidizing agent

Hazardous Decomposition Products: Carbon Monoxide

Carbon Dioxide

Hazardous Polymerization: Will not occur

SPILL OR LEAK PROCEDURES

Steps to be taken in case material is released or spilled: Comply with all applicable health and environmental regulations. Eliminate all sources of ignition. Ventilate area. Spills may be collected with non-combustible absorbent materials.

Waste Disposal: Dispose in accordance with all applicable regulations. Avoid discharge to natural waters.

SPECIAL PROTECTION INFORMATION

Respiratory Protection: Control environmental concentrations below applicable standards. Where respiratory protection is required, use only NIOSH/MSHA approved respirators in accordance with OSHA Standard 29 CFR 1910.134.

Ventilation: Provide dilution ventilation or local exhaust to prevent build-up of vapors.

Personal Protective Equipment: Eye Wash

Safety Shower

Safety Glasses or Goggles

Impervious Gloves

SPECIAL PRECAUTIONS

Handling and Storage: Store below 100°F. Keep away from heat, sparks, and open flame.

Other Precautions: Use only with adequate ventilation. Do not take internally. Keep out of reach of children. Avoid contact with skin and eyes, and breathing of vapors. Wash hands thoroughly after handling, especially before eating or smoking. Keep containers tightly closed and upright when not in use. Avoid conditions which result in formation of inhalable particles such as spraying or abrading (sanding) painted surfaces. If such conditions cannot be avoided, use appropriate respiratory protection as directed under Special Protection Information. Empty containers may contain hazardous residues. Ground equipment when transferring to prevent accumulation of static charge.

4501 SERIES, 5539, 5540

| PRODUCT CODE NO. | 4501 | 4502 | 4503 | 4510 | 4511 | 4520 | 4525 | 4537 | 4540 | 4541 | 4546 | 4550 | 4551 | 4554 | 4560 | 4562 | 4564 | 4570 | 4573 | 4575 | 4580 | 4587 |  | 5539 | 5540 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| "Ingredients, Wt.% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| • Distillates (Petroleum), Hydro-Treated |  |  |  |  |  |  | 6-11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| • Solvent Naphtha (Petroleum), Light, Arom. | 2-7 | 2-7 | 27 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| • Metal Oxide Silicate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 18-23 |  |  |  |  |  |  |  |
| • Calciom Carbonate |  |  | 10-15 |  |  |  |  |  |  |  | 4-9 |  |  |  |  |  |  | 20-25 |  |  |  |  |  |  |  |
| • Carbon Black |  |  |  |  |  |  |  |  |  |  | 1-5 |  | 1-5 |  |  |  |  |  |  |  |  |  |  |  |  |
| Dinitraniline Orange |  |  |  |  | 611 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| • Crystalline Silica, Quartz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | <1 |  |  |  |  |  |  |  |
| • Ethylbenzene (1) |  |  |  |  |  |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| • Distillates (Petroleum), Hydro-Treated, light | 8-13 | 8-13 | 10-15 | 6-11 | 12-17 | 1-5 | 20-25 | 8-13 | 12-17 | 10-15 | 1621 | 6-11 | 6-11 | 6-11 | 10-15 |  | 6-11 |  | 1-5 | 8-13 | 10-15 | 8-13 |  | 32-37 | 38-43 |
| • Solvent Naphtha (Petroleum), Medium, Aliph. | 20-25 | 2227 | 24-29 | 34-39 | 30-35 | 46-51 |  | 34-39 | 28-33 | 30-35 | 22-27 | 28-33 | 40-45 | 40-45 | 28-33 | 46-51 | 36-41 | 28-33 | 46-51 | 30-35 | 34-39 | 28-33 |  |  |  |
| • Organophilic Clay |  |  |  |  |  |  | 15 |  |  |  |  |  | 1-5 |  |  |  |  |  |  |  |  |  |  |  |  |
| • Maleic Modified Rosin Ester |  |  | 1-5 | 1-5 | 1-5 | 1-5 |  | 1-5 | 1-5 | 1-5 | 1-5 |  |  | 1-5 | 1-5 |  | 1-5 |  |  |  | 1-5 | 1-5 |  |  |  |
| • Organophilic Clay |  |  |  |  | 1-5 | 1-5 |  | 1-5 |  | 1-5 | 1-5 |  |  | 1-5 | 1-5 | 1-5 |  |  | 1-5 |  | 1-5 |  |  |  |  |
| • Titanium Dioxide | 18-23 | 12-17 |  | 2-7 |  |  | 26-31 |  | 4-9 | 7-12 | 1-5 | 22-27 |  | 1-5 | 611 | 6-11 | 2-7 |  | 4-9 | 18-23 | 15 | 12-17 |  | 26-31 | 6-11 |
| Alkyd Resin | 18-23 | 20-25 | 20-25 | 24-29 | 20-25 | 34-39 |  | 26-31 | 14-19 | 18-23 | 22-27 | 18-23 | 28-33 | 26-31 | 16-21 | 34-39 | 26-31 | 16-21 | 36-41 | 18-23 | 24-29 | 22-27 |  |  |  |
| Alkyd Resin | 12-17 | 12-17 | 1419 | 10-15 |  |  | 3237 | 10-15 | 18-23 |  | 10-15 | 10-15 | 10-15 | 8-13 | 14-19 |  | 10-15 |  |  | 10-15 | 12-17 | 12-17 |  |  |  |
| Alkyd Resin |  |  |  |  | 10-15 |  |  |  |  | 10-15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hansa Yellow |  |  |  | 2-7 |  |  |  |  | 4-9 |  |  |  |  | 1-5 |  |  |  |  |  |  |  |  |  |  |  |
| Ferri-Ferro Cyanide Complex |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2-7 |  |  |  |  |  |  |  |  |
| • Metal Oxide |  |  |  |  |  |  |  | 1-5 |  |  | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| m-Nitro-p-Toluidinelb naphthol |  |  |  |  |  | 6-11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| • Yellow Iron Oxide |  |  |  |  |  |  |  | 4-9 | 1-5 | 1-5 | 1-5 |  |  |  |  |  |  |  | 1-5 |  |  |  |  |  |  |
| • Basic Zinc Molybdate (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4-9 |  |  |  |  |  |  |  |
| Silicons Alkyd Resin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 34-39 | 44-49 |
| • Magnesium Silicate Hydrate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1-5 |
| Physical Data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| % Volatile by Vol. | 54.4 | 55.2 | 55.9 | 57.8 | 58.0 | 60.4 | 49.9 | 59.1 | 57.2 | 58.6 | 57.6 | 56.1 | 60.2 | 60.9 | 57.3 | 63.0 | 59.9 | 55.3 | 62.0 | 57.7 | 59.7 | 56.6 |  | 51.4 | 53.1 |
| Wt. per Gallon, Ibs. | 9.1 | 8.6 | 8.3 | 8.0 | 7.9 | 7.7 | 9.9 | 8.1 | 8.2 | 8.3 | 8.3 | 9.3 | 7.6 | 7.7 | 8.2 | 8.0 | 7.8 | 11.6 | 7.9 | 8.9 | 7.7 | 8.5 |  | 9.9 | 8.5 |
| Boiling Range, °F. | 308-374 | 308-374 | 308-374 | 308-374 | 308-374 | 308-374 | 273-386 | 308-374 | 308-374 | 308-374 | 308-374 | 308-374 | 308-374 | 308-374 | 308-374 | 316-360 | 308-374 | 316-360 | 308-374 | 308-374 | 308-374 | 308-374 |  | 308-374 | 308-374 |
| Flash Point, °F. | 105 | 105 | 105 | 102 | 102 | 105 | 105 | 100 | 106 | 106 | 105 | 105 | 102 | 109 | 106 | 106 | 100 | 100 | 104 | 105 | 105 | 105 |  | 105 | 105 |
| Lower Explosive Limit | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 1.0 | 0.7 | 1.0 | 0.7 | 0.7 | 0.7 | 0.7 |  | 0.7 | 0.7 |

• Hazardous Ingredients as defined by OSHA, ACGIH or The Glidden Company.

"""If Dramatone is used to tint, ethylene glycol will be present. Refer to MSDS on Dramatone No. 1750 Series for details.

- SARA SECTION 313 CHEMICAL

- SARA SECTION 302 EHS

| CHEMICAL NAME/COMMON NAME | CAS. NO. | ACGI 8-HOUR TWA | H-TLV STEL | OSHA-PEL 8-HOUR TWA | LEL | V.P. |
| --- | --- | --- | --- | --- | --- | --- |
| Distillates {Petroleum), Hydro-Treated; Aliphatic Hydrocarbon Blend | 6474247-8 | 100ppm§ | Not Established | 500 ppm§ | 1.0 | 1 @ 68"F. |
| Solvent Naphtha (Petroleum), Light, Arom.l Aromatic Hydrocarbon Blend | 64742-95-6 | 100ppm§ | Not Established | 500 ppm§ | 1.0 | 10.0@100DF. |
| Metal Oxide Silicate/Brown Iron Oxide | 1309-37-1 | 0.1 mg/m3:!: | Not Established | dOmg/m'MSSiOj+at | N.A. | N.A. |
| Calcium Carbonate/Limestone | 1317-65-3 | 10mglm3 | Not Established | 15 mg/m3 | N.A. | N.A. |
| Carbon Black/Carbon Black | 1333-86-4 | 3.5 mg/m3 | Not Established | 3.5 mg/m3 | N.A. | N.A. |
| Crystalline Silica, Quartz: Silica, Crystalline, Quartz | 14808-60-7 | 0.1 mg/m4 | Not Established | (10mg/m3)/(%SiOz + 2H: | N.A. | N.A. |
| Dinitraniline OrangelDinitraniline Orange | 3468-63-1 | 10 mg/m3 | Not Established | 15 mg/m3 | N.A. | N.A. |
| EthylbenzenelEthylbenzene | 100-41-4 | 100ppm | 1 25 ppm | 100 ppm | 1.0 | 7.1@68°F. |
| Distillates (Petroleum), Hydro-Treated, Light Mineral Spirits | 64742-47-8 | 100ppm§ | Not Established | 500 ppm§ | 0.7 | 2.0@68"F. |
| Solvent Naphtha (Petroleum), Medium, Aliph.JMineral Spirits | 64742-88-7 | 100ppm§ | Not Established | 500 ppm§ | 1.0 | 5 @ 77°F. |
| Organophilic ClaylOrganophilic Clay | 71011-27-3 | 10 mg/m3 | Not Established | 15 mg/m3 | N.A. | N.A. |
| Maleic Modified Rosin Ester/Pentaerythritol Ester | 68333-69-7 | Not Established | Not Established | Not Established | N.A. | N.A. |
| Organophilic ClaylOrganophilic Clay | 68911-87-5 | 10 mg/m3 | Not Established | 15 mg/m3 | N.A. | N.A. |
| Titanium Dioxide/Titanium Dioxide, Rutile | 13463-67-7 | 10mg/m3 | Not Established | 15 mg/m' | N.A. | N.A. |
| Alkyd Resin/Alkyd Resin | 68459-17-6 | Not Established | Not Established | Not Established | N.A. | N.A. |
| Alkyd ResinfAlkyd Resin | 67700-92-9 | Not Established | Not Established | Not Established | N.A. | N.A. |
| Alkyd ResinlAlkyd Resin | 68604-95-5 | Not Established | Not Established | Not Established | N.A. | N.A. |
| Hansa YellowJHansa Yellow | 13515-40-7 | 10 mg/m3 | Not Established | 1 5 mg/m3 | N.A. | N.A. |
| Ferri-Ferro Cyanide Complexion Blue | Supplier Confidential | 10 mg/m3 | Not Established | 15 mg/m3 | N.A. | N.A. |
| Metal OxidelRed Iron Oxide | 1309-37-1 | 10 mg/m3 | Not Established | 15 mg/m3 | N.A. | N.A. |
| m-Nitro-p-Toluidinefb-naphthollToluidineRed | 2425-85-6 | 10 mg/m3 | Not Established | 15 mg/m3 | N.A. | N.A. |
| Yellow Iron Oxide/Yellow Iron Oxide | 51274-00-1 | 10 mg/m3 | Not Established | 15 mg/m3 | N.A. | N.A. |
| Basic Zinc Molybdate/Basic Zinc Molybdate | 13767-32-3 | 10 mg/m3 | Not Established | 15 mg/m3 | N.A. | N.A. |
| Silicone Alkyd Resin.'Silicone Alkyd Resin | Supplier Confidential | Not Established | Not Established | Not Established | N.A. | N.A. |
| Magnesium Silicate, HydratelTalc | 14807-96-6 | 2 mg/m3t | Not Established | 20 mppcf | N.A. | N.A. |

LEL-The lower explosive limit is the lowest concentration (% of volatiles in air) that will produce a flash of fire when an ignition source is present.

V.P.—Vapor pressure in millimeters of mercury at the indicated temperature. mppcf-Millions of particles per cubic foot. N A-Not applicable. mg/m3—Milligrams per cubic meter. ppm-Parts per million. tRespirable Dust §As Stoddard solvent IARC listing due to Crystalline Silica.

Carcinogenicity Listed By: NTP? No     IARC Monograph? Yes     OSHA Regulated? No

C5