FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Material Safety Data Sheet (MSDS)

MSDS#:    117.00

Revision Date:   March 12, 2001

Section 1 — Chemical Product and Company Identification

Bismuth Nitrate

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition. Information on Ingredients

Bismuth Nitrate

CAS#:  10035-06-0

Section 3 — Hazards Identification

Lustrous, colorless crystals. Stinging, acid-like odor.

Slightly toxic by ingestion. Irritating to skin, eyes, and mucous membranes. Avoid all body tissue

contact.

Strong oxidizer, fire risk near organic materials.

flinn at-a-glance

Health-1 Flammability-0 React! vity-3 Exposure-1 Storage-1

0 is low hazard, 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give large quantities of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Non-flammable, non-combustible solid.	NFPA code

Oxidizing agent, fire risk near organic materials. Contact with combustible material may cause fire.	None established

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE and SCBA with full facepiece operated in positive pressure made.

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #3. Store with amides, nitrates and nitrites. Store in a cool dry place. Deliquescent, store in Flinn Chem-Saf Bag.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. Use exhaust ventilation to keep airborne concentrations low.

i 2002 Flinn Scientific, Inc. All Rights Reserved.

page 1 of 2

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Material Safety Data Sheet (MSDS)

Bismuth Nitrate

MSDS#:     117.00

Revision Date:   March 12, 2001

Section 9 — Physical and Chemical Properties

Colorless, hygroscopic crystals

Solubility: Soluble in dilute nitric acid, alcohol and acetone.

Formula: Bi(NO3)3 5H2O

Formula Weight: 485.10

Specific Gravity: 2.83 Decomposes at 30 C

Section 10 — Stability and Reactivity

Avoid contact with strong reducing agents, acids and organic materials. Shelf life: Fair to poor.

Section 11 —Toxicoloqical Information

Acute effects: Irritant. Chronic effects: N.A. Target organs: N.A.

ORL-RAT LD50: 4042 mg/kg IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

N.A. = Not available, not all health aspects of this substance have been folly investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option.

Section 14 — Transport Information

Shipping Name:Not regulated Hazard Class: N/A UN Number: N/A

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (215-136-8), RCRA code D001.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Flinn— "Your Safer Source for Chemicals "

flinn@flinnsci.com  www.flinnsci.com

P.O. Box 219   BatavialL 60510

(800) 452-1261 Fax (866) 452-1436

© 2002 Flinn Scientific, Inc. All Rights Reserved.

page 2 of 2