Uulcan

Materials Company

MATERIAL SAFETY DATA SHEET

Effective Date:    6-1-89 Replaces:   7-15-88

| 1— IDENTIFICATION |  |  |
| --- | --- | --- |
| CHEMICAL NAME Granite | CHEMICAL FORMULA Not Applicable | MOLECULAR WEIGHT Not Applicable |
| TRADE NAME Crushed Stone |  |  |
| SYNONYMS Aggregate, Manufactured Sand |  | DOT IDENTIFICATION NO. None |

| II— PRODUCT AND COMPONENT DATA |  |  |  |
| --- | --- | --- | --- |
| COMPONENT(S) CHEMICAL NAME | CAS REGISTRY NO. | % (Approx) | ACGIH TLV-TWA |
| Granite* | None | 100 | See |
|  |  |  | Section |
|  |  |  | VI |
| *Quartz (crystalline silica) content typically greater | 14808-60-7 | >1 |  |
| than 1% |  |  |  |

| III— PHYSICAL DATA |  |
| --- | --- |
| APPEARANCE AND ODOR Angular particles, light salt-and-pepper colored, ranging in size from pebbles to boulders. No odor. | SPECIFIC GRAVITY 2.6-2.81 |
| BOILING POINT Not Applicable | VAPOR DENSITY IN AIR (Air = 1) Not Applicable |
| VAPOR PRESSURE Not Applicable | % VOLATILE, BY VOLUME 0% |
| EVAPORATION RATE 0 | SOLUBILITY IN WATER Negligible |

IV—REACTIVITY DATA

STABILITY

Stable

CONDITIONS TO AVOID

Avoid contact with incompatible materials.

INCOMPATIBILITY (Materials to avoid)

Contact with powerful oxidizing agents such as fluorine, chlorine trifluoride, manganese trifluoride, and oxygen difluoride may cause fire and/or explosions. Silica dissolves in hydrofluoric acid producing a corrosive gas—silicon tetrafluoride.

HAZARDOUS DECOMPOSITION PRODUCTS

None Known

HAZARDOUS POLYMERIZATION

Not known to polymerize.

V—FIRE AND EXPLOSION HAZARD DATA

FLASHPOINT (Method used)

Not Flammable

FLAMMABLE LIMITS IN AIR

Not Flammable

EXTINGUISHING AGENTS

None Required

UNUSUAL FIRE AND EXPLOSION HAZARDS

None Known

VI—TOXICITY AND FIRST AID

EXPOSURE LIMITS (When exposure to this product and other chemicals is concurrent, the exposure limit must be defined in the workplace.)

Unless specified otherwise, limits are expressed as milligrams of substance per cubic meter of air (mg/M3), 8-hour time-weighted

averages (TWA). Limits for cristobalite and tridymite (other forms of crystalline silica) are equal to one-half of the limits for quartz.

Respirable crystalline silica (quartz): ACG1H TLV, OSHA PEL, and MSHA-proposed PEL = 0.1.

Respirable dust: MSHA =  10 -4- (% respirable quartz + 2).	Total dust: MSHA = 30 4- (% quartz+3).

Other particulates: ACGIH TLV = 10 (total particulates, not otherwise classified); OSHA PEL = 5 (respirable dust, not otherwise

regulated), 15 (total dust, not otherwise regulated); MSHA = 10 (total "nuisance" dust); MSHA-proposed = 5 ("respirable mine dust").

Effects described in this section are believed not to occur if exposures are maintained at or below appropriate TLVs.

Because of the wide variation in individual susceptibility, these exposure limits may not be applicable to all persons and those with medical conditions listed below.

MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE

Inhaling respirable dust may aggravate existing respiratory system disease (s) and/or dysfunctions. Exposure to dust may aggravate existing skin and/or eye conditions.

PRIMARY ROUTE(S) OF EXPOSURE:        Kl Inhalation        D  Skin        D  Ingestion

ACUTE TOXICITY

Exposure to dust may irritate respiratory system, eyes, and skin.

FIRST AID

Dust in eyes: Flush eyes with running water for 15 minutes. Contact a physician if irritation persists or later develops. Dust on previously irritated skin: Wash with soap and water. Contact a physician if irritalion is aggravated.

Dust inhalation: Remove to fresh air. Dust in throat and nasal passages should clear spontaneously. Contact a physician if irritation persists or later develops.

MSDS 3239-002   REV. 6/89

CHRONIC TOXICITY

Chronic exposure to respirable granite dust in excess of appropriate exposure limits has caused silicosis, a progressive pneumoconiosis (lung disease). Chronic tobacco smoking may further increase the risk of developing chronic lung problems.

Symptoms of Silicosis: Not all individuals with silicosis will exhibit symptoms (signs) of the disease. However, silicosis is progressive, and symptoms can appear at any time, even years after exposures have ceased. Symptoms of silicosis may include (but are not limited to): Shortness of breath; difficulty breathing with or without exertion; coughing; diminished work capacity; diminished chest expansion; reduction of lung volume; right heart enlargement and/or failure. Persons with silicosis have an increased risk of pulmonary tuberculosis infection.

Granite is not listed on the NTP, IARC, or OSHA lists of carcinogens. Crystalline silica, a component of this product, is listed by IARC but not by NTP or OSHA. IARC has determined that there is sufficient evidence for carcinogenicity to experimental animals exposed to crystalline silica and limited evidence for carcinogenicity to humans. "Limited evidence" means that a causal relationship is possible; however, other explanations such as chance, bias or confounding factors cannot adequately be excluded. NTP has proposed (1989) to list crystalline silica based on the IARC determination.

VII—PERSONAL PROTECTION AND CONTROLS

RESPIRATORY PROTECTION

For respirable quartz levels that exceed or are likely to exceed an 8hr-TWA of 0.1 mg/M3 an approved dust respirator must be worn. For respirable quartz levels that exceed or are likely to exceed an 8hr-TWA of 0.5 mg/M3 an approved HEPA filter respirator must be worn. If respirable quartz levels exceed or are likely to exceed an 8hr-TWA of 5 mg/M3 a positive pressure, full face respirator or equivalent is required. Follow applicable respirator use, fitting and training standards.

VENTILATION

Local exhaust or general ventilation adequate to maintain exposures below appropriate exposure limits.

SKIN PROTECTION

See "Hygiene" section below.

EYE PROTECTION

Safety glasses with side shields should be worn as minimum protection. Dust goggles should be worn when excessively (visible) dusty conditions are present or are anticipated.

HYGIENE

Wash dust-exposed skin with soap and water. Wash work clothes after each use.

OTHER CONTROL MEASURES

Respirable dust levels should be monitored regularly. Dust levels in excess of appropriate exposure limits should be reduced by all feasible engineering controls, including (but not limited to) wet suppression, ventilation, process enclosure, and enclosed employee work stations.

MSDS 3239-002   REV. 6/89

VIII—STORAGE AND HANDLING PRECAUTIONS

Respirable dust may be generated during processing, handling, and storage. The controls identified in Section Vn of the MSDS should be applied as appropriate.

IX—SPILL, LEAK AND DISPOSAL PRACTICES

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED

— The controls identified in Section VII of the MSDS should be applied as appropriate, • Spilled materials, where dust can be generated, may overexpose cleanup personnel to respirable dust. Wetting of spilled material and/or use of respiratory protective equipment may be necessary. Avoid dry sweeping. This product is not subject to the reporting requirement of Title III of SARA, 1986, and 40 CFR 372.

WASTE DISPOSAL METHOD

Dispose of waste materials only in accordance with applicable federal, state, and local laws and regulations.

| X— TRANSPORTATION |  |
| --- | --- |
| DOT HAZARD CLASSIFICATION None |  |
| PLACARD REQUIRED None |  |
| LABEL REQUIRED Label as required by the applicable state and local | OSHA Hazard Communication standard [29 CFR 1910.1200 (f)], and regulations. |

For Further Information

DATE OF PREPARATION:     June  1,   1989

Contact:    VULCAN MATERIALS COMPANY

Mideast Division MSDS Coordinator 4401 N. Patterson Avenue Winston-Salem, NC 27105 919/767-4600

8 AM—5 PM Eastern Time Monday through Friday For Emergency Information Call:    205/877-3000 (24 hours)

NOTICE: Vulcan Materials Company believes that the information contained on this Material Safety Data Sheet is accurate. The suggested procedure;- are based on experience as of the date of publication. They are not necessarily all-inclusive nor fully adequate in every circumstance. Also, the suggestions should not be confused with nor followed in violation of applicable laws, regulation, rules or insurance requirements.

NO WARRANTY, EXPRESS OR IMPLIED, OF MERCHANTABILITY, FITNESS OR OTHERWISE IS MADE.

MSDS 3239-002    REV. 6/89

MSDS 3239-002    REV. 6/89