MATERIAL SAFETY DATA SHEET

Revised	(Approved by U.S. Department of Labor "Essentially Similar" to Form LSB-OOS-4)

October 1989

Section I

MANUFACTURER'S NAME

NPVLA6-70

700-02

Hysan Corporation

STREET ADDRESS

919 West 38th Street

CITY, STATE, AND ZIP CODE

EMERGENCY TELEPHONE NO.

Chicago, Illinois 60609

312/376-8900

CHEMICAL NAME AND SYNONYMS

N.A.

TRADE NAME

VANDELEX .•'•

CHEMICAL FAMILY

Solvent Degreaser

FORMULA

N.A.

Section II – Hazardous Ingredients

PAINTS, PRESERVATIVES. & SOLVENTS

PIGMENTS

TLV (Unit!)

SOLVENTS

TLV (Unlti)

CATALYST

ADDITIVES

,CL6

OTHERS

HAZARDOUS MIXTURES OF OTHER LIQUIDS. SOLIDS. OR GASES

tlv (Unlit)

1,1,1-Trichloroethane

C-.A.ti.  No.       71-55-6

350 Dem

Propellant:     Carbon  Dioxide

C.A.S.  No.    124-38-9

5000pprn

Section III-*PHYSICAL bAfA

BOILING POINT CF.)

IBP 163

SPECIFIC GRAVITY (H,O = 1)

1.32

VAPOR PRESSURE (mm Ho.)

100mm   @  20uf!

PERCENT  VOLATILE BY VOLUME (%)

100

VAPOR DENSITY |AIR=1)

EVAPORATION RATE

6.0

SOLUBILITY IN WATER

Negligible

APPEARANCE AND ODOR

Clear, thin liquid

having a mild chloroform-lii

e odor.

EXTINGUISHING MEDIA

Section IV * Fire and Expolosion Hazard Data

FLAMMABLE  LIMITS

j I

Water Fog,   Dry Chemical  or COg

Lll

15

SPECIAL FIRE FIGHTING PROCEDURES

Keep  containers  cool.     Use  shielding to protect personnel against

bursting,   rupturing or venting  containers.

AriievateTTemAperatures   (over 130"F)   containers may burst,   vent or rupture.

Revised

October*

THRESHOLD LIMIT VALUE

350 ppm  Estimated

EFFECTS OF OVEREXPOSURE

Inhalation - Anesthetic effects in range of 500 - 1000 ppm.

Eves - Mav cause eve irritation.'

Skin - May cause skin irritation.

EMERGENCY AND FIRST AID PROCEDURES

Ingestion - Induce vomiting.  Call physician.  Inhalation - Remove

to fresh air.  Skin - Wash skin with soap and water.  Eyes - Flush with copious amounts

of water.  Call physician if irritation developes or persists.

| Section VI – Reactivity Data |  |  |  |  |
| --- | --- | --- | --- | --- |
| STABILITY | UNSTABLE | X | CONDITIONS TO AVOID Heat |  |
|  | STABLE |  |  |  |
| INCOMPATABILITY (Material, to avoid) None |  |  |  |  |
| HAZARDOUS DECOMPOSITION | PRODUCTS Hvdroeen chloride, oxides of carbon and minute traces of phosgene |  |  |  |
| and chlorine formed upon combustion. |  |  |  |  |
| HAZARDOUS POLYMERIZATION | MAY OCCUR |  |  | CONDITIONS TO AVOID |
|  | WILL NOT OCCUR |  | X | None |

[«BK|

Section VII

m-lffS&tft

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED

Spill or Leak Procedures

Remove sources of flames, spark or heat.

Provide adeauate  ventilation. •* Wipe  up  spilled material.

waste disposal method    .Do not punc-t,ure or incinerate  containers.    Give, to a-disposal  service

equipped  to  safely handle  and  dispose  of pressurized  containers.

| Section VII Special Protection Info |  |  |
| --- | --- | --- |
| RESPIRATORY PROTECTION rSr«i/y «rpey NOHe Required |  |  |
| VENTILATION | Hood or Fan | SPECIAL jj ^ |
|  | MECHANICAL ICtneral) Maintain Adequate Ventilation | OTHER M A it . A . |
| PROTECTIVE GLOVES | ' EYE PROT None Required | ECTION Safetv Glasses |
| OTHER PROTECTIVE EQUIPMENT None Reauired |  |  |

PRECAUTIONS TO BE TAKEN  IN  HANDLING  AND  STORING

CAUTION  - Keep out of reach of  children.

Contents  under pressure.     Vapors harmful.

Mav  cause   skin irritation.     Do  not store  where  temperatures may exceed  120 F.

other precautions   READ ENTIRE LAmL BEFORE USING.