Material Safety Data Sheet

TREmeo

POLYROOF LV 3 GL

Version  1.3	Print Date 04/28/2007

REVISION DATE: 09/01/2006

SECTION 1 - PRODUCT IDENTIFICATION

Trade name	:    POLYROOF LV 3 GL.

Product code	:    361590803

COMPANY	:    Tremco Incorporated

3735 Green Road Cleveland, OH   44122

Telephone	:    (216) 292-5000 8:30 - 5:00 EST

Emergency Phone:	:    (216) 765-6727 8:30- 5:00 EST

After Hours: Chemtrec 1-800-424-9300

SECTION 2 - HAZARDS IDENTIFICATION

Emergency Overview

Black. Paste. May cause nausea, headaches, and dizziness. May cause drowsiness, weakness, and fatigue. May cause slight irritation to the respiratory system. Leave area to breathe fresh air. Avoid further overexposure. If symptoms persist, get medical attention.

Acute Potential Health Effects/ Routes of Entry

Inhalation         :     May cause nausea, headaches, and dizziness. May cause drowsiness, weakness, and

fatigue. May cause slight irritation to the respiratory system.
Eyes	:     Direct contact may cause moderate irritation. Direct contact may cause temporary

redness and discomfort.

Ingestion	:     May cause gastrointestinal irritation, nausea, and vomiting.

Skin	:     May cause moderate irritation. May cause itching, reddening, inflammation. May cause a

rash. May cause sensitization.

Aggravated Medical Conditions

Pre-existing eye, skin, liver, kidney, and respiratory disorders may be aggravated by exposure.

Chronic Health Effects

Prolonged or repeated skin contact with asphalt may result in skin sensitivity, such as irritation, rashes, and dermatitis. Prolonged or repeated exposure to polycyclic aromatic hydrocarbons and other volatiles which are contained in trace amounts in asphalt have been shown to cause cancer or respiratory damage in animals. Prolonged or repeated exposure to xylene may cause defatting, drying, and irritation of the skin, dermatitis, central nervous system (CNS) effects, heart muscle sensitization and arrhythmia, hearing loss, and brain, liver, kidney damage. Xylene overexposure may affect fetal development. Carbon black is classified by IARC to be a known animal carcinogen and a possible human carcinogen (Group 2B). Carbon black is encapsulated by resin and not expected to have adverse effects unless made airborne. The International Agency for Research on Cancer (IARC) has evaluated ethylbenzene and classified it as a possible human carcinogen (Group 2B) based on sufficient evidence for carcinogenicity in experimental animals, but inadequate evidence for cancer in exposed humans. Prolonged or repeated skin contact with thermally cracked petroleum distillates in these products may result in irritation and dermatitis. Although a direct association between thermally cracked petroleum distillates and cancer or other lung disease has not been established in man, thermally cracked petroleum distillates may contain variable amounts of polynuclear aromatic hydrocarbons (PNA's) and other volatiles which have been shown to cause cancer and respiratory damage in animals. May produce a phototoxic reaction when contaminated skin is exposed to sunlight. Fillers are encapsulated and not expected to be released from product under normal conditions of use.

Target Organs:   Skin, Eye, Lung, Liver, Kidney, Nerve

Material Safety Data Sheet

TREinco

POLYROOFLV3 GL

Version  1.3

REVISION DATE: 09/01/2006

Print Date 04/28/2007

SECTION 3 - PRODUCT COMPOSITION

Chemical Name

Asphalt

Xylene

Styrene-butadiene copolymers

Amorphous silica

Ethylbenzene

Coal tar

Petroleum distillates

Carbon Black

Weight %

CAS-No.

64741-56-6	30.0-60.0

1330-20-7	15.0-40.0

NJTSRN* 51721300-5132P	15.0-40.0

7631-86-9	5.0-10.0

100-41-4	5.0-10.0

8007-45-2	5.0-10.0

64741-81-7	5.0-10.0

1333-86-4	-<1.0

SECTION 4 - FIRST AID MEASURES

Get immediate medical attention for any significant overexposure.

Inhalation Eye contact Skin contact Ingestion

Leave area to breathe fresh air. Avoid further overexposure. If symptoms persist, get medical attention.

Flush with water for at least 15 minutes while holding eye lids apart. Get medical attention immediately.

Clean area of contact thoroughly using soap and water. If irritation, rash or other disorders develop, get medical attention immediately.

Do not induce vomiting unless advised by a physician. Call nearest Poison Control Center or Physician immediately.

SECTION 5 - FIRE FIGHTING MEASURES

Flash point Method

Burning rate Lower explosion limit Upper explosion limit Autoignition temperature Extinguishing media

Hazardous combustion products

Protective equipment for firefighters

Fire and explosion conditions

Not available. Not available.

Non-flammable solid

Not available.

Not available.

Not available.

If water fog is ineffective, use carbon dioxide, dry chemical or foam.

Smoke, fumes.Carbon monoxide and carbon dioxide can form.Oxides of sulfur can form.

Use accepted fire fighting techniques. Wear full firefighting protective clothing, including self-contained breathing apparatus (SCBA).

Product may ignite if heated in excess of its flash point.Vapors may travel to sources of ignition and flashback.Vapor concentrations in enclosed areas may ignite explosively.Empty containers may contain ignitable vapors.

Material Safety Data Sheet

TREltlCO

POLYROOF LV 3 GL.

Version  1.3

REVISION DATE: 09/01/2006

Print Date 04/28/2007

SECTION 6 - ACCIDENTAL RELEASE MEASURES

Use appropriate protective equipment. Avoid contact with material. Remove sources of ignition immediately. Stop flow of material if safe to do so. Contain spill and keep out of water courses. Ventilate area.

SECTION 7 - HANDLING AND STORAGE

Prevent inhalation of vapor, ingestion, and contact with skin eyes and clothing. Keep container closed when not in use. Precautions also apply to emptied containers. Do not smoke, weld, generate sparks, or use flame near container. Do not use in confined or poorly ventilated areas. Personal protective equipment must be worn during maintenance or repair of contaminated mixer, reactor, or other equipment. Store under dry warehouse conditions away from heat and all ignition sources.

SECTION 8 - EXPOSURE CONTROLS / PERSONAL PROTECTION

Personal protection equipment

Respiratory protection

Wear appropriate, properly fitted NIOSH/MSHA approved organic vapor or supplied air respirator when airborne contaminant level(s) are expected to exceed exposure limits indicated on the MSDS. Follow manufacturer's directions for respirator use.

Hand protection Eye protection

Use suitable impervious nitrile or neoprene gloves and protective apparel to reduce exposure.

Wear appropriate eye protection.Wear chemical safety goggles and/or face shield to prevent eye contact. Do not wear contact lenses. Do not touch eyes with contaminated body parts or materials. Have eye washing facilities readily available.

Skin and body protection Protective measures

Engineering measures Exposure Limits

Prevent contact with shoes and clothing.

Use professional judgment in the selection, care, and use.Inspect and replace equipment at regular intervals.

Use only in well ventilated areas. Provide maximum ventilation in enclosed areas.Use local exhaust when the general ventilation is inadequate.

| Chemical Name | CAS Number | Regulation | Limit | Form |
| --- | --- | --- | --- | --- |
| Xylene | 1330-20-7 | ACGIH TWA: ACGIH STEL: OSHA PEL: | 1 00 ppm 150 ppm 435 mg/m3 |  |
| Amorphous silica | 7631-86-9 | ACGIH TWA: ACGIH TWA: OSHA PEL: OSHA PEL: OSHA TWA: | 3 mg/m3 10 mg/m3 15 mg/m3 5 mg/m3 0.8 mg/m3 | Respirable particles. Inhalable particles. Total dust. Respirable fraction. |
| Ethyl benzene | 100-41-4 | ACGIH TWA: ACGIH STEL: OSHA PEL: | 100 ppm 125 ppm 435 mg/m3 |  |

Material Safety Data Sheet

POLYROOF LV 3 GL

Version 1.3

REVISION DATE: 09/01/2006

memo

Print Date 04/28/2007

| Chemical Name | CAS Number | Regulation | Limit | Form |
| --- | --- | --- | --- | --- |
| Carbon Black | 1333-86-4 | ACGIH TWA: OSHA PEL: OSHA TWA: OSHA TWA: | 3.5 mg/m3 3.5 mg/m3 15 mg/m3 5 mg/m3 | Total dust. Respirable fraction. |

SECTION 9 - PHYSICAL AND CHEMICAL PROPERTIES

Form

Color

Odor

PH

Vapour pressure

Vapor density

Melting point/range

Freezing point

Boiling point/range

Water solubility

Specific Gravity

% Volatile Weight

Paste Black Aromatic Not available. Not available. Heavier than air Not available. Not available. 371 °C, 700 °F Negligible 0.98 30%

SECTION 10 - REACTIVITY / STABILITY

Substances to avoid

Stability

Hazardous polymerization

Oxidizing agents.

Material is stable under normal storage, handling, and use.

Will not occur under normal conditions.

SECTION 11 - TOXICOLOGICAL INFORMATION

Xylene, CAS-No.: 1330-20-7

Acute oral toxicity (LD-50 oral) Acute inhalation toxicity (LC-50)

Amorphous silica, CAS-No.: 7631-86-9 Acute oral toxicity (LD-50 oral)

Ethylbenzene, CAS-No.: 100-41-4 Acute oral toxicity (LD-50 oral) Acute dermal toxicity (LD-50 dermal)

3,523 - 8,600 mg/kg   ( Rat) 6,350 mg/l   ( Rat)

22,500 mg/kg   ( Rat)

3,500 mg/kg   ( Rat) 17,800 mg/kg   (Rabbit)

Material Safety Data Sheet

POLYROOF LV 3 GL

Version  1.3

REVISION DATE: 09/01/2006

TRemeo

Print Date 04/28/2007

SECTION 12 - ECOLOGICAL INFORMATION

No Data Available

SECTION 13 - DISPOSAL CONSIDERATIONS

Disposal Method      :    Subject to hazardous waste treatment, storage, and disposal requirements under RCRA. Recycle or incinerate waste at EPA approved facility or dispose of in compliance with federal, state and local regulations.

SECTION 14 - TRANSPORTATION / SHIPPING DATA

TDG / DOT Shipping Description:

NOT REGULATED

SECTION 15 - REGULATORY INFORMATION

North American Inventories:

This product or its components are listed on, or exempt from the Canadian Domestic Substances List.

All components are listed or exempt from the TSCA inventory.

This product or its components are listed on, or exempt from the Canadian Domestic Substances List.

U.S. Federal Regulations:

SARA 313 Components

SARA 311/312 Hazards

OSHA Hazardous Components:

Xylene

Amorphous silica Ethylbenzene Coal tar Carbon Black

OSHA Status: Considered hazardous based on the following criteria:

OSHA Flammability

Regulatory VOC (less water and exempt solvent)

VOC Method 310

Xylene Ethylbenzene

Acute Health Hazard Chronic Health Hazard

1330-20-7

7631-86-9

100-41-4

8007-45-2

1333-86-4

Irritant Carcinogen

Not Regulated :    300 g/l

:    30 %

Material Safety Data Sheet

POLYROOF LV3 GL.

Version  1.3

REVISION DATE: 09/01/2006

U.S. State Regulations:

MASS RTK Components

Penn RTK Components

NJ RTK Components

Xylene

Amorphous silica Ethyl benzene Coal tar

Asphalt

Xylene

Styrene-butadiene copolymers

Amorphous silica

Ethylbenzene

Coal tar

Petroleum distillates

Asphalt

Xylene

Styrene-butadiene copolymers

Amorphous silica

Ethylbenzene

Coal tar

Carbon Black

TREHICO

Print Date 04/28/2007

1330-20-7 7631-86-9 100-41-4 8007-45-2

64741-56-6

1330-20-7

NJ TSRN# 51721300-5132P

7631-86-9

100-41-4

8007-45-2

64741-81-7

64741-56-6

1330-20-7

NJ TSRN# 51721300-5132P

7631-86-9

100-41-4

8007-45-2

1333-86-4

~

Chemicals known to the State of California to cause cancer birth defects and/or other reproductive harm:
100-41-4	Ethylbenzene

1333-86-4	Carbon Black

108-88-3	Toluene

218-01-9	Chrysene

56-55-3	Benzo(a)anthracene

SECTION 16 - OTHER INFORMATION

HMIS Rating

| Health | 2 |
| --- | --- |
| Flammability | 2 |
| Reactivity | 0 |
| PPE |  |

- = Minimum

- = Slight

- = Moderate

- = Serious

- = Severe

Further information:

For Industrial Use Only. Keep out of Reach of Children. The hazard information herein is offered solely for the consideration of the user, subject to their own investigation of compliance with applicable regulations, including the safe use of the product under every foreseeable condition.

Prepared by: Rich Mikol