MSDSNo.:	815

Revision No.:      008
Date:	5/13/97

MATERIAL SAFETY DATA SHEET

HILTI, INC.:	P.O. Box 21148, Tulsa, OK 74121; Ph: 1 800 879 6000; Emergency No.: 1 800 879 4444

PRODUCT:	RM 700 EP USDA Epoxy Mortar (Part B)

DESCRIPTION:   Amine hardener for RM700EP epoxy repair mortar

| INGREDIENTS: PEL: | TLV: | STEL: |
| --- | --- | --- |
| NE | NE | NE |
| NE | NE | NE |
| NE | NE | NE |
| 1 ppm | 1 ppm | NE |
| NE | NE | NE |
| NE | NE | NE |
| NE | NE | NE |

Benzyl Alcohol

Aliphatic Polyamine Adduct

Epoxy Resin Adduct

Diethylene Triamine

Bisphenol A

Nonyl Phenol

n-aminoethylpiperazine

INGREDIENTS AND EXPOSURE LIMITS CAS NUMBER:

00100-51-6

Trade Secret *

00694-83-7

00111-40-0

00080-05-7

25154-52-3

00140-31-8

NJ TSRN 05995500-5449P; WHIMIS TSRN 1135-003

PEL = OSHA Permissible Exposure Limit. TLV = ACGIH Threshold Limit Value. These are 8 hour time-weighted averages unless otherwise indicated by "C" (Ceiling) or "STEL" (Short Term Exposure Limit). NE = None Established

PHYSICAL DATA

APPEARANCE AND ODOR: Amber liquid with amine odor. BOILING POINT: 560°F VAPOR PRESSURE: Not Applicable EVAPORATION RATE: Not Determined SPECIFIC GRAVITY:  1.1

VOC CONTENT:  None MELTING POINT: Not Applicable VAPOR DENSITY:  Not Determined SOLUBILITY IN WATER:  Nil

pH: Not Determined

FIRE AND EXPLOSION HAZARD DATA

FLASH POINT: >200 F, Setaflash

FLAMMABLE LIMITS: Not Applicable

EXTINGUISHING MEDIA: Carbon Dioxide, Dry Chemical, Foam, Water Fog

SPECIAL FIRE FIGHTING PROCEDURES:   A self-contained breathing apparatus should be worn when fighting fires involving chemicals.

UNUSUAL FIRE AND EXPLOSION HAZARDS: Thermal decomposition and combustion products can be formed.

REACTIVITY DATA

INCOMPATIBILITY: Strong oxidizers, peroxides, acids.	STABILITY: Stable.

HAZARDOUS DECOMPOSITION PRODUCTS: Thermal decomposition can yield CO, CO2) aldehydes, and acids. HAZARDOUS POLYMERIZATION: Will not occur. CONDITIONS TO AVOID: Excessive heat for prolonged periods of time. Epoxy resins in uncontrolled amounts.

HEALTH HAZARD DATA KNOWN HAZARDS: Corrosive.

CARCINOGENICITY: No ingredients are classified by IARC, NTP, or OSHA as known or potential carcinogens. PRIMARY ROUTES OF EXPOSURE: Dermal.

SIGNS AND SYMPTOMS OF EXPOSURE: Burns to skin and eyes. Respiratory irritation. MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE:  Eye, skin, and respiratory conditions.

EMERGENCY AND FIRST AID PROCEDURES

EYES: Immediately flush with water for at least 15 minutes. Call a physician. SKIN: Wash immediately with soap and water. Launder contaminated clothing before reuse. INHALATION: Move victim to fresh air. Call a physician if symptoms persist.

INGESTION: If conscious, give plenty of water to drink. Seek medical attention. Do not induce vomiting unless directed to by a physician. Never give anything by mouth to an unconscious person.

OTHER: Referral to a physician is recommended if there is any question about the seriousness of the injury/exposure.

CONTROL MEASURES AND PERSONAL PROTECTIVE EQUIPMENT VENTILATION: General (i.e. natural or mechanically induced fresh air movements). EYE PROTECTION: Splash-proof chemical goggles recommended while mixing product. SKIN PROTECTION: Gloves. Cotton or impermeable (neoprene or rubber).

RESPIRATORY PROTECTION: None normally required with adequate ventilation as noted above. Use a NIOSH-approved organic vapor cartridge respirator when working in areas with little or no air movement. Never enter a confined space unless wearing an appropriate air-supplied respirator (e.g. SCBA or airline with

escape pack).

PRECAUTIONS FOR SAFE HANDLING AND USE

HANDLING AND STORING PRECAUTIONS:   Avoid contact with eyes, skin, or clothing.  Wash thoroughly after handling. Keep from freezing.

SPILL PROCEDURES:   Remove spillage with absorbent material and dispose in accordance with federal, state, and local regulations.

REGULATORY INFORMATION

OSHA HAZARD COMMUNICATION: This MSDS has been prepared in accordance with the federal OSHA Hazard Communication Standard.

HMIS CODES: Healths, Flammability 1, Reactivity 0, PPE B, Goggles

DOT SHIPPING NAME:   Corrosive liquids, n.o.s. (contains diethylenetriamine, n-aminoethylpiperazine), Class 8, UN 1760, Pkg. Gp. Ill, Ltd. Qty.

TSCA INVENTORY STATUS: Chemical components listed on TSCA inventory.

SARA TITLE III, SECTION 313: This product does not contain any toxic chemicals which are subject to reporting under Section 313 of SARA Title III (40 CFR Part 372).

WASTE DISPOSAL METHODS: Consult with regulatory agencies or corporate personnel for disposal methods that comply with local, state, and federal health and environmental regulations.

H EA LTH / SA FETY CONTACTS:    Steve Gerrard (x6309)

Jerry Metcalf   (x6704)

The information and recommendations contained herein are based upon data believed to be correct; however, no guarantee or warranty of any kind expressed or implied is made with respect to the information provided.