MATERIAL SAFETY DATA SHEET

Date Prepared: September 28, 2005

1. PRODUCT/COMPANY IDENTIFICATION

Product

BONSAL® Cl 107

Emergency Telephone:

800-424-9300 (Chemtrec) or 703-527-3887 (Outside USA)

Manufacturer's Name & Address:

Bonsai American/an Oldcastle Company 8201 Arrowridge Blvd Charlotte, NC 28273

Telephone Number for Information:

704-525-1621

2. EMERGENCY AND FIRST AID

J

EMERGENCY INFORMATION:

EYES:

SKIN:

INHALATION:

INGESTION:

Bonsai® Cl 107 Grout is a light gray cementitious powder blend. When in contact with moisture in eyes or on skin, or when mixed with water, it becomes highly caustic (pH > 12) and will damage or burn (as severely as third-degree) the eyes or skin. Inhalation may cause irritation to the moist mucous membranes of the nose, throat and upper respiratory system or may cause or may aggravate certain lung diseases or conditions. Use exposure controls or personal protection methods described in Section 10.

Immediately flush eye thoroughly with water. Continue flushing eye for at least 15 minutes, including under lids, to remove all particles. Call physician immediately.

Wash skin with cool water and pH-neutral soap or a mild detergent. Seek medical treatment-if irritation or inflammation develops or persists. Seek immediate medical treatrnent in the event :of burns, >:;;.:•:/: I-;:c^'^-^Cf-^;.v:;.XV:.v':'':^-:V^'.^:-'vi.•-.•;••. •••'. ':•'••. ;:;;

Remove person to fresh air. If breathing is difficult, administer oxygen. If not breathing, give artificial respiration. Seek medical help if coughing and other symptoms do not subside. Inhalation of large amounts of Cl 107 Grout require immediate medical attention.

Do not induce vomiting. If conscious, have the victim drink plenty of water and call a physician immediately.

-~

3. COMPOSITION INFORMATION

DESCRIPTION:

This product consists of finely ground hydraulic cement, major compounds are:

3CaOSiO2 2CaO-SiO2 3CaOAl2O3 4CaO«AI2O3-Fe2O3

CaSO4'2H2O • SiOi

Tricalcium Silicate Dicalcium Silicate Tricalcium Aluminate Tetracalcium aluminoferrite Calcium Sulfate dihydrate (Gypsum) Silica Sand

CAS #12168-85-3 CAS #10034-77-2 CAS #12042-78-3 CAS #12068-35-8

CAS #7778-18-9 (CAS #13397-24-5) CAS #14808-60-7

4. HAZARDOUS INGREDIENTS

COMPONENT

Hydraulic Cement

Calcium sulfate (CAS #7778-18-9) [Gypsum (CAS #13397-24-5)]

OSHA PEL (8-Hour TWA)

5 mg respirable dust/m3 15 mg total dust/m

5 mg respirable dust/m3 15 mg total dust/m3

ACGIHTLV-TWA (1995-1996)

lOmg total dust/m3

10 mg total dust/m

NIOSH REL (8-Hour TWA)

Iron oxide (CAS #1309-37-1)

10 mg/m3

5 mg/m3

Calcium carbonate (CAS #1317-65-3)

5 mg respirable dust/m

15 mg total dust/m3

10 mg total dust/m3

Magnesium oxide (CAS #1309-48-4)

15 mg total dust/m3	10 mg total dust/m3

Calcium oxide (CAS #1306-78-8)

5 mg/m3

2 mg/m3

10 mg of respirable

0.10 mg respirable	0.05 mg respirable quartz

Crystalline silica (CAS #14808-60-7)

dust/m3 % SiO2 + 2 30 mg of total dust/m-% SiO2 + 2

quartz/m3

dust/m

250 million particles/ft3-% SiO2 + 5 TRACE INGREDIENTS:

Due to the use of substances mined from the earth's crust, trace amounts of naturally occurring, potentially harmful constituents may be detected during chemical analysis.

5. HAZARD IDENTIFICATION

^

POTENTIAL HEALTH EFFECTS:

EYE CONTACT:

SKIN CONTACT:

INHALATION:

NOTE: Potential health effects may vary depending upon the duration and degree of exposure. To reduce or eliminate health hazards associated with this product, use exposure controls or personal protection methods as described in Section 10.

(Acute/Chronic) Exposure to airborne dust may cause immediate or delayed irritation or inflammation of the cornea. Eye contact by larger amounts of dry powder or splashes of wet Cl 107 Grout may cause effects ranging from moderate eye irritation to chemical burns and blindness.

(Acute) Exposure to dry Cl 107 Grout may cause drying of the skin with consequent mild irritation or more significant effects attributable to aggravation of other conditions. Discomfort or pain cannot be relied upon to alert a person to a hazardous skin exposure.

(Chronic) Dry Cl 107 Grout coming in contact with wet skin or exposure to wet Cl 107 Grout may cause more severe skin effects, including thickening, cracking or fissuring of the skin. Prolonged exposure can cause severe skin damage in the form of chemical (caustic) burns.

(Acute/Chronic) Some individuals may exhibit an allergic response upon exposure to Cl 107 Grout. The response may appear in a variety of forms ranging from a mild rash to severe skin ulcers.

(Acute) Exposure to Cl 107 Grout may cause irritation to the moist mucous membranes of the nose, throat and upper respiratory system. Pre-existing upper respiratory and lung diseases may be aggravated by inhalation.

(Chronic) Inhalation exposure to free crystalline silica may cause delayed lung injury including silicosis, a disabling and potentially fatal lung disease, and/or cause or aggravate other lung diseases or conditions.

INGESTION:

(Acute/Chronic) Internal discomfort or il large quantities are swallowed.'

effects are possible if

CARCINOGENIC

C j 107; Grout /is not reebgnized as a carcinogenby NTP;. or IARC. However, it may contain trace amounts of heavy metals recognized as carcinogens by these organizations. In addition, IARC classifies crystalline silica, a trace constituent, as a known human carcinogen (Group I). NTP has characterized respirable silica as "reasonably anticipated to be a carcinogen." (See also Section 13.)

| 1 6. PHYSICAL/CHEMICAL DATA |  |  | i |
| --- | --- | --- | --- |
| APPEARANCE/ODOR: | Gray, white or colored powder, odorless | PHYSICAL STATE: | Solid (Powder) |
| BOILING POINT: | > 1000/C | MELTING POINT: | Not applicable |
| VAPOR PRESSURE: | Not applicable | VAPOR DENSITY: | Not applicable |
| pH (IN WATER) (ASTM D 1293-95) | 12to 13 | SOLUBILITY IN WATER: | Slightly soluble (0,1% to 1.0%) |
| SPECIFIX: GRAVITY (II2O= 1.0): | ^JJ^;^S'.'^^fI\K^::: | :i?y a potation: rate: '•; ;:•;•;':/ | ;;:';-:-Not-appjiehH¦p'.:--.' •'.. '"". '••'.'. •'•::~'-:- •:..•:'. : .: '../;• |
| 1 | 7. FIRE | AND EXPLOSION | 1 |

FLASH POINT:

AUTO IGNITION TEMPERATURE:

None

Not combustible

LOWER EXPLOSIVE LIMIT:      None UPPER EXPLOSIVE LIMIT:        None

FLAMMABLE LIMITS     Not applicable

EXTINGUISHING MEDIA:

Not combustible

SPECIAL FIRE FIGHTING	None

PROCEDURES:

UNUSUAL FIRE AND	None

EXPLOSION HAZARDS:

| HAZARDOUS COMBUSTION PRODUCTS: | None |
| --- | --- |
| 8. STABILITY AND REACTIVITY DATA |  |

STABILITY: CONDITIONS TO AVOID:

INCOMPATIBILITY:

HAZARDOUS DECOMPOSITION: HAZARDOUS POLYMERIZATION:

Product is stable. Keep dry until used.

Unintentional contact with water. Contact with water will result in hydration and produces (caustic) calcium hydroxide.

Wet Cl 107 Grout is alkaline. As such, it is incompatible with acids, ammonium salts and aluminum metal.

Will not occur. Will not occur.

9. PRECAUTIONS FOR HANDLING, STORAGE AND DISPOSAL

HANDLING AND STORAGE

SPILL:

DISPOSAL:

Keep dry until used. Handle and store in a manner so that airborne dust does not exceed applicable exposure limits. Use adequate ventilation and dust collection. Use exposure control and personal protection methods as described in Section 10.

Use dry clean-up methods that do not disperse dust into the air or entry into surface water. Material can be used if not contaminated. Place in an appropriate container for disposal or use. Avoid inhalation of dust and contact with skin and eyes. Use exposure control and personal protection methods as described in Section 10.

Comply with all applicable local, state and federal regulations for disposal of unusable or contaminated materials. Dispose of packaging/containers according to local, state and federal regulations.

10. EXPOSURE CONTROLS/PERSONAL PROTECTION

~

RESPIRATORY PROTECTION:

Use local exhaust or general dilution ventilation to control dust levels below applicable exposure limits. Minimize dispersal of dust into the air.

If local or general ventilation is not adequate to control dust levels below applicable exposure limits or when dust causes irritation or discomfort, use MSHA/NIOSH approved respirators.

EYE PROTECTION: W

SKIN PROTECTION:

Wear Safety glasses with^sitje ^sbtelds or gbgglesitp^avoiid contact with the eyes. In extremely dusty environments and unpredictable environments, wear tight-fitting unvented or indirectly vented goggles to avoid eye irritation or injury. Contact lenses should not be worn when handling cement or cement containing products.

Wear impervious abrasion- and alkali-resistant gloves, boots, long-sleeved shirt, long pants or other protective clothing to prevent skin contact. Promptly remove clothing dusty with dry Cl 107 Grout or clothing dampened with moisture mixed with Cl 107 Grout, and launder before re-use. If contact occurs, wash areas contacted by material with pH neutral soap and water.

11. TRANSPORTATION DATA

Cl 107 Grout is not hazardous under U.S. DOT or TDG regulations.

12. OTHER REGULATORY INFORMATION

Status under US OSHA Hazard Communication Rule 29 CFR 1910.1200:

Status under CERCLA/Siiperfund, 40 CFR 117 and 302:      f^Uh'^^^^j^/^0^

Hazard Category under SARA (Title III), Sections 311 and 312:

Status under SARA (Title 111), Section 313: Status under TSCA (as of May 1997):

Status under the Federal Hazardous Substances Act:

Status under California Proposition 65:

Status under Canadian Environmental Protection Act:

Status under Canadian WHMIS:

Cl 107 Grout is considered a hazardous chemical under this
regulation and should be included in the employer's hazard
communication program.	• ••"'..

.Not listed;:;'':, -.;•./.-\;. •v.:Vv-,:-;;•„::. i:VA:   vV-      '..'.'•:;;.''•'. '.,:-'.-:.'.:;::

Cl 107 Grout qualifies as a hazardous substance with delayed health effects.

Not subject to reporting requirements under Section .113.

Some substances in Cl 107 Grout are on the TSCA inventory list.

Cl 107 Grout is a hazardous substance subject to statutes promulgated under the subject act.

This product contains crystalline silica, a substance known to the State of California to cause cancer. This product also may contain trace amounts of heavy metals known to the State of California to cause cancer, birth defects or other reproductive harm.

Not listed.

Cl 107 Grout is considered to be a hazardous material under the Hazardous Products Act as defined by the Controlled Products Regulations (Class D2A, E - Corrosive Material) and subject to the requirements of WHMIS.

13. OTHER INFORMATION

The information and recommendations contained herein are based upon data believed to be correct. However, no guarantee or warranty of any kind, express or implied, is made with respect to the information contained herein. It is the user's obligation to determine the conditions of safe use of this product.