flinn scientific, inc.

Material Safety Data Sheet (MSDS)	MSDS#: 801 °2

: (MSDS)

—————	Revision Date:August 10, 2006

SECTION 1 — CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

Sulfuric Acid Solution

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

SECTION 2 —COMPOSITION. INFORMATION ON INGREDIENTS

Sulfuric Acid (7664-93-9) 5.3-32%, Water (7732-18-5) 68-94.7% 6M, 5M, 3M, 2M, 1.5M, 1M

CAS#: None Established

SECTION 3 — HAZARDS IDENTIFICATION

Clear, colorless liquid. Sulfurous odor.	flinn Ar-A-GLANCE

Moderately toxic by ingestion. Corrosive to eye, skin, and all other body tissues.	Health-2

Avoid all body tissue contact.	Flammability-0

Very considerable heat generated when diluted with water.	Reactivity-2

Exposure-2 Storage-3

0 is low hazard, 3 is high hazard SECTION 4—FIRST AID MEASURES

lall a physician, seek medical attention for further treatment, observation and support after first aid.

jhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately. Eye: Immediately flush with fresh water for 15 minutes. External: Wash continuously with fresh water for 15 minutes.

Internal: Give 1 to 2 cups of water or milk, followed by a gastric antacid, such as milk of magnesia. Do not induce vomiting. Call a physician or poison control at once.

SECTION 5 — FIRE FIGHTING MEASURES

Non flammable liquid.	NFPA code
When heated to decomposition, emits toxic fumes of SOx.	None

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE	Established
and SCBA with full facepiece operated in positive pressure mode.

SECTION 6 — ACCIDENTAL RELEASE MEASURES

Restrict unprotected personnel from area. Remove all ignition sources and ventilate area. Contain spill with sand and absorbent material, neutralize with sodium bicarbonate or calcium hydroxide and deposit in sealed bag or container. See Sections 8 and 13 for further information.

SECTION 7 — HANDLING AND STORAGE

Flinn Suggested Chemical Storage Pattern: Inorganic #9. Store with acids.

Store in a dedicated acid cabinet and away from any source of water; if an acid cabinet is not available, store in Flinn Saf-Cube.

Use and dispense in a hood.

SECTION 8 — EXPOSURE CONTROLS. PERSONAL PROTECTION

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. •? ventilation to keep airborne concentrations below exposure limits. Always wear a NIOSH-approved respirator with proper , tridges or a positive pressure, air-supplied respirator when handling this material in emergency situations (spill or fire).

Exposure guidelines: TWA 1 mg/m3, STEL 3 mg/m3 (OSHA, ACGIH)

flinn scientific, inc.

Material Safety Data Sheet     Sulfuric Acid Solution

MSDS#:   801.02

Revision Date: August 10, 2006

ECTION 9 — PHYSICAL AND CHEMICAL PROPERTIES

Clear, colorless liquid. Sulfurous odor.

Solubility: Miscible with water evolving much heat.

Concentration: 1-6 Molar

Formula: H2S04

Formula Weight: 98.08

SECTION 10 — STABILITY AND REACTIVITY

Avoid contact with strong bases, reacts violently with water. Always add acid to water, never the reverse. Shelf Life: Good, if stored safely.

SECTION 11 — TOXICOLOGICAL INFORMATION

Acute effects: Severely corrosive Chronic effects: N.A. Target organs: Eyes, skin

ORL-RAT LD50: 2140 mg/kg as sulfuric acid IHL-RAT LC50: 510 mg/m3/2H SKN-RBT LD50: N.A.

N.A. = Not available, not all health aspects of this substance have been fully investigated.

SECTION 12 — ECOLOGICAL INFORMATION

Data not yet available.

SECTION 13 — DISPOSAL CONSIDERATIONS

Please consult with state and local regulations.

•inn Suggested Disposal Method #24b is one option.

r

SECTION 14 — TRANSPORT INFORMATION

Shipping Name: Sulfuric Acid Hazard Class: 8, Corrosive UN Number: UN2796 N/A = Not applicable

SECTION 15 — REGULATORY INFORMATION

TSCA-listed, EINECS-listed (231-639-5), RCRA code D002, D003.

SECTION 16 — OTHER INFORMATION

This Material Safely Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific, Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. The data should not be confused with local, state, federal or insurance mandates, regulations, or requirements and CONSTITUTE NO WARRANTY. Any use of this data and information must be determined by the science instructor to be in accordance with applicable local, state or federal laws and regulations. The conditions or methods of handling, storage, use and disposal of the product(s) described are beyond the control of Flinn Scientific, Inc. and may be beyond our knowledge. FOR THIS AND OTHER REASONS, WE DO NOT ASSUME RESPONSIBILITY AND EXPRESSLY DISCLAIM LIABILITY FOR LOSS, DAMAGE OR EXPENSE ARISING OUT OF OR IN ANY WAY CONNECTED WITH THE HANDLING, STORAGE, USE OR DISPOSAL OF THIS PRODUCT(S).

Consult your copy of the Flinn Chemical Catalog/Reference Manual for additional information about laboratory chemicals

Scientific, Inc. All Rights Reserved.

page 2 of 2