FLinn scientific me.

"Your Safer Source for Science Supplies"

Material Safety Data Sheet (MSDS)

MSDS #:    227.00

Revision Date:   November 25, 2002

SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

Cetyl Alcohol

MSDS #:     227.00

Revision Date:   November 25, 2002

Section 1 — Chemical Product and Company Identification

Section 9 — Physical and Chemical Properties

Cetyl Alcohol

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

White powder or flakes; faint citrus-like odor.	Specific Gravity: 0.82

Solubility: Partially soluble in alcohol Insoluble in water.	Boiling Point: 180 C

Formula: CH3(CH2)14CH2OH	Melting Point: 54-56 C
Formula Weight: 242.50

| Section 2 — Composition. Information on Inaredients |  |
| --- | --- |
| Cetyl Alcohol Synonym 1-hexadecano! CASH: 36653-82-4 |  |
| Section 3 — Hazards Identification |  |
| White powder or flakes; faint citrus-like odor. Mildly toxic by ingestion. Body tissue irritant. Avoid ail body tissue contact. | flinn at-a-glance Health-l Flammability-1 Reactivity-0 Exposure-! Storage-0 0 is low hazard, 3 is high hazard |
| Section 4 — First Aid Measures |  |

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give large quantities of water. Call a physician or poison control at once.

Section 5 — Fire Fiahtinq Measures

Combustible solid.	NFPA code

When heated to decomposition, emits acrid smoke and fumes.	None Established

Flash Point: 230 F    Upper: 8%    Lower: 1%    Autoignition Temperature: 483 F

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE and

SCBA with full facepiece operated in positive pressure mode.

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and i3 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Organic #2. Store with alcohols, glycols, amines and amides. Store in a cool dry place. Use fume hood when heating

Section 10 — Stability and Reactivity

Avoid contact with strong oxidizers. Shelf life Indefinite

Section 11 — Toxicolooical Information

Acute effects: Irritant Chronic effects: N.A. Target organs. N.A.

ORL-RAT LD50: 5 gm/kg IHL-RAT LC50: N.A. SKN-RBT LD50: N.A

N.A = Not available, not all health aspects of this substance have been fully investigated.

Sectioni 12! —- EcoloQicaiUi formation

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method 18b is one option.

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A N/A = Noi applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (253-I49-0).

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. Use exhaust ventilation to keep airborne concentrations low

Flinn Chemical Packaging Prevents Accidents

fltnn@flinnsci.com   www.flinnsci.com

P.O. Box 219   Batavia IL 60510 (800)452-1261 Fax (866) 452-1436

© 2002 Flinn Scientific, Inc. All Rights Reserved

page 1 of 2

© 2002 Flinn Scientific, [nc. All Riahts Reserved.