MSDS#: 9895 Status: Current

Status Date: 11/15/00 Product Name: C.P. DYNEX Internal MSDS#: 857

Current Company Information:

U.N.X. INCORPORATED

707 ARLINGTON BLVD.

GREENVILLE, NC USA 27858

Phone: 252-756-8616     Emergency: 800-255-3924 CHEMTEL

Current Company Short Name:

Manufacturer/Supplier: U.N.X. INCORPORATED 707 ARLINGTON BLVD. GREENVILLE, NC USA 27858 Phone: 252-756-8616     Emergency: 800-255-3924 CHEMTEL

MSDS Prepared: 4/12/93 MSDS Revised: 2/8/00 Keyword: PRESOAK DESTAINER Synonym(s): IG

Physical/Chemical Characteristics

pH:BT 10.4 10.8 Solubility In Water: EXCELLENT

Appearance/Odor: WHITE GRANULAR POWDER WITH NO ODOR Physical State: Solid

DOT INFORMATION:

DOT hazard Class: NON-HAZARDOUS

Label: NONE

Proper Shipping Name: NON HAZARDOUS

Preparers Info.: AMENDT, EVANS, KINCAID

*************************************** COMPOSITION/INFORMATION ON INGREDIENTS

Component Name: SODIUM TRIPOLYPHOSPHATE

Notes: ACUTE ORAL LD50 3.0 g/kg(rat), ACUTE DERMAL LD 50 >2g/kg (rabbit)

Percent: BT    5    15

CAS No.: 7758-29-4

Exposure Limits

Component Name: SODIUM PERBORATE Percent: BT    20    30 CAS No.: 7632-04-4 Exposure Limits

Component Name: SODIUM SULFATE Percent: BT    25    35 Exposure Limits

Component Name: SODIUM METASILICATE Percent: BT    10    20 CAS No.: 6834-92-0 Exposure Limits

Component Name: SODIUM BICARBONATE Percent: BT    10    20 Exposure Limits

FIRE AND EXPLOSION DATA

SPECIAL FIRE FIGHTING PROCEDURES: USE NIOSH/MSHA APPROVED POSITIVE PRESSURE SELF-CONTAINED BREATHING APPARATUS WHEN MATERIAL IS INVOLVED IN A FIRE UNUSUAL FIRE AND EXPLOSION HAZARD: MAY REACT WITH ORGANIC MATERIAL IN PRESENCE OF MOISTURE TO PRODUCE HEAT AND COMBUSTION. AVOID CONTACT WITH STRONG ACIDS.

REACTIVITY DATA

VERY STABLE. CONDITION TO AVOID: EXCESSIVE HEAT AND MOISTURE

INCOMPATIBILITY:OXIDIZABLE OR COMBUSTIBLE ORGANIC MATERIALS, STRONG

OXIDIZERS,NITROGEN COMPOUNDS, ACIDS,ALKALI

HAZARDOUS POLYMERIZATION: WILL NOT OCCUR.

HAZARDOUS DECOMPOSITION PRODUCTS: DECOMPOSES TO GIVE OFF HEAT AND OXYGEN

HEALTH HAZARD DATA

ACUTE OR CHRONIC: INHALATION, CAN IRRITATE NOSE,THROAT, AND LUNGS

SKIN CONTACT, CAN IRRITATE OR CHAP SKIN

INGESTION, WILL CAUSE GASTRIC DISTRESS, LARGE ORAL

DOSES MAY CAUSE SEVERE INTESTINAL DISORDERS AND

POSSIBLE INTESTINAL BLEEDING. CARCINOGENICITY: NOT APPLICABLE

SIGNS AND SYMPTOMS OF EXPOSURE: PRODUCT.EITHER DRY OR IN SOLUTION.IS IRRITANT TO TISSUE. CONTACT WITH EYES CAN CAUSE IRRITATION OR DAMAGE. FIRST AID: FOR SKIN, FLUSH THOROUGHLY WITH WATER UNTIL "SLIPPERY" FEELING IS GONE. FOR EYES, FLUSH WITH WARM WATER FOR AT LEAST 15 MINUTES. CONTACT PHYSICIAN. IF INGESTED.DRINK LARGE VOLUMES OF WATER FOLLOWED BY MILD ACIDS SUCH AS 5% ACETIC ACID.ORANGE JUICE,OR ANY CITRUS JUICE. INDUCE VOMITING. CALL PHYSICIAN.

PRECAUTIONS FOR SAFE HANDLING AND USE

STEPS FOR SPILL OR RELEASE: FOR SMALL SPILL, IMMEDIATELY HOSE DOWN WITH COOL WATER AND DISPOSE DOWN DRAIN. FOR LARGE SPILL, COLLECT AND ADD SLOWLY TO LAR( VOLUME OF COOL WATER WITH CONSTANT STIRRING. THEN SLOWLY NEUTRALIZE TO pH OF 7.0 WITH MURIATIC OR PHOSPHORIC ACID SOLUTIONS. DISPOSE OF DOWN DRAIN AFTER APPROVAL OF APPROPRIATE AUTHORITY. CLEAN AND NEUTRALIZE ALL AREAS OF CONTACT.

WASTE DISPOSAL METHODS:DISPOSE OF CONTAMINATED PRODUCT, EMPTY CONTAINERS AN MATERIALS USED IN CLEANING UP THE SPILLS OR LEAKS IN A MANNER APPROVED FOR THIS MATERIAL. CONSULT APPROPRIATE FEDERAL, STATE AND LOCAL REGULATORY

AGENCIES TO ASCERTAIN PROPER DISPOSAL PROCEDURES.

HANDLING AND STORING: STORE IN DRY PLACE. DO NOT STORE WITH STRONG ACIDS

KEEP CONTAINER CLOSED. AVOID MOISTURE.

CONTROL MEASURES

RESPIRATORY PROTECTION: AVOID BREATHING DUST. IF EXPOSURE LIKELY, USE FILTER OR DUST TYPE RESPIRATOR. MECHANICAL VENTILATION IS ACCEPTABLE. RUBBER OR PLASTIC GLOVES SHOULD BE WORN. SAFETY GLASSES OR GOGGLES SHOULD BE WORN.

SUGGESTED PROTECTIVE MEASURES: EYE BATH AND SAFETY SHOWER IF POTENTIAL FOR SKIN CONTACT, WEAR PROTECTIVE GLOVES AND HAVE AN ACID HAN WASH NEAR BY(ie.VINEGAR SOLUTION.).

WORK/HYGENIC PRACTICES: WASH HANDS.CHANGE OUT OF CLOTHES AS SOON AS POSSIBLE. WASH CLOTHES. SHOWER OR BATHE AS SOON AS POSSIBLE.

OTHER INFORMATION

UNLESS OTHERWISE NOTED, NO COMPONTENTS ARE SARA TITLE 3 SECTION 313 40 CFR

LISTED MATERIALS

NOT MADE WITH VOC'S THAT COULD CAUSE DAMAGE TO THE OZONE LAYER

THE INGREDIENTS OF THIS PRODUCT ARE LISTED ON THE TSCA INVENTORY

FTCL9

PKPA, PKPD, PKLJ, PKPH, PKPJ, PKPL END OF REPORT.