"

~

SSS05224 MAINTENANCE SUPPLY COMPANY, INC.

POST OFFICE BOX 498 HUNTERSVILLE, NORTH CAROLINA 28070

- MATERIAL SAFETY DATA SHEET -

THESE MATERIAL SAFETY DATA SHEETS ARE SENT TO YOU IN COMPLIANCE WITH 29 CFR 1910.1200 WHICH BECAME EFFECTIVE NOVEMBER 25, 1985.

IDENTITY AND DISTRIBUTOR'S INFORMATION

NFPA rating:  Health, 1    Flammabi1ity, 4    Reactivity, 0

Special, 0 HMIS RATING: Health, 1    Flammabi1ity, 4    Reactivity, 0

Personal Protection, B

MANUFACTURED FOR:  Triple S

2 Executive Park Drive

Bill erica, MA 01862 DOT HAZARD CLASSIFICATION:  ORM-D

IDENTITY (TRADE NAME AS USED ON LABEL): SSS CHEWING GUM REMOVER
DATE PREPARED:     02/23/2000	PREPARED BY:  ES

MANUFACTURER'S MSDS NUMBER:  A00183    REVISION - 4 DISTRIBUTOR'S MSDS REFERENCE:    21795 INFORMATION CALLS:  (978) 667-7900 EMERGENCY RESPONSE NUMBER :  1 (800) 255-3924 NOTICE:  JUDGMENT BASED ON INDIRECT TEST DATA

SECTION 1 - MATERIAL IDENTIFICATION AND INFORMATION

COMPONENTS	SARA      OSHA       ACGIH

CHEMICAL AND COMMON NAMES      III List   PEL (ppm)   TLV (ppm)

(Hazardous Components 1% or greater; Carcinogens 0.1% or greater) isobutane / Propane Blend  ~    No        800       800 (CAS # 75-28-5)

(CAS # 74-98-6)	No        1000       1000

carcinogen Reference Source:  None of the above listed.

-1-

SECTION 2 - PHYSICAL/CHEMICAL CHARACTERISTICS

boiling POINT: (concentrate only) = -43.7 deg. F.

SPECIFIC GRAVITY, (H20=l): Concentrate Only = 0.5379

VAPOR PRESSURE: PSIG @ 70 deg. F. (Aerosols): 70

vapor PRESSURE (Non-Aerosols) (mm Hg and Temperature): n/a

vapor density (Air=l): Concentrate only = greater than 1.5

EVAPORATION RATE, (BuAc=l): Faster

SOLUBILITY IN WATER: Slight

WATER REACTIVE: No

APPEARANCE AND ODOR: Clear, odorless spray.

SECTION 3 - FIRE AND EXPLOSION HAZARD DATA FLAMMABILITY as per USA FLAME PROJECTION TEST (Aerosols):

extremely flammable auto ignition temperature: n/e flammability limits in air by % in volume:

% lel: 2.0     % uel: 10.0

flash point and method used (non-aerosols): -156 deg. F. extinguisher media: Foam, dry chemical, carbon dioxide. special fire fighting PRpCEDURE: Cool containers with water.

wear self-contained breathing apparatus. unusual fire & explosion hazards: Do not expose aerosols to

temperatures above 130 degrees F. or the container may

rupture.

Page 1

SSS05224 SECTION 4 - REACTIVITY HAZARD DATA

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not OCCUP

incompatibility (materials TO avoid): strong oxidizing agents. CONDITIONS TO AVOID: Open flame, welding arcs, heat, sparks or any source of ignition.

HAZARDOUS DECOMPOSITION PRODUCTS: CO, C02.

-2-

SECTION 5 - HEALTH HAZARD DATA primary ROUTES OF ENTRY: inhalation

ACUTE EFFECTS:

Inhalation: Product is an asphyxiant at very high concentrations.  Excessive inhalation of vapors can be harmful and may cause headache, disorientation, rapid respiration, nausea, anesthetic effects and possible unconsciousness, vapors are heavier than air and displace oxygen required for breathing.

Eye Contact: May cause burns and frostbite.

Skin Contact: May cause burns and frostbite.

ingestion: Unlikely route of exposure. Gas under normal

(usual) circumstances. chronic effects : Unknown.

MEDICAL CONDITIONS GENERALLY AGGRAVATED BY EXPOSURE: May

aggravate existing eye, skin or upper respiratory conditions.

EMERGENCY FIRST AID PROCEDURES:

eye contact: Flush immediately with fresh water for at least 15 minutes while holding eyelids open. Remove contact lenses if worn. Seek medical attention immediately.

SKIN CONTACT: Treat burned or frostbitten skin by flushing or immersing affected areas in lukewarm water.  If skin is not burned, keep warm and stimulate circulation with massage,  seek medical attention immediately.

inhalation: Remove to fresh air.  Resuscitate if necessary. Get medical attention. Give oxygen.

INGESTION: Unlikely route of exposure.

SECTION 6 - CONTROL AND PROTECTIVE MEASURES

RESPIRATORY PROTECTION (specify type): If vapor concentration

exceeds JLV, use respirator approved by NIOSH to be used in a positive pressure mode.

protective GLOVES: Rubber gloves recommended.

eye protection: Safety glasses recommended.

ventilation REQUIREMENTS: Adequate ventilation to keep vapor concentration below tlv.

OTHER PROTECTIVE CLOTHING AND EQUIPMENT: Self-COntained

respirator should be available for non-routine and emergency situations.

hygienic work practices: wash with soap and water before handling food.  Remove contaminated clothing.

SECTION  7 - PRECAUTIONS FOR SAFE HANDLING AND USE

STEPS TO BE TAKEN IF MATERIAL IS SPILLED OR RELEASED:

isolate hazard area and deny entry.  Remove all ignition sources, ventilate area to disperse vapors,  if liquid gas has not ignited, disperse with water or by flooding.

WASTE DISPOSAL METHODS:  Aerosol cans when vented to atmospheric

Page 2

S

SS05224 pressure through normal use, pose no disposal hazard.

PRECAUTIONS TO BE TAKEN IN HANDLING AND STORAGE: Do not puncture

or Incinerate containers. Do not store at temperatures above 130 degrees F.

OTHER PRECAUTIONS AND/OR SPECIAL HAZARDS: KEEP OUT OF REACH OF

children. Avoid food contamination. Avoid breathing vapors. Avoid contact with skin or eyes.

We believe the statements, technical information and recommendations contained herein are reliable, but they are given without warranty or guarantee of any kind.

-3-