MATERIAL   SAFETY   DATA   SHEET:

Equilon MSDS:  01854ET			04/22/99

ATF Mercon/Dexron III

Telephone Number:

24 Hour Emergency Assistance			General MSDS Assistance

Equiva Services:  877-276-7283			877-276-7285

Chemtrec:  800-424-9300

Name and Address:

Equillon Enterprises, LLC

Product Stewardship

P.O. Box 674414

Houston, TX  77267-4414

Legend:

N.D. – None Detected	N.A. – Not Applicable	N.T. – Not Tested

< - Less Than		> - More Than

SECTION   I   -   NAME

MATERIAL IDENTITY

Product Code and Name:

01854 ATF MERCON/DEXROM III

Chemical Name and/or Family or Description:

Transmission Fluid

SECTION II – COMPOSITION/INFORMATION ON INGREDIENTS

The criteria for listing components in the composition section is as follows:  Carcinogens are listed when present at 0.1% or greater, components which are otherwise hazardous according to OSHA are listed when present at 1.0% or greater, non-hazardous components are listed as 3.0% or greater.  This is not intended to be a complete compositional disclosure.  Refer to section 14 for applicable states right to know and other regulatory information.

Product and/or Component(s) Carcinogenic According to:

OSHA	IARC	NTP	OTHER   NONE (X)

Composition:  Sequence no. and chemical name.  Sequence Chemical Name

01#	Hydrotreated Heavy Paraffinic		 Petroleum Distillates

02#	Hydrotreated Light Paraffinic Petroleum Distillate

03#	Hydrotreated Light Naphthenic Petroleum Distillate

04+	Polymethacrylate

05+	Boronated Substituted Succinimide

Product is non-hazardous according to OSHA (1900.1200)

+  Component is hazardous according to OSHA

#  Component, by definition, is considered hazardous according to OSHA, because it carries the permissible exposure limit (PEL) for mineral oil mist.

CAS Number		Range in %

64742-54-7		65.00 – 79.99

64742-55-8		10.00 – 19.99

64742-53-6		1.00 – 2.99

50867-55-5		1.00 – 2.99

1.00 – 2.99

Exposure Limits referenced by Sequence Number in the Composition Section

SECTION   III   -   HAZARD IDENTIFICATION

Emergency Overview:  Appearance – Dyed red, oily liquid		Odor:  Mild odor

Warning Statement:  None Considered Necessary

HMIS	Health:1		Reactivity: 0		NFPA		Health: 1	Reactivity:  0

Flammability:  1	  Special:  - 				Flammability:  1	Special: -

Potential Health Effects:		Eye	Skin	Inhalation	Ingestion

X	X	X

Effects of Overexposure		-	-	-		-

Acute:

Eyes:  May cause minimal irritation, experienced as temporary discomfort.

Skin:  Brief contact is not irritating.  Prolonged contact, as with clothing wetted with material, may cause defatting of skin or irritation, seen as local redness with possible mild discomfort.  Other than the potential skin irritation effects noted above, acute (short term) adverse effects are ot expected from brief skin contact, see other effects below and Section 11 for information regarding potential long term effects.

Inhalation:  Vapors or mist, in excess of permissible concentrations, or in unusually high concentrations generated from spraying, heating the material or as from exposure in poorly ventilated areas or confined spaces, may cause irritation of the nose and throat, headache, nausea, and drowsiness.

Ingestion:  If more than several mouthfuls are swallowed, abdominal discomfort, nausea, and diarrhea may occur.

Sensitization Properties:  Unknown

Chronic:  No adverse effects have been documented in humans as a result of chronic exposure.  Section 11 may contain applicable animal data.

Medical Conditions Aggravated by Exposure:  Because of its irritating properties, repeated skin contact may aggravate an existing dermatitis (skin condition).

Other Remarks:   None

SECTION IV – FIRST AID MEASURES

Eyes:  Flush eyes with plenty of water for several minutes.  Get medical attention.

Skin:  Wash skin with plenty of soap and water for several minutes.  Get medical attention if skin irritation develops or persists.

Inhalation:  Drowsiness occurs, remove to fresh air and get medial attention if breathing becomes difficult or respiratory irritation persists.

Other Instructions:  Remove and dry-clean or laundry clothing soaked or soiled with this material before resuse.  Dry cleaning of contaminated clothing may be more effective than normal laundering.  Inform individuals responsible for cleaning of potential hazards associated with handling contaminated clothing.

SECTION V – FIRE FIGHTING MEASURES

Ignition Temperature – AIT 9degrees F):  Not determined

Flash Point (degrees F):  >350 (COC)

Flammable Limits (%):  Lower:  Not determined    Upper:  Not determined

Recommended Fire Extinguishing Agents and Special Procedures:  Use water spray, dry chemical, foam, or carbon dioxide to extinguish flames.  Use water spray to cool fire-exposed containers.  Water or foam may cause frothing.

Unusal or Explosive Hazards:  None

Extinguishing Media Which Must Not be Used:  Not determined

Special Protective Equipment for Firefighters:  Wear full protective clothing and positive pressure breathing apparatus.

Page: 2

SECTION VI – ACCIDENTAL RELEAST MEASURES (Transportation Spills:  CEMTREC (800) 424 9300

Procedures in Case of Accidental Release, Breakage or Leakage:  Ventilate area.  Avoid breathing vapor.  Wear appropriate personal protective equipment, including appropriate respiratory protection.  Contain spill if possible.  Wipe up or absorb on suitable material and shovel up.  Prevent entry into sewers and waterways.  Avoid contact with skin, eyes or clothing.

If more than 1,333,333 pounds of product is spilled, then report spill according to SARA 304 and/or CERCLA 102(a) requirements, unless product qualifies for the petroleum exemption (CERCLA Section 101(14)).

SECTION VII – HANDLING AND STORAGE

Precautions to be taken in Handling:  Minimum feasible handling temperatures should be maintained.

Storage:  Periods of exposure to high temperatures should be minimized.  Water contamination should be avoided.

SECTION VIII – EXPOSURE CONTROLS/PESONAL PROTECTION

Protective Equipment (type)

Eye/Face Protection:  Shield recommended to prevent.

Skin Prevention:  Daily wash soap and water

Inhalation Prevention:  Aiborne concentrations should be kept to lowest levels possible.  If vapor, mist or dust is generated and the occupational exposure limit of the is exceeded, use appropriate approved air purifying of air supplied respirator after respirators should always be worn when airborne concentrations of the contaminant or oxygen content is unknown.

Ventilation:  Adequate to meet component occupational exposure limits (see Section 2).

Exposure Limit for Total Product:  None established for product, refer to Section 2 for component exposure limits.

SECTION IX – PHYSICAL AND CHEMICAL PROPERTIES

Appearance:  Dyed red, oily liquid

Odor:  Mild odor

Boiling Point (degrees F):  Not determined

Melting/Freezing point (degrees F):  Not applicable

Specific Gravity (water = 1):  .8596

pH of undiluted product:  Not applicable

Vapor Pressure:  Not determined

Viscosity:  > 37.6 cSt at 40.0 C

VOC Content:  38 g/L

Vapor Density (air=1):  Not determined

Solubility in Water (%):  Not determined

Other:  None

SECTION X – STABILITY AND REACTIVITY

This Material Reacts Violently With:  (If others is checked below, see comments for details)

Air	Water	Heat	Strong Oxidizers	Others	None of These

X

Comments:  None

Products evolved when subjected to heat or combustion:  Toxic levels of carbon monoxide, carbon dioxide, irritating aldehydes and ketones

Hazardous Polymerizations:  DO NOT OCCUR

SECTION XI – TOXICOLOGICAL INFORMATION

Toxicological Information (Animal toxicity data)

Median Lethal Dose  Oral:  LD50 believed to be > 5.00 g/kg (rat) practically non-toxic

Inhalation:  Not determined

Dermal:  LD50 believes to be > 2.00 g/kg (rabbit) practically non-toxic  Irritation index, estimation of irritation (Species)

Skin:  slightly irritation  (Draize) believed to be <15.00/110 (rabbit) no appreciable effects

Sensitization:  Not determined

Other:  Some low viscosity naphthenic oils have caused skin irritation and skin tumors in laboratory animals when repeated applied and left in place between applications.  The significance of this effect to humans has not been established.

SECTION XII – DISPOSAL CONSIDERATIONS

Waste Disposal Methods

This product has been evaluated for RCRA characteristics and does not meet the criteria of a hazardous waste if discarded in its purchased form.  Under RCRA, it is the responsibility of the user of the product to be determine at the time of the disposal, whether the product meets RCRA criteria for hazardous waste.  This is because product uses, transformations, mixtures, processes, etc. may render the resulting materials hazardous.

Remarks:  None

SECTION XIII – TRANSPORT INFORMATION

Transportation DOT:  Proper Shipping Name:  Not regulated

IMDG:  Proper Shipping Name: Not regulated

ICAO:  Proper Shipping Name:  Not regulated

TDG:  Proper Shipping Name:  Not regulated

SECTION XIV – REGULATORY INFORMATION

Federal Regulations:  SARA Title III:  Section 302/304 Extremely Hazardous Substances

Seq. Chemical Name

01	Hydrochloric acid		CAS Number  7647-01-0		Range in %  0.005067

Section 302/304 Extremely Hazardous Substances (cont)

Seq.	 TPQ	 RQ

01	500	5000

Section 311 Hazardous Categorization:

Acute	Chronic	Fire 	Pressure		Reactive		N/A

X

Section 313 Toxic Chemical

Chemical Name

Seq.	Chemical Name				CAS Number			Range in %

01+	Xylene					1330-20-7				0.0075

03+	Hydrochloric acid				7647-01-0				0.005067

04+	Propenoic acid, ethyl ester			140-88-5				0.005067

05+	Ethylbenzene				100-41-4				0.001675

CERCLA/DOT Hazardous Substances (Sequence Numbers and RQ’s):

Seq.	RQ

01+	100

02+	5000

03+	5000

04+	1000

05+	1000

TSCA Inventory Status:

This product, or its components are listed on or are exempt from the Toxic Substance Control Act (TSCA) Chemical Substance Inventory

Other:

A risk assessment has been performed on this product which indicates that no California Proposition 65 warnings are required.

State Regualtions:

California Proposition 65:

The following detectable components of this product are substances, or belong to classes of substances known to the State of California to cause cancer and/or reproductive toxicity.

Chemical Name					CAS Number

Propenoic Acid, ethyl ester				140-88-5

International Regulations:

WEMIS Classification:  Not Controlled

Canada Inventory Status:  This product, or its components, are listed on or are exempt from the Canadian Domestic Substantance List (DSL)

EINECS Inventory Status:  Not determined

Australia Inventory Status:  Not determined

Japan Inventory Status:  Not determined

SECTION XV – ENVINONMENTAL INFORMATION

Aquatic Toxicity:  Not determined

Mobility:  Not determined

Persistence and Biodegradability:  Not determined

Potential to Bioaccumulate:  Not evaluated

Remarks:  Not evaluated

SECTION XVI – OTHER INFORMATION

None

THE INFORMATION CONTAINED IN THIS DATA SHEET IS BASED ON THE DATA AVAILABLE TO US AT THIS TIME, AND IS BELIEVED TO BE ACCURATE BASED UPON THAT DATA.  IT IS PROVIDED INDEPENDENTLY OF ANY SALE OF THE PRODUCT, FOR PUPOSE OF HAZARD COMMUNICATION.  IT IS NOT INTENDED TO CONSTITUTE PRODUCT PERFORMANCE INFORMATION, AND NO EXPRESS OR IMPLIED WARRANTY OF ANY KIND IS MADE WITH RESPECT TO THE PRODUCT, UNDERLYING DATA OR THE INFORMATION CONTAINED WEREIN.  YOU ARE URGED TO OBTAIN DATA SHEETS FOR ALL PRODUCTS YOU BUY, PROCESS, USE OR DISTRIBUTE, AND ARE ENCOURAGED TO ADVISE THOSE WHO MAY USE THE INFORMATION CONTAINED HERIN.

TO DETERMINE THE AFFLICABILITY OR EFFECT OF ANY LAW OR REGULATION WITH RESPECT TO THE PRODUCT, YOU SHOULD CONSULT WITH YOUR LEGAL ADVISOR OR THE APPROPRIATE GOVERNMENT AGENCY.  WE WILL NOT PROVIDE ADVICE ON SUCH MATTERS, OR BE RESPONSIBLE FOR ANY INJURY FORM THE USE OF THE PRODUTCT DESCRIBED HEREIN.  THE UNDERLYING DATA, AND THE INFORMATION PROVIDED HEREIN AS A RESULT OF THAT DATA IS THE PROPERTY OF EQUIVA SERVICES, LLC AND IS NOT TO BE THE SUBJECT OF SALE OR EXHANGE WITHOUT THE EXPRESS WRITTEN CONSENT OF EQUIVA SERVICES, LLC.

Date:  1999-04-22			New  X	Revised, Supersedes:  1999-01-26

Inquiries regarding MSDS should be directed to:

Equiva Services, LLC

Manager Product Stewardship

P.O. Box 674414

Houston, TX  77267-4414

SECTION XVII – PRODUCT LABEL

Label Date:  1999-01-04

READ AND UNDERSTAND SAFETY DATA SHEET BEFORE HANDLING OR DISPOSING OF PRODUCT.  THIS LABEL COMPLIES WITH THE REQUIREMENTS OF THE OSHA HAZARD COMMUNICATION STANDARD (29 CFR 1910.1200) FOR USE IN THE WORKPLACE.  THIS LABEL IS NOT INTENDED TO BE USED WITH PACKAGING INTENDED FOR SALE TO CONSUMERS AND MAY NOT CONFORM WITH THE REQUIREMENTS OF THE CONSUMER PRODUCT SAFETY ACT OR OTHER RELATED REGUALATORY REQUIRMENTS.

0186=54 ATF MERCON/DEXROM III

WARNING STATEMENT

NONE CONSIDERED NECESSARY

PRECAUTIONARY MEASURES

- Avoid prolonged breathing of vapor, mist, or gas.

- Workers should wash exposed skin several times daily with soap and water

FIRST AID

Eye Contact:  Flush eyes with plenty of water for several minutes.  Get medical attention if eye irritation persists.

Skin Contact:  Wash skin with plenty of soap and water for several minutes.  Get medical attention if skin irritation develops or persists.

Ingestion:  If more than several mouthfuls of this material are swallowed, give two glasses of water (16 oz.).  Gen medical attention.

Inhalation:  If irritation, headache, nausea, or drowsiness occurs, remove to fresh air.  Get medical attention if breathing becomes difficult or repiratory irritation persists.

Note to Physician:  None

FIRE
_______________________________________

In case of fire, use water spray, dry chemical, foam or carbon dioxide.  Water may cause frothing.  Use water spray to cool fire-exposed containers.  If mor than 1,333,333 pounds of product is spilled, the report spill according to SARA 304 and/or CERCLA 102 (a) requirements, unless product qualifies for the petroleum exemption (CERCLA Section 101(14)).

Chemical Name					CAS Number			Range in %

Hydrotreated heavy paraffinic petroleum distillates	64742-54-7			55.00-79.99

* Hydrotreated light naphthenic petroleum distillates	64742-53-6			1.00 2.99

- Polymethacrylate					50667-55-5			1.00-2.99

- Boronaed substituted succinimide							1.00-2.99

PRODUCT IS NON-HAZARDOUS (ACCORDING TO OSHA (1910.12))

- COMPONENT IS HAZARDOUS ACCORDING TO OSHA.

COMPONENT, BY DEFINITION, IS CONSIDERED HAZARDOUS ACCORDING TO OSHA.  BECAUSE IT CARRIES THE PERMISSIBLE EXPOSURE LIMITE (PEL) FOR MINERAL OIL MIST.

Pennsylvania Special Hazardous Substance(s)			CAS Number		Range in %

None

EMIS							NFPA

Health:  1		Reactivity:  0				Health:  1			Reactivity:  0

Flammability:  1	Special:  - 					Flammability:  1		Special:  -

Transportation

DOT:

Proper Shipping Name:  Not regulated

CAUTION:  Misuse of empty containers can be hazardous.  Empty containers can be hazardous if used to store toxic, flammable, or reactive materials.  Cutting or welding of empty containers might cause fire, explosion or toxic flumes from residues.  Do not pressurize or expose to open flame or heat.  Keep container closed and drum bungs in place.

Name and Address:

Equilon Enterprises, LLC

P.O. Box 674414

Houston, TX  77267-4414

TRANSPORTATION EMERGENCY:	(877) 276-7283

CEHMTREC:			(800) 424-9300

HEALTH EMERGENCY:		(877) 276-7283