Superior Waier-Jreamg Swvict

Material Safety Data Sheet

Anderson Chemical Company, Inc.	date prepared:	3/28/1939

P.O. Box 4507	MACON, GA   31208	REVISION DATE:	11/9/2001

Ph : (478) 803-6100	Fax : (478) 803-6170	ISSUE DATE:	7/1/2004

CHEMTREC:	(800)424-9300	PRODUCT NUMBER:	4010

SECTION I - IDENTIFICATION
PRODUCT NAME :	ANCOCIDE 4010

CHEMICAL NAME:	Aqueous Solution:  Orthobenzylparachlorophenol; Sodium Hydroxide

CHEMICAL FAMILY:         Biocide
FORMULA:	Cl3HnClO;NaOH

SECTION II - HAZARDOUS INGREDIENTS
MATERIAL:	CAS NUMBER     HAZARD DATA :

Orthobenzylparachlorophenol Sodium Hydroxide

TLV not established LD50 = 5.0 gm/kg (rat) TLV = 2 mg/m3 (ceiling limit) LD50 = 240 mg/kg (rat)

120321

1310732

This product contains no known or suspected carcinogens.

SECTION III - PHYSICAL DATA

pH:	13.0-14.0	VAPOR PRESSURE:	N/A

SPECIFIC GRAVITY:	1.08-1.12	APPEARANCE:	Brown Liquid

BOILING POINT:	212°F	VAPOR DENSITY(AIR=1):	N/A

SOLUBILITY IN WATER:       Complete	ODOR:	Weak organic odor

EVAPORATION RATE (_=1):	N/A

VOLATILITY INCLUDING WATER :         70 - 75% MELTING POINT

SECTION IV - FIRE AND EXPLOSION DATA FLASH POINT : NONE

FLAMMABLE LIMITS AIR (% by Volume) : NA AUTOIGNITION TEMPERATURE : Non-Flammable EXTINGUISHING MEDIA: Dry Chemical, Water Fog, Foam, CO2

FIRE FIGHTING PROCEDURES:  Firefighters should wear full protective clothing and use self contained breathing apparatus.

UNUSUAL FIRE AND EXPLOSION HAZARDS : During a fire, irritating and/or toxic gases and aerosols from the decomposition/combustion products may be present.

SECTION V - REACTIVITY DATA STABILITY : Stable CONDITIONS TO AVOID : NA HAZARDOUS POLYMERIZATION : Does Not Occur INCOMPATIBILITY: Oxidizing Materials

HAZARDOUS DECOMPOSITION PRODUCTS : Thermal decomposition may yield HCI fumes and oxides of Carbon (COx).

SECTION VI - HEALTH DATA THRESHOLD LIMIT VALUE : No TLV established MEDICAL CONDITION AGGRAVATED :

Page 1 of 2

Material Safety Data Sheet

INHALATION : Inhalation may cause irritation of the upper respiratory tract and mucous membranes. INGESTION : Ingestion may cause gastrointestinal irritation and burns to the mouth and throat. EYE CONTACT : Corrosive to eye.  May cause severe eye irritation. SKIN CONTACT : Contact with skin may cause severe irritation and possibly burns and blistering.

SECTION VII FIRST AID

INHALATION:  Remove subject to fresh air.  Seek medical attention.

INGESTION: Drink promptly a large quantity of water. Do not induce vomiting. Avoid alcohol. Get medical attention. EYE CONTACT:  Immediately flush eyes with a large amount of water for 15 minutes. Washing eyes within one minute is essential to achieve maximum effectiveness.  Seek medical attention immediately.

SKIN CONTACT:  Immediately flush contacted area with a large amount of water.  Remove contaminated clothing and wash before reuse.

SECTION VIII EMPLOYEE PROTECTION VENTILATION REQUIREMENTS : Local Exhaust - Preferred RESPIRATORY PROTECTION : Not required under normal handling conditions.

PROTECTIVE CLOTHING : Impermeable boots and coveralls or apron.  Emergency eyewash and shower in work area.   Wear face shield or splash goggles and impermeable gloves.   Avoid contact with skin, eyes or clothing. Harmful if swallowed. Do not breath vapors. Do not get in eyes, on skin, or on clothing.

SECTION IX - SPILL AND DISPOSAL DATA

SPILL:  Contain spill.  Collect spilled material with an inert absorbent such as sand, sawdust or vermiculite. Scoop up and place in an appropriately marked container for proper disposal. Wear recommended protective clothing and equipment

WASTE DISPOSAL : Dispose of according to Federal or approved state procedures under Subtitle C of the Resource Conservation and Recovery Act.

SECTION X - TRANSPORTATION DATA

PROPER SHIPPING NAME : SODIUM HYDROXIDE SOLUTION, UN 1824 HAZARD LABEL : CORROSIVE DOT LABEL REQUIRED PACKING GROUP : II

SECTION XI - OTHER REGULATORY INFORMATION HMIS HEALTH : 3 HMISFLAMMABILITY: 0 HMIS REACTIVITY: 1 HMIS PERSONAL PROTECTION : D

SECTION XII - ADDITIONAL PRECAUTIONARY INFORMATION

Do not contaminate water, food, or feed by storage or disposal. Open dumping is prohibited.   Protect containers from extreme temperature conditions. MSDS 4010 PREPARED BY : Kiran B. Jain Ph.D

Page 2 of 2