FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

MSDS #:    192.00

Revision Date:   March 15, 2001

Section 1 — Chemical Product and Company Identification

Calcium Metal

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition. Information on Ingredients

Calcium

CAS#: 7440-70-2

Section 3 — Hazards Identification

Silver-white; soft metal.

Corrosive solid. Avoid body tissue contact. Violent reaction with water may evolve explosive hydrogen

gas. Flammable solid.

flinn at-a-glance

Health-1 Flammability-3 Reactivity-2 Exposure-1 Storage-2

0 is low hazard, 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give large quantities of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Flammable solid.	NFPA code

Violent reaction with water. Produces hydrogen.	H-l

Fire Fighting Instructions: Use Class D, Met-L-X, or dry sand as a-fire extinguisher. Avoid water	F-l

contact, violent reaction with water. Firefighters should wear PPE and SCBA with full facepiece	R-2

operating in positive mode.	No Water

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Remove all ignition sources and water . Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #1. Store with metals and metal hydrides.

Protect from air. Moisture sensitive material. Store in a Flinn Chem-Saf bag and then inside a Finn Saf-Stor can.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron.

FLINM SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Material Safety Data Sheet (MSDS)

Calcium Metal

MSDS#:     192.00

Revision Date:   March 15, 2001

Section 9 — Physical and Chemical Properties

Silver-white, soft metal.

Solubility: Alkaline earth element. Soluble in acid.

Decomposes in water to liberated hydrogen.

Formula: Ca

Formula Weight: 40.08

Specific Gravity: 1.55 Melting Point: 842 C

Section 10 — Stability and Reactivity

Highly reactive with water and oxidizing agents. Contact with water will evolve extremely flammable hydrogen gas. Shelf life: Good, but only if stored under airtight conditions.

Section 11 —Toxicoloqical Information

Acute effects: Corrosive Chronic effects: N.A. Target organs: N.A.

ORL-RAT LD50: N.A. IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

N.A. = Not available, not all health aspects of this substance have been folly investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #3 is one option.

Section 14 — Transport Information

Shipping Name: Calcium

Hazard Class: 4.3, Dangerous when wet

UN Number: UN1401

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (231-179-5), RCRA code D001, D003.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Flinn Chemical Packaging Prevents Accidents

flinn@flinnsci.com   www.flinnsci.com

P.O. Box 219   BatavialL 60510

(800) 452-1261 Fax (866) 452-1436