MATERIAL SAFETY DATA SHEE

WLC3646R WLC3649V WLC3646T WLC3652T WLC3646V WLC3655T

WLC3649T November 11, 1996

SARGeNT-WGLCH

V W R    5 C I £ N T I f= I C

SECTION I

Product

Chemical Synonyms

Formula

Unit Size

up to 2.5 Kg.

C.A.S. No.

SECTION II

911 Commerce Court

Buffalo Grove, IL 60089        MSDS No

1-800-727-4368	Effective Date

24 HOUR EMERGENCY ASSISTANCE

NAME

CUPRIC SULFATE, 5-HYDRATE

CHEMTREC 800-424-9300

Day 716-226-6177

Copper (II) Sulfate, pentahydrate (1:1:5)

CuSOA-5H2O

NFPA HAZARD RATING

HMIS

HIQH        EXTREME

3	4

: LEAST      SUI3HT    MODERATE
0   ;	1	2

7758-99-8

INGREDIENTS OF MIXTURES

HEALTH HAZARD DATA

SECTION V

Threshold Limited Value |

(Air) As mppe, maa (dust).

Copper (fume) TLV 0.1 mg/m3.

Effects Of OvereXDOSUre I       ingestion: Although a strong emetic, ingestion can cause copper - c - '       poisoning and/or death. SYMPTOMS: Nausea, vomiting, diarrhea, gastnc

pain, and hemorrhagic gastritis. EYES: Corrosive; causes conjunctivitis, swelling of the eyelids, ulceration and burn of the cornea. SKIN: Causes irritation. May cause allergic skin reaction. INHALATION: Causes upper respiratory irritation and congestion of the nasal and mucous membranes.

INHALATION; Remove to fresh air. If breathing has stopped, give mouth-to-mouth. If breathing is difficult, give oxygen. Get medical attention. EYES: Flush thoroughly with water for at least 15 minutes, lifting upper and lower eyelids occasioanlly. Get medical attention. SKIN: Flush with water, then wash with mild soap and water. INGESTION: If swallowed, if conscious, give one or two glasses of water to drink, INDUCE VOMITING and call physician. Never give anything by mouth to an unconscious person.

Emergency and First Aid Procedures

Principal Component(s)

Cupric Sulfate, pentahydrate (1:1:5)

>99 %

DANGER! CORROSIVE. HARMFUL OR FATAL IF SWALLOWED.

IRRITANT TO SKIN, EYES AND MUCOUS MEMBRANES.

PHYSICAL DATA

SECTION III

Specific Gravity (HO = 1)

Melting Point (°F)

653°C (1207°F)

Boiling Point (°F)

Percent Volatile by Volume (%)

N/A

Evaporation Rate

Vapor Pressure (mm Hg)

N/A

Vapor Density (Air=1)

N/A

Solubility in Water

Appreciable (>10%).

Blue crystals or fine blue powder; no odor.

Appearance & Odor

TLV Units

See Section V.

2.28

N/A

N/A

| SECTION VI |  |  | REACTIVITY DATA |  |  |
| --- | --- | --- | --- | --- | --- |
| Stability | Unstable |  |  |  |  |
|  | Stable |  |  |  |  |
| Incompatibility (Materials to Avoid) |  |  | Incompatible with acetylene. Copper salts may react to form explosive acetylides. Copper sulfate can cause ignition upon contact with hydroxylamine due to the release of heat. Reducing agents react vigorously with copper salts. |  |  |
| Hazardous Decomposition Products |  |  |  | Combustion may produce irritating copper fumes and toxic gaseous oxides (sulfur oxides). |  |
| Hazardous | Polymerization |  |  | Conditions to Avoid ¦ |  |
| May Occur |  | Will Not Occur |  |  | Not applicable. |
|  |  | X |  |  |  |
| SECTION VII |  |  | SPILL OR LEAK PROCEDURES |  |  |
| Steps to be taken in material is released |  |  | case or spilled |  | Ventilate the area. Sprinkle lime or soda ash on spill to torm insoluble copper salt. Sweep up and place in a suitable container |

material from entering sewers and waterways.

SECTION IV

s only.

Non-flammable.

Flash Point (Method Used)

FIRE AND EXPLOSION HAZARD DATA

I Flammable Limits in Air [ % by Volume

N/A

Upper

Waste Disposal Method      Discharge, treatment, or disposal may be subject to Federal, State or Local laws.
	—	1    These disposal guidelines are intended for the disposal of catalog-size quantities

Dispose of in accordance with federal, state and local regulations.

Use any media suitable for extinguishing supporting fire.

Extinguisher Media

SPECIAL FIREFIGHTING PROCEDURES

In fire conditions, firefighters should wear protective clothing and a NIOSH/MSHA-approved self-contained breathing apparatus.Cupric Sulfate will not bum, nor will it support combustion. Care should be used to keep material out of streams or other water bodies.

SPECIAL PROTECTION INFORMATION

SECTION VIII

None should be required in normal laboratory use. It dusty conditions prevail, work in a ventilation hood or wear a NIOSH/MSHA-approved dust mask.

Respiration Protection (Specify Type)

| Ventilation | Local Exhaust |  | Recommended. |  | Special | No. |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mechanical (General) |  | Recommended. |  | Other | No. |  |
| Protective Gloves |  | Rubber. |  | Eye Protection |  |  | Chemical safety goggles. |

Safety glasses, smock, apron, vented hood, proper gloves, and eye wash station.

Other Protective Equipment

SECTION IX

SPECIAL PRECAUTIONS

UNUSUAL FIRE AND EXPLOSION HAZARDS

Precautions to be Taken in Handling & Storing

Keep container tightly closed when not in use.

Store in a cool, dry place. Wash thoroughly after handling.

Fire or excessive heat may produce sulfur oxides.

Other Precautions   [Readlat*'onconla'nwDa'ortu8'nS. Do not wear contact lenses when working with chemicals.

Avoid contact with skin, eyes and clothing. Avoid breathing dust. Wash contaminated clothing before reuse.

For laboratory use only. Not for drug, food or household use. Keep out of reach of children.

TN'ON-HbUULATED.

D.O.T.

Approved by U.S. Department of Labor "essentially similar' to form OSHA-20

Revision No'5   [Date

TApproved

Michael Raszeja

11/11/96

1~ Chemical Safety Coordinator

The information contained herein is furnished without warranty of any kind. Employers should use this information only as a supplement to other information gathered by them and must make independent determinations of suitability and completeness of information from all sources to assure proper use of these materials and the safety and health of employees.  * Hazardous Materials Industrial Standards, Printed on recycled paper.