MATERIAL SAFETY DATA SHEET: ANCOOL 3700

Section I - General Information

(000000-000000-   -48TZ)

Date of Issue:

10/30/2006 12:00:00 AM

Chemical Name & Synonyms:

N/A

Chemical Family:

ALKALINE MIXTURE

Manufacturer Name:

CHEM-AQUA, INC

Manufacturer Address:

BOX 152170 IRVING, TEXAS 75015

Prepared By:

k Jain/Chemist

Product Code Number:

48TZ

Supercedes:

2/13/2006 12:00:00 AM

Trade Name & Synonyms:

ANCOOL 3700

Formula is a mixture: [V]

Emergency Phone Number:

800-424-9300

Section II - Hazardous Ingredients

THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS

Chemical Name (Ingredients)

SODIUM HYDROXIDE

SODIUM SILICATE

SODIUM BORATE DECAHYDRATE

SODIUM NITRITE

$ CEILING LIMIT

Hazard

CORROSIVE IRRITANT IRRITANT TOX/IRR

TLV

N/E N/E 1 5MG/M3 1 N/E1

PEL

2 MG/M3 $2 N/E 2

10MG/M32 N/E 2

STEL

2 MG/M3 $1

N/E

N/E

N/E

CAS#

1310-73-2 1344-09-8 1303-96-4 7632-00-0

Section III - Physical Data

Boiling Point (°F):	212°

Vapor Pressure (mm Hg):	12.83

Vapor Density (Air=1):	0.6

pH@100%:	13.5

% Volatile by Volume:	83

H20 Solubility:	COMPLETE

Specific Gravity (H20=1):	1.25

Color:	PALE YELLOW

Odor:	ODORLESS

Clarity:	TRANSPARENT

Evaporation Rate (BuAc=1):	0.46

Viscosity:	NON-VISCOUS

Section IV - Fire and Explosion Hazard

Method Used: P.M.C.C.

Flash Point: >200°F Flammable Limits: HYDROGEN GAS LEL: 4%

UEL: 75% Aerosol Level (NFPA SOB): N/A

Extinguishing Media:

[V] Foam	[V] Alcohol Foam         [V] CO2

[V] Dry Chemical         [V] Water Spray	[ ] Other

NFPA 704 Hazard Rating:

4-Extreme 3-High 2-Moderate 1-Slight 0-lnsignificant

Health: 3 Flammability: 0 Instability: 2 Special:

Special Fire Fighting Procedures:

FIREFIGHTERS SHOULD WEAR A SELF-CONTAINED BREATHING APPARATUS AND FULL PROTECTIVE GEAR. EXTINGUISHING MEDIA SHOULD BE CHOSEN BASED ON THE NATURE OF THE SURROUNDING FIRE. COOL FIRE-EXPOSED CONTAINERS WITH WATER SPRAY TO PREVENT BURSTING.

1 of 5

MATERIAL SAFETY DATA SHEET: ANCOOL 3700

Unusual Fire and Explosion Hazards:

PROLONGED CONTACT WITH REACTIVE METALS, SUCH AS ALUMINUM, COPPER, BRASS, BRONZE, CHROMIUM, MAGNESIUM, TIN, ZINC, AND ALLOYS, CAN CAUSE THE FORMATION OF FLAMMABLE HYDROGEN GAS WHICH CAN FORM AN EXPLOSIVE MIXTURE WITH AIR. THE USE OF WATER SPRAY (FOG) WHILE EFFECTIVE, MAY CAUSE FROTHING AND FOAMING. NEVER USE A WATER JET AS THIS WILL JUST SPREAD THE FIRE. USE CARE AS SPILLS MAY BE SLIPPERY.

Section V - Health and Hazard Data

Threshold Limit Value:

NOT ESTABLISHED.

Effects of Overexposure:

Acute: (Short Term Exposure)

EYE CONTACT: CORROSIVE. CAUSES BURNS, CORNEAL DAMAGE, AND POSSIBLE BLINDNESS. SKIN CONTACT: CORROSIVE. CAUSES

LOSS MAY OCCUR AT BURN SITE. INHALATION: CAUSES BURNS TO THE RESPIRATORY TRACT, NOSE, MOUTH, AND THROAT WITH DISCOMFORT, NASAL DISCHARGE, SNEEZING, COUGHING, RAPID HEARTBEAT, AND CHEST PAIN. INHALATION OF MIST MAY CAUSE CHEMICAL PNEUMONITIS WHICH CAN CAUSE DAMAGE AND MAY BE FATAL. INHALATION OF LARGE AMOUNTS MAY CAUSE SYSTEMIC EFFECTS DUE TO ABSORPTION OF NITRITE BY LUNG TISSUE CAUSING EFFECTS SIMILAR TO INGESTION. INGESTION: CORROSIVE. CAUSES BURNS TO THE MOUTH, THROAT, ESOPHAGUS, AND STOMACH WITH NAUSEA AND PAIN. SYMPTOMS MAY INCLUDE VOMITING OF BLOOD. BLOOD LOSS THROUGH DAMAGED TISSUE CAN LEAD TO LOW BLOOD PRESSURE AND SHOCK, AND MAY BE FATAL. OVEREXPOSURE MAY CAUSE METHEMOGLOBINEMIA (REDUCED OXYGEN CARRYING CAPACITY OF THE BLOOD) WITH CYANOSIS (BLUISH DISCOLORATION OF THE SKIN) PROGRESSING TO DIZZINESS, INCOORDINATION, LOSS OF CONSCIOUSNESS, SHORTNESS OF BREATH, AND INCREASED PULSE RATE.

Chronic: (Long Term Exposure)

OVEREXPOSURE MAY CAUSE METHEMOGLOBINEMIA (REDUCED OXYGEN CARRYING CAPACITY OF THE BLOOD) WITH CYANOSIS (BLUISH DISCOLORATION OF THE SKIN) PROGRESSING TO DIZZINESS, INCOORDINATION, LOSS OF CONSCIOUSNESS, SHORTNESS OF BREATH, AND INCREASED PULSE RATE. LONG-TERM INGESTION PRODUCES UNSPECIFIED PATHOLOGICAL CHANGES IN THE ADRENALS, BRAIN, HEART, KIDNEYS, LIVER, LUNGS, AND SPLEEN. CHRONIC INHALATION MAY CAUSE INFLAMMATORY CHANGES IN THE NOSE, SINUSES, AND BRONCHIAL REGIONS. CHRONIC EXPOSURE MAY RESULT IN BORISM SUCH AS DRY SKIN, ERUPTIONS, AND GASTROINTESTINAL DISTURBANCES. MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE ARE PRE-EXISTING RESPIRATORY AND SKIN CONDITIONS SUCH AS ASTHMA, EMPHYSEMA, AND DERMATITIS; PRE-EXISTING LIVER, BLOOD, AND KIDNEY DISEASES. TARGET ORGANS: CENTRAL NERVOUS SYSTEM, BLOOD FORMING ORGANS, HEART, LIVER, LUNGS, SPLEEN, BLOOD, AND KIDNEYS. THE PRIMARY ROUTES OF EXPOSURE ARE SKIN AND EYE CONTACT.

r Primary Routes of Entry

1 [V] Inhalation	[V] Ingestion	[V] Absorption

Emergency First Aid Procedures:

Inhalation:

REMOVE FROM THE AREA TO FRESH AIR. IF NOT BREATHING, CLEAR THE AIRWAY AND START MOUTH TO MOUTH ARTIFICIAL RESPIRATION. GET IMMEDIATE MEDICAL ATTENTION.

Eye Contact:

IMMEDIATELY RINSE THE EYES WITH WATER. REMOVE ANY CONTACT LENSES AND CONTINUE FLUSHING FOR AT LEAST 15 MINUTES.

MEDICAL ATTENTION.

Skin Contact:

IMMEDIATELY REMOVE CONTAMINATED CLOTHING AND SHOES. FLUSH AFFECTED AREAS WITH LARGE AMOUNTS OF WATER FOR 20 TO 30 MINUTES. GET IMMEDIATE MEDICAL ATTENTION. DISCARD CLOTHING AND SHOES.

Ingestion:

GIVE 3 TO 4 GLASSES OF WATER, BUT DO NOT INDUCE VOMITING. IF VOMITING OCCURS, GIVE FLUIDS AGAIN. GET IMMEDIATE MEDICAL ATTENTION. DO NOT GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS OR CONVULSING PERSON.

Notes to Physician:

PROBABLE MUCOSAL DAMAGE MAY CONTRAINDICATE THE USE OF GASTRIC LAVAGE. MEASURES AGAINST CIRCULATORY SHOCK, RESPIRATORY DEPRESSION, AND CONVULSIONS MAY BE NEEDED. ABSORPTION OF THIS PRODUCT MAY LEAD TO THE FORMATION OF METHEMOGLOBIN WHICH, IN SUFFICIENT CONCENTRATION, CAUSES CYANOSIS. SINCE REVERSION OF METHEMOGLOBIN TO HEMOGLOBIN OCCURS SPONTANEOUSLY AFTER TERMINATION OF EXPOSURE, MODERATE DEGREES OF CYANOSIS NEED TO BE TREATED ONLY BY SUPPORTIVE MEASURES SUCH AS BED REST AND OXYGEN INHALATION. THOROUGH CLEANSING OF THE ENTIRE CONTAMINATED AREA OF THE BODY INCLUDING SCALP AND NAILS IS OF UTMOST IMPORTANCE. IF CYANOSIS IS SEVERE, INTRAVENOUS INJECTION OF METHYLENE BLUE, 1 MG/KG OF BODY WEIGHT, MAY BE OF VALUE. CYANOCOBALAMIN (VITAMIN B-12), 1 MG INTRAMUSCULARLY, WILL SPEED RECOVERY. INTRAVENOUS FLUIDS AND BLOOD TRANSFUSIONS MAY BE INDICATED IN VERY SEVERE EXPOSURES.

Section VI - Toxicity Information

2 of 5

MATERIAL SAFETY DATA SHEET: ANCOOL 3700

^

Product Contains Chemicals Listed as Carcinogen or Potential Carcinogen By:

[ ] IARC	[ ] NTP	[ ] OSHA	[ ] ACGIH	[ ] Other

VOC CONTENT: 0% BY WEIGHT, 0% BY VOLUME, 0 G/L

SODIUM HYDROXIDE ORL-MSE LD50: 40 MG/KG 3. ORL-RBT LDLO: 500 MG/KG 3. SKN-RBTTCLO: 25 PPH 3. SKN-RBT SOT: 500 MG/24H SEVERE 3. EYE-RBT SDT: 1 MG/24H SEVERE 3.

SODIUM SILICATE

ORL-RAT LD50: 1500-3200 MG/KG (100% SOLIDS BASIS) 3.

ORL-RAT: 200,600,1800 PPM/3 MONTHS; CHANGES IN THE BLOOD CHEMISTRY 3.

ORL-RAT: 600,1200 PPM; DECREASED # OF BIRTHS AND SURVIVAL TO WEANING 3.

ORL-DOG: 2.4 G/KG/DAY/4 WEEKS; KIDNEY EFFECTS 3.

EYE: CORNEAL AND IRIDAL INVOLVEMENT, SEVERE CONJUNCTIVAL IRRITATION, OCULAR IRRITATION STILL PRESENT 7 DAYS AFTER TREATMENT. IT YIELDED A SCORE OF 19.7 AFTER 1 DAY AND 4.0 AFTER 14 DAYS. 3.

PRIMARY SKIN IRRITATION SCORE: 3 TO ABRADED SKIN, 0 TO INTACT SKIN. EXPERIENCE HAS SHOWN PRODUCT WHEN GETTING ON COLLARS AND CUFFS CAUSES

IRRITATION. 3.

FREQUENT INGESTION OVER EXTENDED PERIODS OF TIME OF GRAM QUANTITIES OF SILICATES IS ASSOCIATED WITH THE FORMATION OF KIDNEY STONES AND OTHER SILICEOUS URINARY CALCULI IN HUMANS. 3.

SODIUM BORATE DECAHYDRATE IHL-RATLC50:>2MG/L4. ORL-MAN LDLo: 709 MG/KG 4. ORL-RAT LD50: 2660 MG/KG 4. SKN-RBT LD50: >10,000 MG/KG 4. SKN-RBT: NON-IRRITANT 3. EYE-RBT: IRRITANTS.

HIGH DOSE ANIMAL INGESTION STUDIES ON RATS, MICE AND DOGS DEMONSTRATED REPRODUCTIVE AND DEVELOPMENTAL EFFECTS. 3.

SODIUM NITRITE ORL-HMN TDLO: 14 MG/KG 4. ORL-HMN LDLO: 71 MG/KG 4. ORL-RAT LD50: 120 MG/KG 3. IHL-RAT LC50: 1.45 MG/L/4H 3. EYE-RBT SDT: 500 MG/24H MILD 4.

| Section VII - Reactivity Data |  |
| --- | --- |
| U-,,-,,^«..= D«l,,m=ri»,»i«.. |  |
| " Stability — • • I [V] Stable [ ] Unstable Conditions to Avoid: NONE KNOWN | [V] Will not occur [ ] May occur Conditions to Avoid: N/A |

Incompatibility (Materials to Avoid):

STRONG OXIDIZING AGENTS SUCH AS CHLORINE BLEACH AND CONCENTRATED HYDROGEN PEROXIDE. REDUCING AGENTS SUCH AS SODIUM THIOSULFATE, ACIDS, METAL HYDRIDES OR ALKALAI METALS; AMINES, AMMONIUM SALTS, CYANIDES, NITRITES, ACETALDEHYDE, ACROLEIN, CHLORINATED OR FLUORINATED HYDROCARBONS, CHLORINE TRIFLUORIDE, HYDROQUINONE, LEATHER, MALEIC ANHYDRIDE, ORGANIC HALOGEN OR NITRO COMPOUNDS, PHOSPHORUS PENTOXIDE, TETRAHYDROFURAN, WATER, WOOL, CARBON MONOXIDE GAS FORMS UPON CONTACT WITH REDUCING SUGARS, FOOD AND BEVERAGE PRODUCTS, CAN ETCH GLASS. PROLONGED CONTACT WITH REACTIVE METALS, SUCH AS ALUMINUM, COPPER, BRASS, BRONZE, CHROMIUM, MAGNESIUM, TIN, ZINC, AND ALLOYS, CAN CAUSE THE FORMATION OF FLAMMABLE HYDROGEN GAS WHICH CAN FORM AN EXPLOSIVE MIXTURE WITH AIR.

Hazardous Decomposition Products:

OXIDES OF CARBON, SODIUM, AND NITROGEN. HYDROGEN GAS; HYDROGEN CYANIDE IN A REDUCING ATMOSPHERE.

Section VIII - Spill Or Leak Procedures

Steps to be Taken if Material is Released or Spilled:

WEAR APPROPRIATE PROTECTIVE CLOTHING. USE CARE AS SPILLS MAY BE SLIPPERY. SHUT OFF SOURCE OF LEAK. DIKE AND

3 of 5

MATERIAL SAFETY DATA SHEET: ANCOOL 3700

CONTAIN SPILL. ABSORB WITH AN INERT MATERIAL AND TRANSFER ALL MATERIAL INTO A PROPERLY LABELED CONTAINER FOR DISPOSAL. PREVENT PRODUCT FROM CONTAMINATING SOIL OR FROM ENTERING SEWAGE AND DRAINAGE SYSTEMS AND BODIES OF WATER. FLUSH AREA WITH WATER.

Waste Disposal Method(s):

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL, STATE, AND LOCAL REGULATIONS.

Neutralizing Agent:

USE DILUTE ACIDS SUCH AS HYDROCHLORIC ACID OR VINEGAR. ADD CAUTIOUSLY WHILE MIXING. WEAR APPROPRIATE PROTECTIVE CLOTHING.

Section IX - Special Protection information

Required Ventilation:

MISTS. LOCAL VENTILATION IS PREFERRED, BECAUSE IT PREVENTS DISPERSION INTO WORK AREAS BY CONTROLLING IT AT ITS SOURCE.

Respiratory Protection:

RESPIRATORS SHOULD BE SELECTED BY AND USED UNDER THE DIRECTION OF A TRAINED HEALTH AND SAFETY PROFESSIONAL FOLLOWING REQUIREMENTS FOUND IN OSHA'S RESPIRATOR STANDARD (29 CFR 1910.134) AND ANSI'S STANDARD FOR RESPIRATORY PROTECTION (Z88.2-1992). FOR CONCENTRATIONS ABOVE THE TLV AND/OR PEL BUT LESS THAN 10 TIMES THESE LIMITS, A NIOSH APPROVED HALF-FACEPIECE RESPIRATOR EQUIPPED WITH APPROPRIATE CHEMICAL CARTRIDGES MAY BE USED. FOR CONCENTRATIONS GREATER THAN 10 TIMES THE TLV AND/OR PEL, CONSULT THE NIOSH RESPIRATOR DECISION LOGIC FOUND IN PUBLICATION NO. 87-116 OR ANSI Z88.2-1992.

Glove Protection:

NEOPRENE OR NITRILE RUBBER GLOVES SHOULD BE WORN. ENSURE COMPLIANCE WITH OSHA'S PERSONAL PROTECTIVE EQUIPMENT (PPE) STANDARD FOR HAND PROTECTION, 29 CFR 1910.138.

Eye Protection:

CHEMICAL GOGGLES AND A FACE SHIELD SHOULD BE WORN WHEN HANDLING. ENSURE COMPLIANCE WITH OSHA'S PERSONAL PROTECTIVE EQUIPMENT (PPE) STANDARD FOR EYE AND FACE PROTECTION, 29 CFR 1910.133.

Other Protection:

WEAR PROTECTIVE CLOTHING WHEN HANDLING. A SAFETY SHOWER AND AN EYEWASH STATION SHOULD BE AVAILABLE.

Section X - Storage and Handling Information

,- Storage Temperature	.     r- Storage Conditions

iMax:120°F	Min: 35°F	[V] Indoors	[] Outdoors	[] Heated	[] Refrigerated

Precautions to be Taken in Handling and Storing:

ALWAYS STORE MATERIAL IN ITS ORIGINAL CONTAINER. KEEP CONTAINER TIGHTLY CLOSED WHEN NOT IN USE. KEEP FROM

Other Precautions:

KEEP OUT OF REACH OF CHILDREN. READ THE ENTIRE LABEL BEFORE USING THE PRODUCT. FOLLOW THE LABEL DIRECTIONS.

Section XI - Regulatory Information

Chemical Name	CAS Number	Upper % Limit

None.

Those Ingredients listed above are subject to the reporting requirements of 313 of Title III of the Superfund Amendments and Reauthorization Act of 1986 and 40 CFR part 372.

Please call 1-800-527-9919 for additional information if you are a California customer. This MSDS is not intended for users in the state of

California.

Section XII - References

2. OSHA PEL. 3. VENDOR'S MSDS. 4. REGISTRY OF TOXIC EFFECTS OF CHEMICAL SUBSTANCES, CCINFOWeb, 2006. ALL THE

ON THE TSCA INVENTORY OR OTHERWISE EXEMPTED FROM LISTING. - IRR:IRRITANT, FLAM/FLAMM:FLAMMABLE, COMB:COMBUSTIBLE, CORR:CORROSIVE CARC:CARCINOGENIC, TOX:TOXIC, N/A:NOT APPLICABLE, N/E:NOT ESTABLISHED, COC:CLEVELAND OPEN CUP, PMCC:PENSKY-MARTIN CLOSED CUP, TCC:TAGLIABUE CLOSED CUP, LEL:LOWER EXPLOSION LIMIT, UELUPPER EXPLOSION LIMIT, NFPA:NATIONAL FIRE PROTECTION ASSOCIATION, IARC:INTERNATIONAL AGENCY FOR THE RESEARCH ON CANCER, NTP:NATIONAL

4 of 5

MATERIAL SAFETY DATA SHEET: ANCOOL 3700

TOXICOLOGY PROGRAM, OSHA:OCCUPATIONAL SAFETY & HEALTH ADMINISTRATION, ACGIH:AMERICAN CONFERENCE OF GOVERNMENTAL INDUSTRIAL HYGIENISTS, TLV:THRESHOLD LIMIT VALUE, PEUPERMISSIBLE EXPOSURE LIMIT, STEL:SHORT-TERM EXPOSURE LIMIT, MLD:MILD, MOD:MODERATE, SEV:SEVERE, MUTiMUTAGENIC, ASPHYX:ASPHYXIANT, PNOS:PARTICLES (INSOLUBLE) NOT OTHERWISE SPECIFIED, SDT:STANDARD DRAIZE TEST, ORLORAL, IHL:INHALATION, HMN:HUMAN

THE INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORMULATION. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THE DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF.

CHEM-AQUA, INC assumes no responsibility for personal injury or property damage caused by the use, storage, or disposal of the product in a manner not recommended on the product label. Users assume all risks associated with such unrecommended use, storage, or disposal of the product.

©2006 NCH Corporation All rights reserved.

5 of 5