MSDS No.:
Revision No.:
Date:
Page:

815 008

5/13/97 1 of 6

MATERIAL SAFETY DATA SHEET

HILTI, INC.:	P.O. Box 21148, Tulsa, OK 74121; Ph: 1 800 879 6000; Emergency No.: 1 800 879 4444

PRODUCT:	RM 700 EP USDA Epoxy Mortar Part A

DESCRIPTION:   Epoxy repair mortar, resin portion

For:

CABARRUS COUNTY SCHOOL DIST

P O BOX 388

CONCORD, NC 28026-0388

P.O. No. 50736

| INGREDIENTS AND EXPOSURE LIMITS INGREDIENTS: CAS NUMBER: PEL: | TLV: | STEL: |
| --- | --- | --- |
| NE | NE | NE |
| NE | NE | NE |
| NE | NE | NE |
| NA | NA | NE |

Modified Diglycidyl Ether of Bisphenol A25068-38-6
Aikyl Glycidy! Ether	68609-97-°

Epichlorohydrin-polyglycol adduct	26142-30-3

Carbon Black	01333-86-4

PEL = OSHA Permissible Exposure Limit. TLV = ACGIH Threshold Limit Value. These are 8 hour time-weighted averages unless otherwise indicated by "C" (Ceiling) or "STEL" (Short Term Exposure Limit). NA = Not Applicable. NE = None Established

PHYSICAL DATA

APPEARANCE AND ODOR: Pale amber liquid. Slight odor. BOILING POINT: Not Applicable (decomposes) VAPOR PRESSURE: Not Applicable EVAPORATION RATE: Not Determined SPECIFIC GRAVITY: 1.1

VOC CONTENT: None MELTING POINT: Not Applicable VAPOR DENSITY: Not Determined SOLUBILITY IN WATER:  Nil

pH: Not Determined

FLASH POINT: >200°F, Setaflash EXTINGUISHING MEDIA: Not Applicable

FIRE AND EXPLOSION HAZARD DATA

FLAMMABLE LIMITS:  Not Applicable

SPECIAL FIRE FIGHTING PROCEDURES:   A self-contained breathing apparatus should be worn when fighting fifco involving chemicals.

UNUSUAL FIRE AND EXPLOSION HAZARDS: Thermal decomposition and combustion products can be formed.

REACTIVITY DATA STABILITY: Stable.

INCOMPATIBILITY: Strong oxidizers, peroxides, acids.

HAZARDOUS DECOMPOSITION PRODUCTS: Thermal decomposition can yield CO and CO2. HAZARDOUS POLYMERIZATION: Will not occur.

CONDITIONS TO AVOID:  Excessive heat for prolonged periods of time.   Reactions with some curing agents can generate large amounts of heat.

HEALTH HAZARD DATA

KNOWN HAZARDS: Eye and skin irritation with possible burns. Sensitization.

CARCINOGENICITY: No ingredients are classified by IARC, NTP, or OSHA as known or potential carcinogens. PRIMARY ROUTES OF EXPOSURE:  Dermal.

SIGNS AND SYMPTOMS OF EXPOSURE: Irritatipn to skin and eyes. Can cause eye and skin burns. Can cause allergic skin reactions with susceptible individuals.

MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE:  Eye, skin, and respiratory conditions.

EMERGENCY AND FIRST AID PROCEDURES EYES: Immediately flush with plenty of water. Call a physician. SKIN: Wash with soap and water. Launder contaminated clothing before reuse. INHALATION: Move victim to fresh air. Call a physician if symptoms persist.

INGESTION: If conscious, give plenty of water to drink. Seek medical attention. Do not induce vomiting unless directed to by a physician. Never give anything by mouth to an unconscious person.

OTHER: Referral to a physician is recommended if there is any question about the seriousness of the injury/exposure.

CONTROL MEASURES AND PERSONAL PROTECTIVE EQUIPMENT VENTILATION: General (i.e. natural or mechanically induced fresh air movements). EYE PROTECTION: Wear splash-proof chemical goggles while mixing resin and hardener. SKIN PROTECTION: Gloves recommended. Cotton or impervious (neoprene or rubber).

RESPIRATORY PROTECTION: None normally required. Use a NIOSH-approved organic vapor cartridge respirator when working in areas with little or no air movement. Never enter a confined space unless wearing an appropriate air-supplied respirator, (e.g. SCBA or airline with escape pack).

PRECAUTIONS FOR SAFE HANDLING AND USE

HANDLING AND STORING PRECAUTIONS: Avoid contact with eyes, skin, or clothing. Wash thoroughly after handling. Store in a cool dry place. Store below 100°F. Keep from freezing. Keep container closed when not in use.

SPILL PROCEDURES: Remove spillage with absorbent material. Wipe away excess.

REGULATORY INFORMATION

OSHA HAZARD COMMUNICATION: This MSDS has been prepared in accordance with the federal OSHA Hazard Communication Standard..

HMIS CODES: Health 2, Flammability 1, Reactivity 0, PPE B

DOT SHIPPING NAME: Non-regulated.

TSCA INVENTORY STATUS: Chemical components listed on TSCA inventory.

SARA TITLE III, SECTION 313: This product does not contain any toxic chemicals which are subject to reporting under Section 313 of SARA Title III (40 CFR Part 372).

WASTE DISPOSAL METHODS: Consult with regulatory agencies or corporate personnel for disposal methods that comply with local, state, and federal health and environmental regulations.

HEALTH / SAFETY CONTACTS:    Steve Gerrard (x6309)

Jerry Metcalf   (x6704)

The information and recommendations contained herein are based upon data believed to be correct; however, no guarantee or warranty of any kind expressed or implied is made with respect to the information provided.

MSDS 815, Page 2 of 6