MSDS#: 9702 Status: Current

Status Date: 4/15/01 Product Name: C.P. MAR-VEL Internal MSDS#: 838

Current Company Information:

U.N.X. INCORPORATED

707 ARLINGTON BLVD.

GREENVILLE, NC USA 27858

Phone: 252-756-8616     Emergency: 800-255-3924 CHEMTEL

Current Company Short Name:

Manufacturer/Supplier: U.N.X. INCORPORATED 707 ARLINGTON BLVD. GREENVILLE, NC USA 27858 Phone: 252-756-8616     Emergency: 800-255-3924 CHEMTEL

MSDS Prepared: 4/20/95 MSDS Revised: 4/15/01 Keyword: MANUAL DETERGENT Synonym(s): POT AND PAN DETERGENT

Physical/Chemical Characteristics

Boiling Point: EQ 85   C Specific Gravity: EQ 1.05 H2O=1 Vapor Density: GT 1 Air=l

pH: BT 7 8

Solubility In Water: EXCELLENT

Appearance/Odor: YELLOW LIQUID WITH LEMON ODOR Physical State: Liquid

DOT INFORMATION:

DOT hazard Class: NON HAZARDOUS

Label: NONE

Proper Shipping Name: NON HAZARDOUS

Preparers Info.: Amendt, Evans, Kincaid

I********************************* COMPOSITION/INFORMATION ON INGREDIENTS

Component Name: WATER Percent: GT    60 CAS No.: 7732-18-5 Exposure Limits

Component Name: TS#PSRN57948000-5004P Notes: NJ TRADE SECRET NUMBER Percent: BT    15    20 Exposure Limits

Component Name: DIETHANOLAMINE

Notes: LD(50) 7460mg/kg ORAL RAT; LD(50) 620mg/kg DERMAL RABBIT

Percent: LT    1 CAS No.: 111-42-2 Exposure Limits

OSHAPEL: 15 MG/M3

ACGIH TLV: 2 MG/M3

Component Name: DODECYL BENZENE SULFONIC ACID Notes: LD(50) 2320mg/kg ORAL RAT Percent: BT    0    10 CAS No.: 25155-30-0 Exposure Limits ACGIH TLV: 10MG/M3

FIRE AND EXPLOSION DATA

SPECIAL FIRE FIGHTING PROCEDURES: USE NIOSH/MSHA APPROVED SELF CONTAINED BREATHING APPARATUS WHERE PRODUCT IS INVOLVED IN FIRE. UNUSUAL FIRE AND EXPLOSION HAZARD: NONE

REACTIVITY DATA

VERY STABLE. CONDITION TO AVOID: HEAT INCOMPATIBILITY: NOT APPLICABLE HAZARDOUS POLYMERIZATION WILL NOT OCCUR.

HEALTH HAZARD DATA

ACUTE OR CHRONIC: INHALATION, CAN IRRITATE NOSE/THROAT, AND LUNGS INGESTION, IRRITANT WILL CAUSE GASTRIC DISTRESS,VOMITING AND DIARRHEA

CARCINOGENICITY: NOT APPLICABLE

SIGNS AND SYMPTOMS OF EXPOSURE: EYE CONTACT CAN CAUSE BURNING AND EXTREME

IRRITATION.

FIRST AID: FOR EYES, FLUSH WITH WARM WATER FOR AT LEAST 15 MINUTES. CONTACT

PHYSICIAN. IF INGESTED,DRINK LARGE VOLUMES OF WATER

INDUCE VOMITING. CALL PHYSICIAN.

PRECAUTIONS FOR SAFE HANDLING AND USE

STEPS FOR SPILL OR RELEASE: FOR SMALL SPILL, IMMEDIATELY HOSE DOWN WITH COOL

WATER AND DISPOSE DOWN DRAIN. FOR LARGE SPILL.DIKE, COLLECT AND CONTACT LOCAL

AUTHORITIES ABOUT DISPOSAL.

WASTE DISPOSAL METHODS: SEE STEPS FOR SPILL OR RELEASE.

HANDLING AND STORING: STORE IN COOL DRY AREA. KEEP CONTAINER CLOSED.

CONTROL MEASURES

RESPIRATORY PROTECTION: AVOID BREATHING MIST. IF EXPOSURE LIKELY, USE FILTER

OR RESPIRATOR.

MECHANICAL VENTILATION IS ACCEPTABLE.

RUBBER OR PLASTIC GLOVES SHOULD BE WORN.

SAFETY GLASSES OR GOGGLES SHOULD BE WORN.

OTHER PROTECTIVE MEASURES: HAVE EYE BATH AND SAFETY SHOWER CLOSE BY. IF

POTENTIAL FOR SKIN CONTACT, WEAR PROTECTIVE GLOVES AND HAVE AN ACID HAND WA<

NEAR BY(ie. VINEGAR SOLUTION.).

WORK/HYGENIC PRACTICES: WASH HANDS,CHANGE OUT OF CLOTHES AS SOON AS

POSSIBLE. WASH CLOTHES. SHOWER OR BATHE AS SOON AS POSSIBLE.

OTHER INFORMATION

HMIS: HEALTH-0, FLAMMABILITY-0, REACTIVITY-0, SPECIAL-PERSONAL PROTECTION - C

UNLESS OTHERWISE NOTED, NO COMPONENTS ARE SARA TITLE 3 SECTION 313 40 CFR LISTED MATERIALS.

NOT MADE WITH VOC'S THAT COULD CAUSE DAMAGE TO THE OZONE LAYER THE INGREDIENTS OF THIS PRODUCT ARE ON THE TSCA INVENTORY

END OF REPORT.