MSDS 28-1 January 1, 2002

DuPont Performance Coatings

Material Safety Data Sheet

Nason® Clears

| SECTION 1 - Product and Company Identification | INGREDIENTS CAS # VAPOR | EXPOSURE |
| --- | --- | --- |
|  | PRESSURE | LIMITS |
| Manufacturer: E.I. DuPont de Nemours & Co. |  | 8 & 12 hour |
| DuPont Performance Coatings |  | TWA |
| Wilmington, DE, 19898 |  | A None |
|  |  | O None |
| Telephone: Product Information: (800) 441-7515 |  |  |
| Medical Emergency: (800) 441-3637 | BETA-(3-(2H-BENZOTRIAZOL-2-YL)-4-HYDROXY-5- |  |
| Transportation Emergency: (800) 424-9300 (CHEMTREC) | TERTBUTYLPHENYL)PROPIONATE |  |
|  | 104810-47-1 None | A None |
| Product: NASON® CLEARS |  | O None |
| DOT Shipping Name: See DOT addendum. | BIS(l,2,2,6,6-PENTAMETHYL-4-PIPERDINYL)SEBACATE |  |
|  | 41556-26-7 None | A None |
| Hazardous Materials Information: See Section 10. |  | 0 None |
| SECTION 2 - Composition, Information on Ingredients | BUTYL ACETATE |  |
|  | 123-86-4 10.0 | A 200.0 ppm |
| INGREDIENTS CAS # VAPOR EXPOSURE |  | 15 min STEL |
| PRESSURE UNITS |  | A 150.0 ppm |
| ACETONE |  | 0 150.0 ppm |
| 67-64-1 180.0 @68.0°F A750.0ppm |  |  |
| 15 min STEL | ETHYL ACETATE |  |
| A 500.0 ppm | 141-78-6 76.0 | A 400.0 ppm |
| 0 1000.0 ppm |  | O 400.0 ppm |
| D 500.0 ppm |  |  |
| 8 & 12 hour | ETHYL 3-ETHOXY PROPIONATE |  |
| TWA | 763-69-9 None | A None |
| . |  | O None |
| ACRYLIC POLYMER-A |  |  |
| Not Avail None A None | ETHYLBENZENE |  |
| 0 None | 100-41-4 7.0 | A 125.0 ppm |
|  |  | 15 min STEL |
| ACRYLIC POLYMER-B |  | A 100.0 ppm |
| 69215-54-9 None A None |  | 0 100.0 ppm |
| 0 None |  | D 25.0 ppm |
|  |  | 8 & 12 hour |
| ACRYLIC POLYMER-C |  | TWA |
| 71839-66-2 None A None |  |  |
| 0 None | ETHYLENE GLYCOL MONOBUTYL ETHER ACETATE |  |
|  | 112-07-2 0.3 | D 10.0 ppm |
| ACRYLIC RESIN |  | Skin |
| Not Avail None A None |  | A None |
| 0 None |  | O None |
| ALKYD RESIN | HEXYL ACETATE ISOMERS |  |
| 67763-06-8 None A None | 88230-35-7 0.7 | A 50.0 ppm |
| 0 None |  | hexyl acetate |
|  |  | O None |
| AROMATIC HYDROCARBON-A |  |  |
| 64742-94-5 10.0 D 100.0 ppm | METHYL AMYL KETONE |  |
| A None | 110-43-0 2.1 | A 50.0 ppm |
| 0 None |  | O 100.0 ppm |
| AROMATIC HYDROCARBON-B | METHYL ETHYL KETONE |  |
| 64742-95-6 10.0@ 25.0 °C D 50.0 ppm | 78-93-3 71.0@0.0 | A 300.0 ppm |
| A None |  | 15 min STEL |
| 0 None |  | D 300.0 ppm |
|  |  | 15 min TWA |
| BENZENE, l-CHLORO-4 CTRIFLUOROMETHYL) |  | A 200.0 ppm |
| 98-56-6 5.3 S 25.0 ppm |  | O 200.0 ppm |
| CEIL |  | D 200.0 ppm |
| D 20.0 ppm |  | 8 & 12 hour |

MSDS 28-1 January 1, 2002

"Copyright 2001. E. I. DuPont de Nemours and Company. All rights reserved. Copies may be made only for those using DuPont products."

MSDS 28-1 January 1, 2002

| INGREDIENTS | CAS # VAPOR PRESSURE | EXPOSURE UNITS | INGREDIENTS CAS # VAPOR EXPOSURE PRESSURE LIMITS |
| --- | --- | --- | --- |
|  |  | TWA | XYLENE |
|  |  |  | 1330-20-7 9.0 @ 25.0 °C D 150.0 ppm |
| METHYL ISOBUTYL KETONE |  |  | 15 min STEL |
|  | 108-10-1 15.0 | A 75.0 ppm | O 100.0 ppm |
|  |  | 15 min STEL | 8 hr PEL |
|  |  | A 50.0 ppm | O 100.0 ppm |
|  |  | 0 100.0 ppm | D 100.0 ppm |
|  |  |  | 8 & 12 hour |
| NAPHTHALENE |  |  | TWA |
|  | 91-20-3 1.0 @ 52.6 °C | A 10.0 ppm | A 150.0 ppm |
|  |  | 0 10.0 ppm | 15 min STEL |
|  |  |  | A4 Carcinogen |
| PETROLEUM NAPHTHA |  |  |  |
|  | 64742-89-8 50.0@ 25.0 °C | 0 400.0 ppm | 1,2,4-TRIMETHYL BENZENE |
|  |  | 15 min STEL | 95-63-6 7.0 @ 44.4 °C A 25.0 ppm |
|  |  | D 100.0 ppm | O 25.0 ppm |
|  |  | A 300.0 ppm | *A=ACGIH, 0=OSHA, D=DuPont, S=Suppliers. Limits are 8 hour TWA unless |
|  |  | 0 300.0 ppm | otherwise specified. Vapor pressure @25°C unless otherwise noted. |
| POLYESTER RESIN-A |  |  |  |
|  | Not Avail None | A None | SECTION 3 - Hazards Information |
|  |  | 0 None |  |
|  |  |  | Potential Health Effects: |
| POLYESTER RESIN-B |  |  | Inhalation: |
|  | 65086-73-9 None | A None | May cause nose and throat irritation. May cause nervous system |
|  |  | 0 None | depression characterized by the following progressive steps: |
|  |  |  | headache, dizziness, nausea, staggering gait, confusion, |
| POLYESTER RESIN-C |  |  | unconsciousness. Reports have associated repeated and prolonged |
|  | 68604-67-1 None | A None | overexposure to solvents with permanent brain and nervous system |
|  |  | 0 None | damage. |
| I |  |  | Ingestion: |
| POLYOL RESIN |  |  | May result in gastrointestinal distress. |
|  | Not Avail None | A None | Skin or eye contact: |
|  |  | 0 None | May cause irritation or burning of the eyes. Repeated or prolonged |
|  |  |  | liquid contact may cause skin irritation with discomfort and dermatitis. |
| PROPYLENE GLYCOL MONOMETHYL ETHER ACETATE |  |  |  |
|  | 108-65-6 3.7 | D 10.0 ppm | Other Potential Health Effects In addition to those listed above: |
|  |  | 12hrTWA |  |
|  |  | A None | ACETONE |
|  |  | 0 None | May cause irritation of the mucous membranes. The following medical |
|  |  |  | conditions may be aggravated by exposure: respiratory system skin |
| SUBSTITUTED BENZOTRIAZOLE-B |  |  | Repeated or prolonged liquid contact may cause skin irritation with |
|  | 127519-17-9 9.0 @ 25.0 °C | A None | discomfort and dermatitis. Overexposure may cause damage to any of |
|  |  |  | the following organs/systems: blood eyes kidneys liver respiratory |
|  |  | 0 None | system skin Aspiration may occur during swallowing or vomiting. |
|  |  |  | resulting in lung damage. Material may be harmful or fatal if |
| TOLUENE |  |  | swallowed. Ingestion may cause headache, nausea, vomiting, |
|  | 108-88-3 22.0 | 0 300.0 ppm | dizziness, and drowsiness. |
|  |  | CEIL |  |
|  |  | 0 200.0 ppm | ACRYLIC POLYMER-A |
|  |  | D 50.0 ppm | May cause temporary upper respiratory and/or lung irritation with |
|  |  | 8 & 12 hour | cough, difficult breathing, or shortness of breath. Contact may cause |
|  |  | TWA | skin irritation with discomfort or rash. May cause eye irritation with |
|  |  | A 50.0 ppm | discomfort, tearing, or blurred vision. |
|  |  | Skin |  |
|  |  | 0 500.0 ppm | AROMATIC HYDROCARBON-A |
|  |  | 10 min TWA | Laboratory studies with rats have shown that petroleum distillates can |
|  |  | Maximum | cause kidney damage and kidney or liver tumors. These effects were |
|  |  |  | not seen in similar studies with guinea pigs, dogs, or monkeys. |
| VM&P NAPHTHA |  |  | Several studies evaluating petroleum workers have not shown a |
|  | 64742-89-8 12.0 | 0 400.0 ppm | significant increase of kidney damage or an increase in kidney or liver |
|  |  | 15 min STEL | tumors. |
|  |  | D 100.0 ppm |  |
|  |  | A 300.0 ppm | AROMATIC HYDROCARBON-B |
|  |  | 0 300.0 ppm | Laboratory studies with rats have shown that petroleum distillates can |

cause kidney damage and kidney or liver tumors. These effects were

"Copyright 2001. E. I. DuPont de Nemours and Company. All rights reserved. Copies may be made only for those using DuPont products."

MSDS 28-1 January 1, 2002

MSDS 28-1 January 1, 2002

not seen in similar studies with guinea pigs, dogs, or monkeys. Several studies evaluating petroleum workers have not shown a significant increase of kidney damage or an increase in kidney or liver tumors.

BIS(l,2,2,6,6-PENTAMETHYL-4-PIPERDINYL)SEBACATE

Repeated exposure may cause allergic skin rash, itching, swelling.

BUTYLACETATE

May cause abnormal liver function. The following medical conditions may be aggravated by exposure: respiratory system. May cause eye irritation with discomfort, tearing, or blurred vision. Tests for embryotoxic activity in animals has been inconclusive. Rats exposed to very high airborne levels have exhibited high frequency hearing deficits. The significance of this to man is unknown. Has been toxic to the fetus in laboratory animals at doses that are toxic to the mother.

ETHYL ACETATE

Prolonged and repeated high exposures of laboratory animals resulted in secondary anemia with an increase in white blood cells; fatty degeneration, cloudy swelling and an excess of blood in various organs.

ETHYL 3-ETHOXY PROPIONATE

Has been toxic to the fetus in laboratory animals at doses that are toxic to the mother.

ETHYLBENZENE

Is an IARC, NTP or OSHA carcinogen. Individuals with preexisting diseases of the central nervous system, lungs, liver, or kidneys may have increased susceptibility to the toxicity of excessive exposures. Recurrent overexposure may result in liver and kidney injury. Studies in laboratory animals have shown reproductive, embryotoxic and developmental effects.

ETHYLENE GLYCOL MONOBUTYL ETHER ACETATE

May destroy red blood cells. May cause abnormal kidney function. May cause temporary upper respiratory and/or lung irritation with cough, difficult breathing, or shortness of breath. The following medical conditions may be aggravated by exposure: central nervous system, gastrointestinal system, kidneys, liver, Dermatitis. Can be absorbed through the skin in harmful amounts. Overexposure may cause damage to any of the following organs/systems: blood , kidneys, liver. Ingestion may cause headache, nausea, vomiting, dizziness, and drowsiness.

METHYL AMYL KETONE

Ingestion studies on laboratory animals showed that very high oral doses caused increased liver and kidney weights. Over exposure may cause eye, nose and throat irritation. Repeated or prolonged liquid contact may cause skin irritation and dermatitis. May cause central nervous system effects such as dizziness, headache, nausea, and loss of consciousness. Repeated and prolonged overexposure to solvents may lead to permanent brain and nervous system damage. Inhalation of high vapor concentrations may cause drowsiness and irritation. Prolonged or repeated skin contact may cause drying, cracking, or irritation.

METHYL ETHYL KETONE

Material is irritating to mucous membranes and upper respiratory tract. Persons with certain types of neurological disease such as multiple sclerosis should consult a physician prior to exposure. Increased susceptibility to the effects of this material may be observed in people with preexisting disease of any of the following: central nervous system, eyes, respiratory system, skin. Contact may cause skin irritation with discomfort or rash. Can irritate or burn eyes. Prolonged or repeated overexposure may cause any of the following: Conjunctivitis, Dermatitis. High concentrations have caused

embryotoxic effects in laboratory animals. Methyl ethyl ketone has been demonstrated to potentiate (i.e., shorten the time of onset) the peripheral neuropathy caused by either n-hexane or methyl n-butyl ketone. MEK by itself has not been demonstrated to cause peripheral neuropathy. Mice that were force fed (gavage) showed teratogenic effects embryofetotoxicity and maternal toxicity. The no observed effect level was estimated to be 25g/50 kg (110 Ib.) woman. Laboratory animals exposed to high airborne levels showed tissue changes in the nasal passages. Aspiration may occur during swallowing or vomiting, resulting in lung damage. May cause central nervous system depression with headache, stupor, uncoordinated or strange behavior, or unconsciousness.

METHYL ISOBUTYL KETONE

Individuals with preexisting diseases of the central nervous system or lungs may have increased susceptibility to the toxicity of excessive exposures. Recurrent overexposure may result in liver and kidney injury.

NAPHTHALENE

Individuals with preexisting diseases of the liver or kidneys may have increased susceptibility to the toxicity of excessive exposures. Recurrent overexposure may result in liver and kidney injury.

PETROLEUM NAPHTHA

Laboratory studies with rats have shown that petroleum distillates can cause kidney damage and kidney or liver tumors. These effects were not seen in similar studies with guinea pigs, dogs, or monkeys. Several studies evaluating petroleum workers have not shown a significant increase of kidney damage or an increase in kidney or liver tumors.

PROPYLENE GLYCOL MONOMETHYL ETHER ACETATE

May cause eye irritation with discomfort, tearing, or blurred vision. May cause moderate eye burning. Recurrent overexposure may result in liver and kidney injury. May cause irritation of the upper respiratory passages.

TOLUENE

Chromosomal changes in the circulating blood of exposed workers have been reported. The significance of these reports is unclear because of exposure to other substances. Increased susceptibility to the effects of this material may be observed in people with preexisting disease of any of the following: central nervous system kidneys liver respiratory system skin. May cause eye irritation with discomfort, tearing, or blurred vision. Can be absorbed through the skin in harmful amounts. Recurrent overexposure may result in liver and kidney injury. High airborne levels have produced irregular heart beats in animals and occasional palpitations in humans. Rats exposed to very high airborne levels have exhibited high frequency hearing deficits. The significance of this to man is unknown. Aspiration may occur during swallowing or vomiting, resulting in lung damage. Material may be harmful or fatal if swallowed. WARNING: This chemical is known to the State of California to cause birth defects or other reproductive harm.

VM&P NAPHTHA

Laboratory studies with rats have shown that petroleum distillates can cause kidney damage and kidney or liver tumors. These effects were not seen in similar studies with guinea pigs, dogs, or monkeys. Several studies evaluating petroleum workers have not shown a significant increase of kidney damage or an increase in kidney or liver tumors.

XYLENE

"Copyright 2001. E. I. DuPont de Nemours and Company. All rights reserved. Copies may be made only for those using DuPont products."

MSDS 28-1 January 1, 2002

MSDS 28-1 January 1, 2002

Increased susceptibility to the effects of this material may be observed in people with preexisting disease of any of the following: bone marrow cardiovascular system central nervous system kidneys liver lungs Can be absorbed through the skin in harmful amounts. Can irritate or burn eyes. Recurrent overexposure may result in liver and kidney injury. High exposures may produce irregular heart beats. Canada classifies Xylene as a developmental toxin as high exposures to xylenes in some animal studies have been reported to cause health effects on the developing fetus/embryo. These effects were often at levels toxic to the adult animal. The significance of these effects to humans is not known. Aspiration may occur during swallowing or vomiting, resulting in lung damage. Material may be harmful or fatal if swallowed. Prolonged or repeated skin contact may cause drying, cracking, or irritation.

PRODUCT CODE

SECTION 4 - First Aid Measures

First Aid Procedures: Inhalation:

If affected by inhalation of vapor or spray mist, move to fresh air. If not breathing, give artificial respiration, preferably mouth-to-mouth. If breathing difficulty persists, or occurs later, consult a physician. Ingestion:

In the unlikely event of ingestion, DO NOT INDUCE VOMITING. Call a physician immediately and have names of ingredients available. Skin or eye:

In case of eye contact, immediately flush with plenty of water for at least 15 minutes; call a physician. In case of skin contact, wash thoroughly with soap and water. If irritation occurs, contact a physician.

SECTION 5 - Firefighting Measures

See Section 11 for exact values.
LFLO.0%	UFL13.1%

Flash Point (Closed Cup) Flammable limits Extinguishing media:

Universal aqueous film-forming foam, carbon dioxide, dry chemical. Fire fighting procedures:

Full protective equipment, including self-contained breathing apparatus, is recommended. Water from fog nozzles may be used to prevent pressure build-up. Fire & explosion hazards:

For flammable liquids, vapor/air will ignite when an ignition source is present. In other cases, when heated above the flash point, emits flammable vapors which, when mixed with air, can burn or be explosive. Fine mists or sprays may be flammable at temperatures below the flash point.

SECTION 6 - Accidental Release Measures

Steps to be taken in case material is released or spilled:

Ventilate area. Remove sources of ignition. Prevent skin and eye contact and breathing of vapor. Wear a property fitted air-purifying respirator with organic vapor cartridges (NIOSH approved TC-23C), eye protection, gloves and protective clothing. Confine, remove with inert absorbent, and dispose of properly.

SECTION 7 - Handling and Storage

Precautions to be taken in handling and storing:

Observe label precautions. If combustible (flashpoint between 100 -200 ° F), keep away from heat, sparks and flame. If flammable (flashpoint less than 100 ° F), also keep away from static discharges and other sources of ignition. If material is extremely flammable (flashpoint less than 20 ° F) or flammable, VAPORS MAY IGNITE EXPLOSIVELY OR CAUSE FLASH FIRE, respectively. Vapors may spread long distances. Prevent buildup of vapors. Close container after each use. Ground containers when pouring. Wash thoroughly

after handling and before eating or smoking. Do not store above 120 0 F. If product is waterbased, do not freeze. Other precautions:

If material is a coating: do not sand, flame cut, braze or weld dry coating without a NIOSH approved respirator or appropriate ventilation.

SECTION 8 - Exposure Controls or Personal Protection

Engineering controls and work practices: Ventilation:

Provide sufficient ventilation in volume and pattern to keep contaminants below applicable exposure limits.

Respiratory:

Do not breathe vapors or mists. Wear a properly fitted air-purifying respirator with organic vapor cartridges (NIOSH approved TC-23C) and particulate filter (NIOSH TC-84A) during application and until all vapors and spray mists are exhausted. In confined spaces, or in situations where continuous spray operations are typical, or if proper air-purifying respirator fit is not possible, wear a positive pressure, supplied-air respirator (NIOSH TC-19C), In all cases, follow respirator manufacturer s directions for respirator use. Do not permit anyone without protection in the painting area.

Protective clothing:

Neoprene gloves and coveralls are recommended.

Eye protection:

Desirable in all industrial situations. Goggles are preferred to prevent eye irritation. If safety glasses are substituted, include splash guard or side shields.

SECTION 9 - Physical and Chemical Properties

Evaporation Rate

Slower than Ether

NIL

Heavier than air

-17-0°(C)

-92--73°(C)

7.36-9.17

0.88-1.10

46.15-94.53

41.06 - 93.21

5.47 - 53.85

6.80 - 58.94

Solubility in water

Vapor Density

Approx. boiling range (°C)

Approx. freezing range (°C)

Gallon weight (Ibs/gal)

Specific gravity

Percent volatile by volume

Percent volatile by weight

Percent solids by volume

Percent solids by weight

SECTION 10 - Stability and Reactivity

Stability:

Stable

None reasonably foreseeable

Incompatibility (materials to avoid):

Hazardous decomposition products:

CO, C02, smoke, and oxides of any heavy metals that are reported in

Section 2.

Hazardous polymerization:	Will not occur.

Sensitivity to static discharge:

For flammable materials (flashpoint less than 100 °F) and
combustibles (flashpoint between 100-200 ° F) if heated above the
flashpoint, solvent vapors in air may explode if static grounding and
bonding is not used during transfer of this product.
Sensitivity to mechanical impact:	Not Applicable

SECTION 11 - Additional Information

INGREDIENTS (Product Specific)

401-20 Acrylic Polymer-B, Butyl Acetate, Ethyl Acetate, Ethyl 3-Ethoxy Propionate, Ethylbenzene(0.8%*@), Methyl Isobutyl Ketone(2%*@), Petroleum Naphtha, Polyester Resin-C, Propylene Glycol Monomethyl Ether Acetate, Toluene(2-2%*@), VM&P Naphtha, Xylene(3-4%*@)

"Copyright 2001. E. I. DuPont de Nemours and Company. All rights reserved. Copies may be made only for those using DuPont products."

MSDS 28-1 January 1, 2002

MSDS 28-1 January 1, 2002

GAL WT: 8.08 WT PCT SOUDS:41.38 VOL PCT SOUDS:34.29
SOLVENT DENSITY: 7.20 V	OC LE: 4.7 VOC AP: 4.7

FLASH POINT: 20 'F to below 73-F H: 2 F: 3 R: 0 OSHA STORAGE: IB TSCA STATUS: In compliance       PHOTOCHEMICALLY REACTIVE: NO

GAL WT: 7.97 WT PCT SOUDS:35.87 VOL PCT SOLIDS:30.67
SOLVENT DENSITY: 7.37	VOC LE: 4.2 VOC AP: 3.1

FLASH POINT: 20*F to below 73*F H: 2 F: 3 R: 1 OSHA STORAGE: IA TSCA STATUS: In compliance       PHOTOCHEMICALLY REACTIVE: NO

401-21 Acetone, Acrylic Polymer-A, Butyl Acetate, Ethyl 3-Ethoxy Propionate, Ethylbenzene(0.5%*@), Methyl Amyl Ketone, Methyl Isobutyl Ketone(2%*@), Polyester Resin-C, Propylene Glycol Monomethyl Ether Acetate, Toluene(2%*@] VM&P Naphtha, Xylene(2-2%*@)

GAL WT: 7.90 WT PCT SOUDS:41.50 VOL PCT SOUDS:33.67
SOLVENT DENSITY: 6.95	VOC LE: 4.1 VOC AP: 3.2

FLASH POINT: Below 20*F H: 2 F: 3 R: 0 OSHA STORAGE: IA TSCA STATUS: In compliance        PHOTOCHEMICALLY REACTIVE: NO

497-00 Acetone, Acrylic Polymer-A, Aromatic Hydrocarbon-B, Butyl Acetate, Ethylbenzene(1.9%*@), Hexyl Acetate Isomers, Methyl Amyl Ketone, Methyl Ethyl Ketone(l%*@), Polyester Resin-B , Xylene(8-10%*@), 1,2,4-Trimethyl Benzene(l-4%*)

GAL WT: 7.73 WT PCT SOUDS:37.11 VOL PCT SOUDS:29.93
SOLVENT DENSITY: 6.92	VOC LE: 4.2 VOC AP: 3.0

FLASH POINT: 20'F to below 73"F H: 2 F: 3 R: 0 OSHA STORAGE: IA TSCA STATUS: In compliance        PHOTOCHEMICALLY REACTIVE: YES

419-00 Acrylic Polymer-C, Alkyd Resin, Butyl Acetate, Ethylbenzene (0.3%*@ Toluene(9%*@), VM&P Naphtha, Xylene(l-l%*@)

GAL WT: 7.59 WT PCT SOLIDS:43.78 VOL PCT SOUDS:36.19
SOLVENT DENSITY: 6.70	VOC LE: 4.3 VOC AP: 4.3

FLASH POINT: 20*F to below 73*F H: 2 F: 3 R: 0 OSHA STORAGE: IB TSCA STATUS: In compliance       PHOTOCHEMICALLY REACTIVE: NO

422-14 Acrylic Polymer-A, Aromatic Hydrocarbon-A, Butyl Acetate, Ethylbenzene (2.6-6.6%*@), Naphthalene(0-l%*@), Toluene(47%*@), Xylene(20-24%*@), 1,2,4-Trimethyl Benzene(0-l%*)

GAL WT: 7.36 WT PCT SOLIDS: 6.80 VOL PCT SOLIDS: 5.47
SOLVENT DENSITY: 7.26	VOC LE: 6.9 VOC AP: 6.9

FLASH POINT: 20 • F to below 73* F H: 2 F: 3 R: 0 OSHA STORAGE: IA TSCA STATUS: In compliance       PHOTOCHEMICALLY REACTIVE: YES

466-00 Acetone, Acrylic Resin, Benzene,l-Chloro-4 (Trifluoromethyl), Bis(l,2,2,6,6-Pentamethyl-4-Piperdinyl) Sebacate, Butyl Acetate, Ethyl 3-Ethoxy Propionate, Methyl Amyl Ketone, Polyester Resin-A, Substituted Benzotriazole-B

GAL WT: 9.17 WT PCT SOUDS:48.64 VOL PCT SOLIDS:47.75
SOLVENT DENSITY: 9.01	VOC LE: 2.5 VOC AP: 1.8

FLASH POINT: 20*F to below 73*F H: 2 F: 3 R: 1 OSHA STORAGE: IB TSCA STATUS: In compliance       PHOTOCHEMICALLY REACTIVE: NO

483-99 Acrylic Polymer-A, Beta-(3-(2h-Benzotriazol-2-YI)-4-Hydroxy-5-Tertbutylphenyl)Propionate, Bis(l,2,2,6,6-Pentamethyl 4-Piperdinyl) Sebacate, Ethyl Acetate, Ethyl 3-Ethoxy Propionate, Ethylbenzene(0.3%*@), Ethylene Glycol Monobutyl Ether Acetate(3%*@), Methyl Amyl Ketone, Methyl Isobutyl Ketone(3%*@), Polyester Resin-C, Polyol Resin(9%*), Toluene(3 %*@), Xylene(l%*@)

GAL WT: 8.41 WT PCT SOUDS:58.94 VOL PCT SOLIDS-.53.85
SOLVENT DENSITY: 7.48	VOC LE: 3.5 VOC AP: 3.5

FLASH POINT: 20 * F to below 73* F H: 2 F: 3 R: 0 OSHA STORAGE: IA TSCA STATUS: In compliance       PHOTOCHEMICALLY REACTIVE: NO

Footnotes:

TSCA: in compliance = In compliance with TSCA Inventory requirements for commercial purposes.

ACGIH = American Conference of Governmental Industrial Hygienists IARC =   International Agency for Research on Cancer NTP =      National Toxicology Program OSHA =   Occupational Safety and Health Administration PNOR=    Particles Not Otherwise Regulated PNOC=    Particles Not Otherwise Classified STEL=     Short Term Exposure Limit TWA =     Time-Weighted Average

*	=	Section 313 Supplier Notification: These chemicals are subject to

the reporting requirements of Section 313 of the Emergency planning and Right-to-Know Act of 1986 and of 40 CFR 372. @ =         Clean Air Act Hazardous Air Pollutant.

#	=	EPCRA Section 302 - Extremely Hazardous Substance.

NOTICE:

The information on this Material Safety Data Sheet relates only to the specific material designated herein and does not relate to use in combination with any other material or in any process.

Product Manager - Refinish Sales Prepared by: Emily L. Taylor

486-00 Acetone, Acrylic Polymer-A, Benzene,l-Chloro-4 (Trifluoromethyl), Butyl Acetate, Ethyl 3-Ethoxy Propionate, Ethylbenzene(1.8-4.4%*@), Ethylene Glycol Monobutyl Ether Acetate(2 %*@), Methyl Amyl Ketone, Polyester Resin-B, Xylene(13-16%*@)

GAL WT: 7.93 WT PCT SOUDS:38.73 VOL PCT SOUDS:33.76
SOLVENT DENSITY: 7.32	VOC LE: 3.8 VOC AP: 2.8

FLASH POINT: Below 20*F H: 2 F: 3 R: 1 OSHA STORAGE: IA TSCA STATUS: In compliance       PHOTOCHEMICALLY REACTIVE: YES

496-00 Acetone, Acrylic Polymer-B, Benzene,l-Chloro-4 (Trifluoromethyl), Butyl Acetate, Ethylbenzene(4.8%*@), Methyl Ethyl Ketone(7%*@), Xylene(20-24%*@)

"Copyright 2001. E. I. DuPont de Nemours and Company. All rights reserved. Copies may be made only for those using DuPont products."

MSDS 28-1 January 1, 2002