FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

MSDS #:    456.00

Material Safety Data Sheet (MSDS)	Revision Date: November 25,2002

r

Section 1 — Chemical Product and Company Identification

Lithium Nitrate

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition. Information on Ingredients

Lithium Nitrate

CAS#: 7790-69-4

Section 3 — Hazards Identification

Colorless odorless powder.	FLINN AT-A-GLANCE

Slightly toxic by ingestion and inhalation. Irritant to body tissues.	Health-1

Strong oxidizer, explosion risk when shocked or heated.	Flammability-0

Reactivity-3 Exposure-1 Storage-2

0 is low hazard, 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give large quantities of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Non flammable solid.	NFPA code

Oxidizer; explosion risk when shocked or heated.	None Established

When heated to decomposition, emits toxic fumes of NOx.

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE

and SCBA with full facepiece operated in positive pressure mode.

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #3. Store with amides, nitrates and nitrites. Store in a Flinn Chem-Saf bag and then inside a Flinn Saf-Stor can.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron.

© 2002 Flinn Scientific, Inc. All Rights Reserved.	page 1 OF 2

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Material Safety Data Sheet (MSDS)

Lithium Nitrate

MSDS #:    456.00

Revision Date:   November 25, 2002

^

Section 9 — Physical and Chemical Properties

Colorless, odorless powder. Solubility: Soluble in water and alcohol. Formula: LiNO3 Formula Weight: 68.95

Specific Gravity: 2.38 Melting Point: 261 C

Section 10 — Stability and Reactivity

Avoid contact with strong reducers and organic materials. Shelf life: Indefinite, if stored safely.

Section 11 —Toxicoloqical Information

Acute effects: Irritant, stomach pains, vomiting, diarrhea, nausea,

dizziness and headache

Chronic effects: Convulsions

Target organs: Blood, central nervous system

ORL-RAT LD50: N.A. IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

N.A. = Not available, not all health aspects of this substance have been fully investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option.

Section 14 — Transport Information

Shipping Name: Lithium nitrate Hazard Class: 5.1, Oxidizer

UN Number: UN2722

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (232-218-9), RCRA code D001.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

/^   I

Improve Student Lab Results— Use Flinn Chemicals

flinn@flinnsci.com   www.flinnsci.com

P.O. Box 219   Batavia IL 60510

(800) 452-1261 Fax (866) 452-1436

© 2002 Flinn Scientific, Inc. All Rights Reserved.

page 2 of 2