r

COMPANY. USA

A  DIVISION  OF  EXXON   CORPORATION

- G27 SOLVENT

DATE ISSUED:      11/O4/88 SUPERSEDES DATE:  O4/O7/88

MATERIAL SAFETY DATA SHEET

EXXON COMPANY, U.S.A.    P.O. BOX 2180    HOUSTON. TX 77252-2180

A    IDENTIFICATION AND EMERGENCY INFORMATION

PRODUCT NAME 627 SOLVENT

CHEMICAL NAME Petroleum Solvent

PRODUCT CODE 131031 - O0627

CAS NUMBER 8052-41-3

PRODUCT APPEARANCE AND ODOR Clear water-white liquid Mineral spirits odor

EMERGENCY TELEPHONE NUMBER (713) 656-3424

B.    COMPONENTS AND HAZARD INFORMATION

COMPONENTS

CAS NO. OF COMPONENTS

APPROXIMATE CONCENTRATION

8052-41-3 It consists predominantly of C7-C1O hydrocarbons.

This product can be defined as: Stoddard Sol vent

100%

This product contains:

C7-C10 saturated hydrocarbons

Toluene

Xylene

Ethylbenzene

C8+ Aromatlcs

Mixture

108-88-3

133O-2O-7

1OO-41-4

Mixture

Approximately 83% Approx1 mate1y 0.4% Approximately O.8% Approximately O.2% Approximately 14%

See Section E for Health and Hazard Information.

See Section H for additional Environmental Information.

HAZARDOUS MATERIALS IDENTIFICATION SYSTEM (HMIS) Health Flammabillty Reactivity BASIS

'•-••': 12	O     Recommended by Exxon

EXPOSURE LIMIT FOR TOTAL PRODUCT 100 ppm (525 mg/m3) for an 8-hour workday

500 ppm 8-hour TWA

BASIS

Recommended by the American Conference of Governmental

Industrial Hygienists (ACGIH) for Stoddard Solvent — -

OSHA Regulation 29 CFR 191O.1OOO

PRIMARY ROUTES OF ENTRY

AND EMERGENCY AND FIRST AID PROCEDURES

EYE CONTACT

If splashed Into the eyes, flush with clear water for 15 minutes or until Irritation
subsides.  If Irritation persists, call a physician.	...'•••••

SKIN

In case of skin contact, remove any contaminated clothing and wash skin thoroughly with soap and water.

INHALATION

If overcome by vapor, remove from exposure and call a physician immediately.  If breathing Is Irregular or has stopped, start resuscitation, administer oxygen, if available.

INGESTION

If ingested, DO NOT induce vomiting; call a physician immediately.

^

D.    FIRE AND EXPLOSION HAZARD INFORMATION

FLASH POINT (MINIMUM)

COMBUSTIBLE  -  Per DOT 49 CFR 173.115

38'C (100'F)

ASTM D 56, Tag Closed Cup

AUTOIGNITION TEMPERATURE

Approximately 254'C (49O*F) ASTM D 2155

NATIONAL FIRE PROTECTION ASSOCIATION (NFPA) - HAZARD IDENTIFICATION Health Flammabillty Reactivity   BASIS

02	0       Recommended by the National Fire Protection Association

HANDLING PRECAUTIONS

This liquid is volatile and gives off invisible vapors.  Either the liquid or vapor may settle in low areas or travel some distance along the ground or surface to ignition sources where they may ignite or explode.

Keep product away from ignition sources, such as heat, sparks, pilot lights, static electricity, and open flames.

FLAMMABLE OR EXPLOSIVE LIMITS (APPROXIMATE PERCENT BY VOLUME IN AIR)

Estimated values: Lower Flammable Limit O.9%    Upper Flammable Limit 7%

EXTINGUISHING MEDIA AND FIRE FIGHTING PROCEDURES

Foam, water spray (fog), dry chemical, carbon dioxide and vaporizing liquid type extinguishing agents may all be suitable for extinguishing fires involving this type of product, depending on size or potential size of fire and circumstances related to the situation.  Plan fire protection and response strategy through consultation with local fire protection authorities or appropriate specialists.

The following procedures for this type of product are based on the recommendations In the National Fire Protection Association's "Fire Protection Guide on Hazardous Materials", Eighth Edition (1984):

Use dry chemical, foam or carbon dioxide.  Water may be ineffective, but water should be used to keep fire-exposed containers cool.  If a leak or spill has Ignited, use water spray to disperse the vapors and to protect men attempting to stop a leak.  Water spray may be used to flush spills away from exposures.  Minimize breathing gases, vapor, fumes or decomposition products.  Use supplied-air breathing equipment for enclosed or confined spaces or as otherwise needed.

NOTE:  The inclusion of the phrase "water may be ineffective" is to Indicate that although water can be used to cool and protect exposed material, water may not extinguish the fire unless used under favorable conditions by experienced fire fighters trained in fighting all types of flammable liquid fires.

DECOMPOSITION PRODUCTS UNDER FIRE CONDITIONS

Fumes, smoke, carbon monoxide, aldehydes and other decomposition products, incomplete combustion.

in the case of

"EMPTY" CONTAINER WARNING

"Empty" containers retain residue (liquid and/or vapor) and can be dangerous.  DO NOT PRESSURIZE, CUT, WELD, BRAZE, SOLDER, DRILL, GRIND OR EXPOSE SUCH CONTAINERS TO HEAT, FLAME, SPARKS, STATIC ELECTRICITY, OR OTHER SOURCES OF IGNITION; THEY MAY EXPLODE AND CAUSE INJURY OR DEATH.  Do not attempt to clean since residue is difficult to remove. "Empty" drums should be completely drained, properly bunged and promptly returned to a drum reconditloner. All other containers should be disposed of in an environmentally safe manner and in accordance with governmental regulations.  For work on tanks refer to Occupational Safety and Health Administration regulations, ANSI Z49.1, and other governmental and industrial references pertaining to cleaning, repairing, welding, or other contemplated operations.

E.    HEALTH AND HAZARD INFORMATION

VARIABILITY AMONG INDIVIDUALS

Health studies have shown that many petroleum hydrocarbons and synthetic lubricants pose potential human health risks which may vary from person to person.  As a precaution, exposure to liquids, vapors, mists or fumes should be minimized.

EFFECTS OF OVEREXPOSURE (Signs and symptoms of exposure)

High' vapor concentrations (greater than approximately 10OO ppm) are Irritating to the eyes .•and the respiratory tr.act, may.cause headaches and dizziness, are anesthetic, and may have ...Other central nervous system effects Including death.

i

NATURE OF HAZARD AND TOXICITY INFORMATION

Prolonged or repeated skin contact with this product tends to remove skin oils possibly leading to Irritation and dermatitis; however, based on human experience and available toxlcologlcal data, this product 1s judged to be neither a "corrosive" nor an "Irritant" by OSHA criteria.

Product contacting the eyes may cause eye Irritation.

Laboratory animal studies have shown that prolonged and repeated Inhalation exposure to light ' hydrocarbon vapors 1n the same naphtha boiling range as this product can produce adverse kidney effects 1n male rats.  However, these effects were not observed 1n similar studies with female rats and male and female mice and in limited studies with other animal species. Additionally, in a number of human studies, there was no clinical evidence of such effects at -normal occupational levels.  It 1s therefore highly unlikely that the kidney effects observed 1n male rats have significant Implications for humans exposed at or below the recommended vapor limits 1n the workplace.

Product has a low order of acute oral and dermal toxicity, but minute amounts aspirated Into the lungs during Ingestion or vomiting may cause mild to severe pulmonary Injury and possibly death.

This product is judged to have an acute oral LD5O (rat) greater than 5 g/kg of body weight, and an acute dermal LD50 (rabbit) greater than 3.16 g/kg of body weight.

PRE-EXISTING MEDICAL CONDITIONS WHICH MAY BE AGGRAVATED BY EXPOSURE Petroleum Solvents/Petroleum Hydrocarbons - Skin contact may aggravate an existing dermatitis.

F.    PHYSICAL DATA

The following data are approximate or typical  values and should not be used for precise design purposes.

BOILING RANGE Approximately  152-202*C   (305-396*F)

SPECIFIC GRAVITY  (15.6  C/15.6 C) 0.79

MOLECULAR WEIGHT

140

VAPOR PRESSURE Less than 1O mm Hg O 25* C ASTM D 2879

VAPOR DENSITY (AIR = 1)

Approximately 4.8     -------- ........ -

PERCENT VOLATILE BY VOLUME 1OO ®> 1 atm. and 25* C (77' F)

PH

EVAPORATION RATE (n-BUTYL ACETATE Less than O. 1

Essentially neutral

POUR,   CONGEALING OR MELTING POINT Less  than  -18'C   (0* F) Pour  Point  by ASTM  D  97

VISCOSITY 0.95  cP     *  25'C  ASTM   D   445

1 ATM. AND 25 C (77 F)

1)

SOLUBILITY   IN WATER <?   1   ATM.   AND 25  C   (77   F)

Negligible;   less   than O. 1%

G.    REACTIVITY

^

This product is stable and will not react violently with water. Hazardous polymerization will not occur. Avoid contact with strong oxidants such as liquid chlorine, concentrated oxygen, sodium hypochlorite or calcium hypochlorite.

H.    ENVIRONMENTAL INFORMATION

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED

Shut off and" eliminate all ignition sources.  Keep people away.  Recover free product.  Add sand. earth or other suitable absorbent to spill area.  Minimize breathing vapors.  Minimize skin contact.  Ventilate confined spaces.  Open all windows and doors.  Keep product out of sewers and watercourses by diking or impounding.  Advise authorities if product has entered or may enter sewers, watercourses, or extensive land areas.

Assure conformity with applicable governmental regulations.  Continue to observe precautions for volatile, combustible vapors, from absorbed material.

THE FOLLOWING INFORMATION MAY BE USEFUL IN COMPLYING WITH VARIOUS STATE AND FEDERAL REGULATIONS UNDER VARIOUS ENVIRONMENTAL STATUES:

REPORTABLE QUANTITY (RQ), EPA REGULATION 40 CFR 302 (CERCLA Section 1O2)

Not appl1 cable

THRESHOLD PLANNING QUANTITY (TPQ), EPA REGULATION 40 CFR 355 (SARA Sections 301-304)

Not applicable

TOXIC CHEMICAL RELEASE REPORTING, EPA REGULATION 40 CFR 372 (SARA Sections 311-313)

Not applicable

Acute   Chronic  Fire    Pressure  Reactive EPA HAZARD CLASSIFICATION CODE:  Hazard  Hazard   Hazard  Hazard    Hazard    Not Applicable

XXX	XXX

I.    PROTECTION AND PRECAUTIONS

VENTILATION

Use only with ventilation sufficient to prevent exceeding recommended exposure limit or buildup of explosive concentrations of vapor in air.  Use explosion-proof equipment.  No smoking or open 11ghts.

RESPIRATORY PROTECTION

Use supplied-air respiratory protection in confined or enclosed spaces, if needed.

PROTECTIVE GLOVES Use chemical-resistant gloves, if needed, to avoid prolonged or repeated skin contact.

EYE PROTECTION

Use splash goggles or face shield when eye contact may occur.

OTHER PROTECTIVE EQUIPMENT	,

Use chemical-resistant apron or other impervious clothing, if needed, to avoid contaminating regular clothing which could result in prolonged or repeated skin contact.

WORK PRACTICES / ENGINEERING CONTROLS

Keep containers and storage containers closed when not in use.  Do not store near heat, sparks, flame or strong oxidants.  To prevent fire or explosion risk from static accumulation and discharge, effectively ground product transfer system in accordance with the National Fire Protection Association standard for petroleum products.

PERSONAL HYGIENE

Minimize breathing vapor or mist.  Avoid prolonged or repeated contact with skin.  Remove contaminated clothing; launder or dry-clean before reuse.  Remove contaminated shoes and thoroughly clean and dry before reuse.  Cleanse skin thoroughly after contact, before breaks and meals, and at end of work period.  Product is readily removed from skin by waterless hand cleaners followed by washing thoroughly with soap and water.

J.    TRANSPORTATION AND OSHA RELATED LABEL INFORMATION

TRANSPORTATION INCIDENT INFORMATION

For further Information relative to spills resulting from transportation incidents, refer to latest Department of Transportation Emergency Response Guidebook for Hazardous Materials Incidents, DOT P 580O.3.

DOT IDENTIFICATION NUMBER

UN 1255

OSHA REQUIRED LABEL INFORMATION

In compliance with hazard and right-to-know requirements, the following OSHA Hazard Warnings should be found on a label, bill of lading or invoice accompanying this shipment.

DANGER!

COMBUSTIBLE

Note:  Product label will contain additional non-OSHA related information.

The information and recommendations contained herein are, to the best of Exxon's knowledge and belief, accurate and reliable as of the date issued.  Exxon does not warrant or guarantee their accuracy or reliability, and Exxon shall not be liable for any loss or damage arising out of the use thereof.

The Information and recommendations are offered for the user's consideration and examination, and it is the user's responsibility to satisfy Itself that they are suitable and complete for its particular use.  If buyer repackages this product, legal council should be consulted to insure proper health, safety and other necessary Information is included on the container.

The Environmental Information Included under Section H hereof as well as the Hazardous Materials Identification System (HMIS) and National Fire Protection Association (NFPA) ratings have been Included by Exxon Company, U.S.A. in order to provide additional health and hazard classification information. The ratings recommended are based upon the criteria supplied by the developers of these rating systems, together with Exxon's Interpretation of the available data.

FOR ADDITIONAL INFORMATION ON HEALTH EFFECTS CONTACT:

DIRECTOR OF INDUSTRIAL HYGIENE

EXXON COMPANY, U.S.A.

P. 0. BOX 2180 ROOM 3157

HOUSTON, TX 77252-2180

(713) 656-2443

FOR OTHER PRODUCT INFORMATION CONTACT:

MANAGER. MARKETING TECHNICAL SERVICES

EXXON COMPANY, U.S.A.

P. 0. BOX 218O ROOM 2355

HOUSTON, TX  77252-2180

(713) 656-5949