MATERIAL SAFETY DATA SHEET:MICROLIZER. O/G

DATE OF ISSUE: 04/27/2000	SUPERCEDES: 01/19/2000

SECTION I - GENERAL INFORMATION
Chemical Name & Synonyms:	Trade Name & Synonyms:

N/A	MICROLIZER, O/G

Chemical Family:	Formula Mixture: X

AQUEOUS UREA/PHOSPHATE SOLUTION

Manufacturer's Name:

MANTEK, DIVISION OF NCH CORP.

Address:

BOX 152170

IRVING, TEXAS 75015

Prepared By:	Product Code Number	Emergency Phone Number

R STOLLEY/CHEMIST	0365	800-424-9300

SECTION II -HAZARDOUS INGREDIENTS

THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS
Chemical Name (Ingredients)	Hazard         TJLV	PEL	STEL	CAS #

UREA	$	NOTEST. 1       NOT EST. 2      NOT EST.      57-13-6

TETRAPOTASSIUM PYROPHOSPHATE	IRRITANT     NOT EST 1.      NOT EST 2.      NOTEST.     7320-34-5

$ IRRITANT/MUTAGEN/TUMORIGEN

Boiling Point (f):

Vapor Pressure (MM HG):

Vapor Density (Air=1):

PH@100%:

% Volatile by Volume:

H20 Solubility:

SECTION III - PHYSICAL DATA

210	Specific Gravity (H20=1):

18	Color:

0.6	Odor:

6.5-7.5	Clarity

70	Evaporation Rate (BU A/C=1):

COMPLETE	Viscosity:

1.28

BLUE

AMMONIACAL

TRANSPARENT

0.10

NON-VISCOUS

SECTION IV - FIRE AND EXPLOSION HAZARD
Flash Point:	Flammable Limits:	LEL:	DEL:

NON-FLAM/	N/A	N/A	N/A

Extinguishing Media:

Foam:X	Alcohol Foam:X	CO2:X	Dry ChemicahX	Water SprayiX	OtherX

Special Fire Fighting Procedures:

FIREFIGHTERS SHOULD WEAR A SELF-CONTAINED BREATHING APPARATUS AND FULL PROTECTIVE GEAR. EXTINGUISHING MEDIA SHOULD BE CHOSEN BASED ON THE NATURE OF THE SURROUNDING FIRE.

Unusual Fire and Explosion Hazards:

USE WATER SPRAY TO COOL FIRE EXPOSED CONTAINERS TO PREVENT BURSTING.
NFPA Hazard Rating:	(nonsignificant 1=Slight 2=Moderate 3=High 4=Extreme)

Health:2	Flammability:0	lnstability:0	Special:

SECTION V - HEALTH HAZARD DATA Threshold Limit Value:

NOT ESTABLISHED FOR MIXTURE. Effects of Overexposure:

-Acute(Short Term Exposure)

EYE CONTACT: MAY CAUSE IRRITATION SEEN AS TEARING AND REDNESS. SKIN CONTACT: MAY CAUSE

SECTION VIII - SPILL OR LEAK PROCEDURES Steps to be Taken if Material is Released or Spilled:

CONTAIN SPILL AND FLUSH TO SANITARY SEWER. WEAR APPROPRIATE APPROPRIATE CLOTHING. Waste Disposal Method(s):

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL, STATE AND LOCAL REGULATIONS. Neutralizing Agent:

N/A

SECTION IX - SPECIAL PROTECTION INFORMATION

Required Ventilation:

TYPICAL USE OF THE PRODUCT IS OUTDOORS. GENERAL VENTILATION IS NORMALLY ADEQUATE.

Respiratory Protection:

TYPICAL USE OF THIS PRODUCT DOES NOT REQUIRE THE USE OF A RESPIRATOR.

Glove Protection:

NEOPRENE OR NITRILE RUBBER GLOVES IF REPEATED OR PROLONGED SKIN CONTACT IS LIKELY.

Eye Protection:

SAFETY GLASSES IF THE METHOD OF USE PRESENTS THE LIKLIHOOD OF EYE CONTACT.

Other Protection:

GENERAL-DUTY WORK CLOTHES AND SHOES.

SECTION X - STORAGE AND HANDLING INFORMATION

Storage Temperature:	lndoors:X	Outdoors:	Heated:	Refrigerated:

Minimum Temperature:32 F.	Maximum Temperature: 120 F

Precautions to be taken in Handling and Storing:

ALWAYS STORE MATERIAL IN ITS ORIGINAL CONTAINER. KEEP FROM FREEZING. IF PRODUCT FREEZES ALLOW IT TO SLOWLY WARM TO ROOM TEMPERATURE AND STIR THOROUGHLY BEFORE USING.

Other Precautions:

KEEP OUT OF REACH OF CHILDREN. READ THE ENTIRE LABEL BEFORE USING THE PRODUCT. FOLLOW THE LABEL DIRECTIONS.

SECTJON XI - REGULATORY INFORMATION
Chemical Name	CAS Number	Upper % Limit

Those Ingredients listed above are subject to the reporting requirements of 313 of Title III& of the Superfund Amendments and Reauthorization Act of 1986 and 40 CFR part 372.

PLEASE CALL 1-800-527-9919 FOR ADDITIONAL INFORMATION IF YOU ARE A CALIFORNIA CUSTOMER. THIS MSDS IS NOT INTENDED FOR USERS IN THE STATE OF CALIFORNIA

SECTION XII - REFERENCES

- THRESHOLD LIMIT VALUES FOR CHEMICAL SUBSTANCES AND PHYSICAL AGENTS
AND BIOLOGICAL EXPOSURE INDICES, ACGIH, 1999.

- OSHA, PEL.

- REGISTRY OF TOXIC EFFECTS OF CHEMICAL SUBSTANCES, CCINFODisc, 1999.

- VENDOR'S MSDS.

ALL COMPONENTS IN THIS PRODUCT CAN BE FOUND IN THE CURRENT TSCA INVENTORY.

IRR:IRRITANT, FLAM/FLAMM:FLAMMABLE, COMB:COMBUSTIBLE, CORR:CORROSIVE CARC:CARCINOGENIC, TOX:TOXIC, N/A:NOT APPLICABLE, N/E:NOT ESTABLISHED, COC: CLEVELAND OPEN CUP, PMCC:PENSKY-MARTIN CLOSED CUP, TCC:TAGLIABUE CLOSEDCUP, LEL:LOWER EXPLOSION LIMIT, UEL:UPPER EXPLOSION LIMIT, NFPA:NATIONAL FIRE PROTECTION ASSOCIATION, IARC:INTERNATIONAL AGENCY FOR THE RESEARCH ON CANCER, NTP:NATIONAL TOXICOLOGY PROGRAM, OSHA:OCCUPATIONAL SAFETY & HEALTH ADMINISTRATION, ACHGIH:AMERICAN CONFERENCE OF GOVERNMENTAL INDUSTRIAL HYGIENISTS, TLV:THRESHOLD LIMIT VALUE, PEL:PERMISSIBLE EXPOSURE LEVEL, STEL:SHORT-TERM EXPOSURE LIMIT, MLD:MILD, MOD:MODERATE, SEV:SEVERE,

MUTrMUTAGENIC, ASPHYX:ASPHYXIANT, PNOC: PARTICULATES NOT OTHERWISE CLASSIFIED

THE INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORMULATION HOWEVER NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THESE DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF.

MANTEK, DIVISION OF NCH CORP. assumes no responsibility for personal injury or property damage caused by the use, storage, or disposal of the product in a manner not recommended on the product label. Users assume all risks associated with such unrecommended use.storage or disposal of the product."

IRRITATION SEEN AS ITCHING AND REDNESS. INHALATION: MAY CAUSE IRRITATION SEEN AS COUGHING AND SNEEZING. INGESTION: MAY CAUSE IRRITATION WITH POSSIBLE NAUSEA, VOMITING AND DIARR- HEA. INGESTION OF EXTREMELY LARGE AMOUNTS MAY DEPRESS THE CENTRAL NERVOUS SYSTEM AND CAUSE DROWSINESS.

-Chronic (Long Term Exposure)

NO HUMAN CHRONIC EFFECTS KNOWN. TARGET ORGANS: CENTRAL NERVOUS SYSTEM, KIDNEYS MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE: PRE-EXISTING SKIN AND RESPIRATORY CONDITIONS SUCH AS ASTHMA, EMPHYSEMA AND DERMATITIS.

Primary Routes of Entry:	Inhalation:	lngestion:X	Absorption:

Emergency and First Aid Procedures: -Inhalation:

REMOVE FROM THE AREA TO FRESH AIR. SEEK MEDICAL ATTENTION IF RESPIRATORY IRRITATION DEVELOPS OR IF BREATHING BECOMES DIFFICULT.

-Eye Contact:

RINSE THE EYES WITH WATER. REMOVE ANY CONTACT LENSES AND CONTINUE FLUSHING WITH PLENTY OF WATER FOR SEVERAL MINUTES. SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS.

-Skin Contact:

WASH AFFECTED AREAS WITH PLENTY OF SOAP AND WATER FOR SEVERAL MINUTES. SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS.

-Ingestion:

GIVE 3 TO 4 GLASSES OF WATER, BUT DO NOT INDUCE VOMITING. IF VOMITING OCCURS, GIVE FLUIDS AGAIN. SEEK MEDICAL ATTENTION IF DISCOMFORT OCCURS.

-Notes to Physician:

IF A LARGE QUANTITY HAS BEEN INGESTED, TEST FOR HYPOCALCEMIA AND TREAT WITH 10 MG OF 10% CALCIUM GLUCONATE BY SLOW INTRAVENOUS INFUSION, IF NECESSARY.

SECTION VI - TOXICITY INFORMATION

Product Contains Chemicals Listed as Carcinogen or Potential Carcinogen By:
IARC: No	NTP: No	OSHA: No	ACGIH: No	OTHER: No

UREA

ORL-RAT LD50: 8471 MG/KG	3.

STANDARD DRAIZE TEST SKN-HMN: 22 MG/3D-I MLD	3.

SKN-RAT TDLo: 3024 MG/KG-4W-C	3.

IHL-RAT TCLo: 288 MG/M3/17W-I	3.

TUMORIGENIC DATA: ORL-RAT TDLo: 821 G/KG/1Y-C	3.

MUTAGENIC DATA: DNA INHIBITION-HMN LYMPHOCYTE:	600 MMOL/L  3.

DNA DAMAGE-MUS LYMPHOCYTE: 628 MMOL/L	3.

TETRAPOTASSIUM PYROPHOSPHATE

ORL-RAT LDLo: 4640 MG/KG	4.

ORL-RAT LD50: 2444 MG/KG	4.

SKN-RBT LD50: > 4640 MG/KG	4.

13-WEEK FEEDING STUDY ON RATS SHOWED KIDNEY EFFECTS IN HIGH ORAL DOSES. 4.

EYE-RBT SDT: 41.0/110.0/24H  37.0/110.0  SIGNIFICANT IRRITATION  4.

SKN-RBT SDT: 0.8/8.0  MILD IRRITATION     4.

SECTION VII - REACTIVITY DATA

Stability:	Stable:X	Unstable:

Conditions to Avoid:   N/A Incompatibility (Materials to Avoid):

STRONG OXIDIZING AGENTS SUCH AS CHLORINE BLEACH AND CONCENTRATED HYDROGEN PEROXIDE; STRONG ACIDS AND BASES, NITRATES, ALUMINUM.

Hazardous Decomposition Products:

AMMONIA, OXIDES OF CARBON, NITROGEN AND PHOSPHOROUS; BIURET.

Hazardous Polymerization:	May Occur:	Will Not OccunX

Conditions to Avoid:   N/A