1030 Id-01

MAINTENANCE SUPPLY COMPANY, INC.

POST OFFICE BOX 493 HUNTERSVILLE* NORTH CAROLINA 28078

- MATERIAL SAFETY DATA SHEET -COMPLIES WITH USOL SAFETY AND HEALTH REGULATIONS, (29 CFR 1910.120C

EMERGENCY PHONE:  (704) 875 6513

MSDS REFERENCE:    2O630

revision date:    4/13/87

SECTION   1   -    IDENTIFICATION

MANUFACTURED   FOR:       MAINTENANCE   SUPPLY   COMPANY,    I

PoST   OFFICE    BOX  498 HUNTERSVILLE, NC       28078

PR 0 DUCT:	Chewing gum remover

TRADE   NAME:	GUMBO

SECTION   2   -   HAZARDOUS    INGREDIENTS

Composition							%		TLV

Dichlorodifluorimethane	99	1000

section   3 -  physical data

Boiling Point:  -21.6 degrees F.

VAPOR   PRESSure:    (MM HG):  70.2

VAPOR DENSITY (Air =1): 4.28

SOLUBILITY IN WATER: 0*028 WT. %

SPECIFIC GRAVITY (H2O=1): 1.311 @ 25 DEGREES C.

PERCENT VOLATILE bY VOLUME: 1OO

EVAPORATION RATE: N/A

APPEARANCE AND ODORS COLORLESS GAS WITH SLIGHT ETHEREAL JDOR

SECTION 4 - FIRE AND EXPLOSION HAZARDS

FLASH POINT: NONFLAMMABLE

FLAMMABLE- LIMITS: N/A

EXTINGUISHING MEDlA: SUITABIbe for surrounding fire

SPECIAL fire FIGHTting procedures:  None

UNUSUAL Fire AND EXPLOSION hazards:  Product is pressurized and may explode if subjected to extreme heat

section 5 - health hazard data

EFFECTS  of  overexposure:

Eyes: CAN   CAUSE blindness

skini:   can cause frostbite

inhalation:  cause cardiac         arrhythmia and myocardial depression

•

Emergency First Aid Procedures

Eyes: cover with soft

i.eft wool.    call  a physician,

Skin:  cover with soft will, call a physician

INHALATION:   remove to fresh air.  Give artificial respiration.  Call a physician.  Do not administer adrenalin.

SECTION   6   -   REACTIVITY   DATA

Stability:    STABLE

CONDITIONS   TO   AVOID:    HIGH   HEAT INCOMPATIBILITY:   HOT   REACTIVe  Metals

HAZARdous   DECOMPOSITIon products:    PHOSGENE  and  FLUORIDES

HAZ ardous    Polymerization:  Will not occur

SECTION      7   -   SPILL   OR   LEAK   PROCEDURES

STEPS   TO be   TAKEN   [N   CASE    MATERIAL   IS   RELEASED   OB    SPILLed:

ASSURE    adequate ventilation.

wasTE   Dlisposal:   dispose   AS   SOLID   WASTE.  Do not   PUNCTUre   qr INCINERATE   CANS*

SECTION   8   -   SPECIAL   PROTECTION I     ventilation:    LoCAL    EXHAUST    Recommendations

eye  protecti.on:  Good practice

SECTION   9   -   PRECAUTIONS:      HANDLING*   STORAGE   AND   USAGE

PReCAUTIONS   TO  be  taAKEN    [N handling   AND   STorage:   Do   NOT   STORE above 120 degrees F.