S.C. Johnson Wax

4=Very High

3=High

2=Moderate

l=Slight

0=Insignificant

HAZARD RATING
HMIS	NFPA

Racine, Wisconsin 53403-5011

| Phone:(414) 631-2777 J | Health | 3 |
| --- | --- | --- |
| 1 | Flamrnability | 1 |
| 0 | Reactivity | 0 |

Emergency Phone:(800) 228-5635

MATERIAL SAFETY DATA SHEET

SECTION I-PRODUCT IDENTIFICATION

| >ROOUCT NAME: TRIAD II |  | GERMICIDAL |  | CLEANER AND | DEODORANT |  |  | PRODUCT CODE 13960- |  | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IKEMICAL NA | Oli COMMON |  | NAME | pATE ISSUED: : 11/14/91 |  | SUPERSEDES: 09/19/91 | PREPARED BY: Terry A. Chemical |  | Meyers Info. | Adm. |

SECTION II-INGREDIENT INFORMATION

| INGREDIENTS | WEIGHT % | EXPOSURE LIMIT |
| --- | --- | --- |
| Quaternary Ammonium Compound (CAS# 68424-95-3) | 1-3 | NE |
| Juaternary Ammonium Compound (CAS# 68424-85-1) | 1-3 | NE |
| Ethoxylated Alcohols (CAS# 68131-39-5) | 1-5 | NE |
| Water (CAS# 7732-18-5) | balance | NE |
| Sodium Metasilicate (CAS# 6834-92-0) | 0.2-0.8 | None |
| Petrasodium Salt of EDTA (CAS/ 64-02-8) | 1-3 | NE |
| See Regulatory Information (Section XII) for explanation of bracketed information. |  |  |

SECTION III-PHYSICAL DATA

| kpp£A*ANC£/cooR: Clear, green liquid with mild mint odor. | JPECIFIC GRAVITf (H20-1): 1.02 |  |
| --- | --- | --- |
| /APOR PRESSURE (on Kj): 17 (@ 20 C) | PERCENT VOLATILE BY VOLUME (X): | 89-90 |
| solubility in water: Complete | VAPOR DENSITY (Air*!): < 1 |  |
| FEEZING POINT CF): about 32 | BOILING POINT CF): ND |  |
| M: 13 | EVAPORATION RATE (Butyl Acetate*!): | ND |
| /OC (as packaged. «inus K20): ETHANOL | THEORETICAL VOC (lb/g«l): ND |  |

SECTION IV-FIRE AND EXPLOSION INFORMATION

| :LASH POINT Cf) |  | (Method |  | Used): | ND |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lAmABU LIMITS: |  |  | NU |  |  |  |  |  |  |  |  |
| iXTIKOJISHING MEDIA: |  |  |  | Foam. | C02. | Dry | Chemical. |  | Water |  | Fog. |
| SPECIAL | riREFIGHTING |  | PROCEDURES: |  | Normal |  | fire | fighting |  | procedures may be used. |  |
| MUSUAL | FIRE AND | EXPLOSION HAZARDS: |  |  |  | No special |  | hazards |  | known. |  |

SECTION V-HEALTH HAZARD DATA

«oute of entry:   Eye contact. Skin contact.

signs ato symptoms:  Direct contact of product with eyes can cause irritation. Prolonged or repeated contact of undiluted product with skin may cause irritation. Product may cause irritation and nausea if taken internally.

'irs! »it> P80CEDURES:   Flush eyes with water for 15 minutes. FE irritation persists, seek medical aid. Wash skin with plenty of water. If irritation persists, seek medical aid. If product is swallowed, drink large amounts water or milk and seek medical aid.

MEDICAL  CONDITIONS  GENERALLY  RECOGNIZED  AS  BEING AGGRAVATED  BY  EXPOSURE:

SECTION VI-REACTIVITY DATA

| stability: Stable |  |
| --- | --- |
| STABILITY-CONDITIONS TO AVOID: | None known |
| INCOMPATIBILITY: ND |  |
| <A2A«DOUS DECOMPOSITION PRODUCTS: combustion. | When exposed to fire, produces normal products of |
| (AZASDOUS POLYMERIZATION: Will | not occur. |
| (AZARDOUS POLYMERIZATION-CONDITIONS TO AVOID: NA |  |

SECTION VII-SPILL OR LEAK PROCEDURES

steps 10 be taken ik case material is released o« spilled:       Do not let spilled or leaning
material enter watercourse. Absorb with oil-dri or similar inert material.
Sweep or scrape up and containerize. Hay be toxic to aquatic life.

waste disposal information:    PESTIClDAL WASTE - Observe all applicable Federal/State regulations and local ordinances regarding disposal of pesticide wastes.

SECTION VIII-SPECIAL PROTECTION INFORMATION

!ESPiRjuc*r protection:   No special requirements under normal use conditions.

/entilation:General room ventilation adequate.

If prolonged or repeated contactis possible: Rubber.

'ROTECTIVE  GLOVES:

^eoprene.

;te PKOTECTIOM:  For undiluted product: Chemical workers splash-proof goggles Eye protection not required unless contact with undiluted product is oossible.

Use  good  personal  hygiene  practices.

JTHER  PTOTECIIVE  MEASURES:

SECTION   IX-6PECIAL   PRECAUTIONS

3RECAUTIOKART labeling:        DANGER:   Keep  out  of   reach  of   children.   CORROSIVE.   Causes eye and skin damage.   Do not get  in eyes,   on skin,   or on clothing.   Wear goggles  or  face  shield and rubber gloves when handling.   Harmful   if swallowed.   Avoid  contamination of   food.

other handling and storage conditions:            Product  residue may  remain  on/in  empty
containers.   All  precautions   for  handling   the  product  must   be   used   in
landling  the  empty  container  and  residue.   Wash   thoroughly   after  handling.
Keep   from   freezing.   Keep  out   of   reach   of   children.

SECTION   X-ADDITIONAL   INFORMATION

additional information:         EPA  Registration  No.:      6836-73-4822.    EPA   EST.   No.: 4822-WI-l.   This  MSDS   also  covers   product  codes:      3961   and   3964.

SECTION   XI-TRANSPORTATION   INFORMATION

>ot class:     Corrosive   Liquid.

71NA   1760	'

shipping name:       Disinfectants.   Corrosive  material.

dot kotes:     Containing:   Quaternary Ammonium Compounds.

KA-Hot  Applicable,   NE-Not  Established,   NSR-Uo  Special   Requirement,   NO-Kot  Determined

The information herein it given in good faith.  No warranty expressed or  implied is made.  Any use of  these data and information must be determined by the user to be  in accordance with applicable  Federal,   State,   and  local   laws and regulations. The  information contained  in  this   form  is  confidential   and   is  submitted solely  for your  organization's   internal   use.