Material Safety Data Sheet

Required under USOL Safety and Health Regulations lor Shipyard Employment (29 CFR 1915)

U.S. Department of Labor

Occupational Safety and Health Administration

OMB No 1218-0074 Expiration Date 05/31 /86

7-22-86

R

Section I

Manufacturer's Name

HANDI-CLEAN PRODUCTS, INC.

'dress (Number, Street. City. State, and ZIP Code)

301   SWING ROAD

Chemical Name and Synonyms

Emergency Telephone Number

(919)   292-3083

GREENSBORO, N. C. 27402

Trade Name

andSynonymi     FORMULA  H4C   DEODORANT

Chemical	Formula

Family    ORGANIC AEROMATICS	See   Section   II

Section II - Hazardous Ingredients

PPM K,     TLV (Units)

TLV (Units)

Isopropyl Alcohol

400

CAS NO. 67-63-0

Mineral Oil

300

CAS NO. 64742-48-9

HAZARD RATING

0	• MINIMAL HAZARD

1	SLIGHT HAZARD

- • MODERATE HAZARD

- • SERIOUS HAZARD

- - SEVERE HAZARD

Section III - Physical Data

| Boiling Point (°F) | 180°F | Specific Gravity (H2O1) | .805 |
| --- | --- | --- | --- |
| ^apor Pressure <a 70°F | 31 | Percent Volatile by Volume (%) | 96.5 |
| Vapor Density (AIR=1) | Liauid | Evaporation Rate Water =i\ | .22 |

Solubility in Water

.10

| Appearance and Odor Light Amber - Strong | Eucaliptis Orange Odor |
| --- | --- |
| Section IV - Fire and Explosion Hazard Data |  |
| Flash Point (Method Used) 70°F Open Cup | Flammable Limits' Lei Uel 1.10 6.10 |
| Extinguishing Media Dry Chemcial Filled or C02 |  |
| Special Fire Fighting Procedures Hand extinguisher or | sand |
| Unusual Fire and Explosion Hazards Keep awav from open flames |  |
| Page 1 (Continued on Reverse Side) fw™ OSHA 20 (Rev 3/84) |  |

| SECTION V - HEALTH HAZARD DATA |  |  |
| --- | --- | --- |
| THRESHOLD LIMIT VALUE | 300 PPM |  |
| EFFECTS OF OVEREXPOSURE | None |  |
| EMERGENCY AND FIRST AID PROCEDURES In case child or other person drinks product; DO NOT INDUCE VOMITING. |  | Call Physici? |

SECTION VI - REACTIVITY DATA

STABILITY

UNSTABLE

CONDITIONS TO AVOID

STABLE

incompatability (iMairriais lu avonlj

X

None

HAZARDOUS DECOMPOSITION PRODUCTS

None

HA7ARDOUS POLYMERIZATION

MAY OCCUR

WILL NOT OCCUR

X

CONDITIONS TO AVOID

| SECTION | VII - SPILL OR LEAK | PROCEDURES |
| --- | --- | --- |
| STEPS TO BE TAKEN IN CASE MATERIAL IS Stop at source. Mop up with | RELEASED OR SPILLED rags or towels. |  |
| WASTE DISPOSAL METHOD Bury in approved landfill away from public water. |  |  |

| SECTION VIII - SPECIAL PROTECTION INFORMATION |  |  |
| --- | --- | --- |
| RESPIRATORY PROTECTION {Specify tvpf) None needed - deodorant only |  |  |
| VENTILATION Normal | LOCAL EXHAUST Normal | SPLCIAL None |
|  | MECHANICAL Idflll-rull Not applicable | '""" N/A |
| PROTECTIVE GLOVES EYL PROTF Not needed Not |  | needed |
| OTHE.R PROTECTIVE7 EQUIPMENT Not needed |  |  |

| SECTION IX - SPECIAL | PRECAUTIONS |
| --- | --- |
| PRECAUTIONS TO BE. TAKEN IN HANDLING AND STORING Always store in orginal container. |  |
| OTHER PRECAUTIONS Keeo awav from onen flames. |  |

PAGE (2)

GPO   934.110

Form OSHA-20