| MATERIAL SAFETY DATA SHEET N/A = Not Applicable (Prepared According to 29 CFR 1910.1200) |  | Effectlv* Dale q/RR |
| --- | --- | --- |
| SECTION"! -PRttmJCriDENllI-TCATlON |  |  |
| Product Name: SUNBRITE STRIPPER Generic Name: Non-Ammoniated Stripper Supplier* Name: Saffelle, Inc. = ' Supplier* Address: P.O. Box, 2657, Chapel Proper Shipping Name: Compound Wash NOI Liquid | Chemical Family: N/A PhorwNumber: (919)929-3430 Formula: N/A Hill, NC 27515-2657 / | NPCA Hazardous Material* Identification System |
|  |  | Health 2 |
|  |  | Flammabillty 0 |
|  |  | Reactivity 0 |
|  |  | Maximum Personal Protection B |
|  | SECTION 2 - INGREDIENTS |  |
| Chamkal Name Water 2-Butoxyethanol Sodium Tripolyphosphate Sodium Hydroxide Sodium Tallate Sodium Metasilicate Sodium Xylene Sulfonate Monoethanolamine Anionic Surfactant Nonionic Surfactant , Perfume Oils Dye | CAS. NO. WT.% PEL TWA-TLV STEL-TLV CARCINOGEN 7732-18-5 to 100 No 111-76-2 Oto5 50ppm 25 ppm N/A No 7758-29-4 0 to 5 No 1310-73-2 Oto5 2mg/M3 2mg/M3 N/A . .No 61790-45-2 Oto5 No 10213-79-3 Oto5 . No 1300-72-7 Oto5 No . 14143-5 ' Oto5: 3 ppm 3 ppm 6 ppm No 25155-30-0 Oto5 No 26027-38-3 Oto5 No . . s 100-52-7 Oto5 . . -No N/A Oto5 , No |  |

SECTION 3 - PHYSICAL DATA

Boiling Range (°F):	215 °F initial       Specific Gravity:	1.07

Vapor Pressure:	20mmHg(5>680FVapor Density:	(Air=1):>1

% Volatile:	80+	pH:	13.3

Solubility In Water:	100%	Evaporation Rate:	(Water=1):<1

Physical DescriptiomThin red liquid with characteristic odor

SECTION 4 - FIRE AND EXPLOSION HAZARD DATA

Upper explosive limit: N/A Lower explosive limit: N/A

N/A

Flash Point (Method Used):

Extinguishing Media:	N/A

Special F1r» Fighting Procedures:      Not flammable

Unusual Fire and Explosion Hazards: N/A

? - REACTTVnYPflTA

Stability:	Stable	Hazardous PolymertzatIon:Will not OCCUr

Incompatibility (Material* to Avoid):     Strong adds Hazardous Decomposition Products: Alkaline vapors in a fire

Precautions To Be Taken In Handling and Storage:

Keep out of reach of children. For use by trained personnel only.

Keep container closed during storage. For institutional and industrial use onl;

Effects of Overexposure Primary Route of Entry:

Skin:

Severe skin irritant. May cause moderate reddening,

swelling and possible skin damage.

Severe eye irritant. Liquid and mists may damage the

Eyes:

eyes, causing corneal damage.

Vapors may be irritating to mucous membranes in the

Inhalation:

nose, throat and lungs.

May cause headache, dizziness, nausea, vomiting and diarrhea. May also cause red blood cell hemolysis and possible liver, kidney and lung damage.

Ingestlon:

First Aid Procedures:	•"',••'••

skin:         Flush with large quantities of water. Seek medical attention if
irritation persists.	-	. •   .  •

Eyes:        Flush eyes with large quantities of water. Seek medical attention if irritation persists.

inhalation:   Get to fresh air. Seek medical attention if irritation persists.

Ingestlon:   Drink two glassesof water, induce vomiting immediately. Seek medical attention.

No special requirements

Respiratory Protection:

Protective Gloves:	Waterproof recommended

Other Protective Equipment:      N/A

Ventilation:

Provide local exhaust to keep TLV of Section 2 ingredients
below acceptable limit.
Eye Protection:    Safety glasses recommended	N	'     '• ••

Steps To Be Taken In Cass Material Is Released or Spilled:

Mop up or otherwise absorb and hold for disposal

Waste Disposal Method:

Any method in accordance with applicable laws

Manufactured By:

Betco Corporation    1001 Brown Avenue    Toledo, Ohio 43607    419-241-2156

Product Name & Number: Geraldine    #104