Material Safety Data Sheet

May be used to comply with

OSHA's Hazard Communication Standard, ' CFR 1910.1200. Standard must be nsulted for specific requirements.

U.S. Department of Labor

Occupational Safety and Health Administration (Non-Mandatory Form) Form Approved OMB No. 1218-0072

*JTITY (As Used on Label and List)

SHEILA  SHINE  AEROSOLS

Note: Blank spaces are not permitted. If any item is not applicable, or no information is available, the space must be marked to indicate that.

Section I

| Manufacturer's Name ..SHEILA SHINE, INC. | Emergency Telephone Number 303/893-7774 or 303/629-1123 |
| --- | --- |
| Address (Number, Street, City, State, and ZIP Code) 1201 N. W. First Avenue | Telephone Number for Information 305/379-1881 |
| Miami, Florida - 33136-2807 | Date Prepared September 13, 1994 |
|  | Signature of Preparer (optional) |

Section II — Hazardous Ingredients/Identity Information

Hazardous Components {Specific Chemical Identity; Common Name(s))        OSHA PEL

Perchloroethylene  CAS   127-18-4

ACGIH TLV

•100  ppm

Other Limits Recommended

% (optional)

18-20

Methyl Salicyclate  CAS   119-36-8

Apx.1%

Xylene  CAS   1330-20-7

100 ppm.

18-20

Heavy Paraffinic Distillate   CAS   64742-54-7

Propellent:     C0?  -  CAS   #124-38-9

5000  ppm

Not   known

59-61

Ml  components  are  on  the  TSCA list.

Section III — Physical/Chemical Characteristics

| Boiling Point | N/A | Specific Gravity (HzO = 1) | .983 @ 25°c |
| --- | --- | --- | --- |
| Vapor Pressure (mm Hg.) | 3.9 | Melting Point | N/A |
| Vapor Density (AIR =- 1) | 5.4 | Evaporation Rate (Butyl Acetate • 1) | N/A |

Solubility in Water

Insoluble

Appearance and Odor

Solvent - Odor - Wintergreen

| Section IV — Fire and Explosion Hazard | Data |  |  |  |
| --- | --- | --- | --- | --- |
| Flash Point (Method Used) 110°F |  | Flammable Limits N/A | LEL N/A | DEL N/A |

Extinguishing Media

Dry Chemicals — Foam

Special Fire Fighting Procedures Wear  self-contained  breathing  apparatus,   helmet  w/shield,   boots   and  coat

L        al Fire and Explosion Hazards

from oxidizers , open flames or sparks.

Rating:     Health  2	Flammability   2	Reactivity  0	Special  —

(Reg£OJjiu,ce locally)	OSHA 174, Sept. 1985

SHEILA SHINE AEROSOLS - continued

Section V — Reactivity Data

| Stability | Unstable |  | Conditions lo Avoid Isolate from oxidizers, | open flame and sparks | (electric equi |
| --- | --- | --- | --- | --- | --- |
|  | Stable | X |  |  |  |

Incompatibility (Materials to Avoid)

Isolate  strong  oxidizers,   such as   chromates,   peroxides.

Hazardous Decomposition or Byproducts

Carbon Monoxide,   Carbon Dioxide

| Hazardous Polymerization | May Occur |  | Conditions to Avoid Oxidizers - open | flames - sparks |
| --- | --- | --- | --- | --- |
|  | Will Not Occur | X |  |  |

Section VI — Health Hazard Data

Route(s) of Entry:

Ingestion?

A

Inhalation? X

Skin? X

Health Hazards (Acute and Chronic) Eye   Contact:      Flush with water   tor   i:>   fUnutes-L-oncact   physician. ' TphalaMnn;     Rpmnvp  to  fresh  air-if breathing  is   difficult,   give   oxygen. Skin  Contact:     Remove  contaminated  clothing  - wash  thoroughly with soap   fc water   for  15

Swallowing:     If   swallowed-do not

Carcinogenicity:

NTP?

induce vomiting -   call physician  immediately.

OSHA Regulated?

IARC Monographs?

N/A

N/A

N/A

Signs and Symptoms of Exposure Dizziness  -  Nausea

Medical Conditions

Generally Aggravated by Exposure

Vomiting - Diarrehea, irritation to eyes, nose and throat.

Emergency and First Aid Procedures

Section Vli — Precautions for Safe Handling and Use

Steps to Be Taken in Case Material Is Released or Spilled	. ,	. .	,

Use  absorbent material  to  soak up   spill-Dike  area  if  needed,   Ventilate  area,   Mop  up   and

dispose  of.     Follow hazard  precaution  as   prescribed   by  State,. Local  and  Federal       ~~

afinn.s . _ ; _ '. _ . -

Waste Disposal Method Recycle  or  dispose  of-observing     Local,   State  and Federal Regulations.

Precautions to Be Taken in Handling and Storing Store  in  cool,   well ventilated  area.     Container  should be  tightly  sealed.

Other Precautions

Section VIII — Control Measures

Respiratory Protection (Specify Type) If needed,   use MSHA/NTUSH Respirator   organic vapors.

| Ventilation | Local Exhaust Necessary | Special |  |
| --- | --- | --- | --- |
|  | Mechanical (General) | Other |  |
| Protective Gloves Eye Protection Chemical Resistance |  |  | Safety glasses |

Other Protective Clothing or Equipment

Work/Hygienic Practices

This information is offered in good faith but without guarantee.  « U.S.G.P.O.: 1986-491-529/4.5775

Conditions   are beyond  out