Manufacturer Name and Address:

Weyerhaeuser Company

Tacoma WA 98477

Emergency Phone: (206) 924-5000

Additional Information: (206) 924-3865

Material Safety Data Sheet

Urea Formaldehyde (UF) Bonded Products

Product Identification

| Product | Manufacturing Location |
| --- | --- |
| Agriboard | Sprinafield, OR |
| Door Core Particleboard | Adel, GA Marshfield, Wl |
| Medium Density Fiberwood | Moncure, NC |
| Microboard™ | Moncure, NC |
| Particleboard | Adel, GA Klamath Palls, OR Sprinqfield. OR |

board door core, chipboard, flakeboard

Synonyms:    •

Date Prepared: Date Revised: Prepared by:

07/26/89

06/20/94

Corporate Safety & Health

j| Hazardous Ingredients/Identity Information

| Chemical or Common Name CAS* | Percent | Exposure Limits1 |  |
| --- | --- | --- | --- |
| Wood CAS# None | 85-94 | OSHA PEL-TWA OSHA PEL-STEL ACGIH TLV-TWA ACGIH TLV-STEL ACGIH TLV-TWA OSHAPEbTWA | 5 mg/m3 (a) 10 mg/m3 (a) 5 mg/m3 (b) 10 mg/m3 (b) 1 mg/m3 (c) 2.5 mg/m3 (d) |
| Urea formaldehyde resin solids* CAS# 901 1-05-6 | 4-15 | OSHA PEL-TWA OSHA PEL-STEL ACGIH TLV-STEL C | 0.75 ppm (e) 2 ppm (e) 0.3 ppm (e) |
| Paraffin Wax CAS# 8002-74-2 | 0-1 | OSHA PEL-TWA ACGIH TLV-TWA | 2 mg/m3 (f) 2 mo/m3 (f) |

- softwood or hardwood total dust

- softwood total dust

- selected hardwood total dust (beech, oak, others)

- Western red cedar total dust

- tree gaseous formaldehyde

- paraffin wax fume

' Based on 1989 OSHA Permissible Exposure Limits (PEL) 1 Contains less than 0.1 % free formaldehyde

Appearance and Odor:

A matrix of light brown or buff-colored interlocking wood fibers/ particles/flakes having a slightly aromatic odor. The wood component of these products may consist of alder, aspen, beech, birch, cottonwood, fir, gum, hemlock, hickory, maple, oak, pecan, pine, poplar, spruce, walnut and/or Western red cedar.

•El Physical/Chemical Characteristics

BOILING POINT (1 atm):	NAP

VAPOR PRESSURE (mm Hg):	NAP

VAPOR DENSITY (Air=1; 1 atm):	NAP

SPECIFIC GRAVITY (H2O=1):	0.4O0.90

MELTING POINT:	NAP

EVAPORATION RATE (Butyl Acetate=1):	NAP

SOLUBILITY IN WATER (% by Weight):	<0.1

% VOLATILE BY VOLUME:	0

Fire and Explosion Hazard Data

. 'F)a?h Point (Method Used):; NAP .; {. --Cv >; '•^^'•••^^'••.^•^••••:':::^ Flammable Limits:

LEL: See below under "Unusual Fire and Explosion Hazards"

UEL: NAP Extinguishing Media:

Water, carbon dioxide, sand or dry chemical. Autoignition Temperature:

400°F-500°F (204°C-260°C). Special Rrefighting Procedures:

None. Unusual Fire and Explosion Hazards:

Depending on moisture content, and more importantly, particle

diameter, wood dust may explode in the presence of an ignition

source. An airborne concentration of 40 grams (40,000 mg) of

dust per cubic meter of air is often used as the LEL for wood

dusts.

Q Reactivity Data

Stability:

() Unstable	(x) Stable

Conditions to Avoid: NAP Incompatibility (Materials to Avoid):

Avoid contact with oxidizing agents. Avoid open flame. This product may ignite at temperatures in excess of 400°F (204°C).

Hazardous Decomposition or By-Products: Thermal decomposition products include carbon monoxide, carbon dioxide, aliphatic aldehydes, rosin acids, terpenes, and polycyclic aromatic hydrocarbons.

Hazardous Polymerization:
() May Occur	(x) Will Not Occur

H Precautions for Safe Handling and Use

Steps to be Taken In Case Material Is Released or Spilled:

Not applicable for product in purchased form. Wood dust generated from sawing, sanding, drilling or routing of this product may be vacuumed or shoveled for recovery or disposal. Avoid dusty conditions and provide good ventilation. Use NIOSH/MSHA-approved respirator and goggles where ventilation is not possible.

MS DSWC182-07 06/94

0 Precautions for Safe Handling and Use, cont.

Waste Disposal Method:

If disposed of or discarded in its purchased form, incineration is preferable. Dry land disposal is acceptable in most states. It is, however, the user's responsibility to determine at the time of disposal whether your product meets RCRA criteria for hazardous waste. Follow applicable federal, state and local regulations.

Precautions to be Taken In Handling and Storage: No special handling precautions are required. Keep in cool, dry place away from open flame. This product may release small quantities of gaseous formaldehyde. Store in well ventilated area.

Other Precautions:

A NIOSH/MSHA-approved full-face respirator or half-mask respirator with chemical goggles must be worn when the formaldehyde and/or wood dust exposure limits are exceeded. It is recommended that the full-face and half-mask respirators have a combination formaldehyde and dust cartridge.

Health Hazard Data

Primary Health Hazards:

The primary health hazards posed by this product are thought to be due to exposure to wood dust or free gaseous formaldehyde. Primary Route(s) of Exposure: () Ingestion:

(x) Skin:	Dust

(x) Inhalation:	Dust or gas

Acute Health Hazards — Signs and Symptoms of Exposure/ Emergency and First-Aid Procedures: INGESTION: Not applicable with normal use. EYE CONTACT: Gaseous formaldehyde may cause temporary irritation or a temporary burning sensation. Wood dust may cause mechanical irritation. Treat dust in eye as foreign object. Flush with water to remove dust particle. Get medical help if irritation persists.

SKIN CONTACT: High concentrations of gaseous formaldehyde may cause allergic contact dermatitis in sensitized individuals resulting in redness, itching, and occasionally, hives. Wood dust of certain species may elicit allergic contact dermatitis in sensitized individuals, as well as mechanical irritation. These products may be irritating to the skin from drying or mechanical abrasion experienced during frequent handling. Get medical help if rash, irritation, or dermatitis persists.

SK)N ABSORPTION: Not known to occur with normal use. INHALATION: Gaseous formaldehyde may cause temporary irritation to the hose and throat. Wood dost rnay cause unpleasant deposit/obstruction in the nasal passages, resulting in dryneSs of nose, dry cough and headaches. Remove to fresh air. Get medical help if persistent irritation, severe coughing, or breathing difficulty occurs.

Medical Conditions Generally Aggravated by Exposure: Gaseous formaldehyde or wood dust may aggravate preexisting respiratory conditions or allergies. Chronic Health Hazards:

Wood dust(s), depending on the species, may cause allergic
contact dermatitis with prolonged, repetitive contact, and respira
tory sensitization after prolonged exposure to elevated dust levels.
Carcinogenicity Listing:
(x) NTP:	Formaldehyde

(x) IARC Monographs:	Formaldehyde

(x) OSHA Regulated:	Formaldehyde

Gaseous formaldehyde has been shown t.b cause cancer in certain laboratory animals after long-term exposure to very high concentrations (14+ ppm), far above those normally found in the workplace with this product. Wood dust has been alleged to cause nasal/ paranasal sinus cancer (certain European hardwoods: oak and beech).

n Control Measures

Personal Protective Equipment:

RESPIRATORY PROTECTION - Not applicable for product in purchased form. A NIOSH/MSHA-approved respirator is recommended when allowable exposure limits may be exceeded. PROTECTIVE GLOVES - Not required.  However, cloth, canvas or leather gloves are recommended to minimize potential mechanical irritation from handling product.

EYE PROTECTION - Not applicable for product in purchased form. Goggles or safety glasses are recommended when machining this product.

OTHER PROTECTIVE CLOTHING OR EQUIPMENT -- Not applicable for product in purchased form. Outer garments may be desirable in extremely dusty areas. WORK/HYGIENE PRACTICES -- Follow good hygienic and housekeeping practices. Clean up areas where dust settles to avoid excessive accumulation of this combustible material. Minimize blowdown or other practices that generate high airborne-dust concentrations. Ventilation:

LOCAL EXHAUST - Provide local exhaust as needed so that exposure limits are met.

MECHANICAL (GENERAL) -- Provide general ventilation in processing and storage areas as needed so that exposure limits are met.

SPECIAL -- Self-contained breathing apparatus (SCBA) recommended when fighting fire. OTHER - NAP

H Transportation Data

DOT Proper Shipping Name:	Not Regulated

Hazard Class/Division Number:

ID Number:

Packing Group:

Label/Placard Required:

DOT Hazardous Substance:

•TO User's Responsibility

The information contained in this Material Safety Data Sheet is based
on the experience of occupational health and safety professionals
and comes from sources believed to be accurate or otherwise
technically correct. It is the user's responsibility to determine if this
information is suitable for their applications and to follow safety
precaptions as may be necessary. The user has the responsibility to
make sure that this; sheet is the most up-to-date issue,	V

Additional Information

Definition of Common Terms:

ACGIH	= American Conference of Governmental Industrial

Hygienists

C	=  Ceiling Limit

CAS#	=   Chemical Abstracts System Number

IARC	=  International Agency for Research on Cancer

MSHA	=  Mining Safety and Health Administration

NAP	=   Not Applicable

NAV	=  Not Available

NIOSH	=   National Institute for Occupational Safety and Health

NTP	=   National Toxicology Program

OSHA	=  Occupational Safety and Health Administration

PEL	=  Permissible Exposure Limit	.

Sf EL	=  Short-Term Exposure Limit (15 minutes)

TLV	= Threshold Limit Value

TWA	=  Time-Weighted Average (8 hours)

MSDS WC182-07 06/94

Manufacturer Name and Address:

Weyerhaeuser Company

Tacoma WA 98477

Emergency Phone: (206) 924-5000

Additional Information: (206) 924-3865

Material Safety Data Sheet

Phenol Formaldehyde (PF) Bonded Products

| Product Identification

| Product | Manufacturing Location |
| --- | --- |
| Hardboard Siding | Klamath Falls, OR |
| Ply Veneer | Springfield, OR |
| Plywood | Dierks, AR Klamath Falls, OR Millport, AL Mountain Pine, AR Philadelphia, MS Plymouth, NO Wright City, OK |

Medium Density Siding

Synonyms: Date Prepared: Date Revised: Prepared by:

07/27/89

06/22/94

Corporate Safety & Health

] Hazardous Ingredients/Identity Information

| Chemical or Common Name CAS# | 'ercent | Exposure Limits1 |  |
| --- | --- | --- | --- |
| Wood CAS# None | 84-99 | OSHA PEL-TWA OSHA PEL-STEL ACGIH TLV-TWA ACGIH TLV-STEL ACGIH TLV-TWA OSHA PEL-TWA | 5 mg/m3 (a) 10mg/m3 (a) 5 mg/m3 (b) 10 mg/m3 (b) 1 mg/m3 (c) 2.5 mg/m3 (d) |
| 2Linerboard (paper Itoer/cellulose; CAS# 9004-34-6 | 17-23 | OSHA PEL-TWA OSHA PEL-TWA ACGIH TLV-TWA | 15 mg/m3 (e) 5 mg/m3 (f) 10 mg/m3 (e) |
| 'Phenol Formaldehyde Resin Solids GAS#None | 1-15 | OSHA PEL-TWA OSHA PEL-STEL ACGIH TLV Ceiling ••'.- .'".". . • • ". '"".'.. | 0.75 ppm (g) 2 ppm 5(g) 0.3 ppm (g) '• "•-. ..-'-; ' -. '• .'.. |
| 'Cured Acrylic Paint CAS# None | 0-2 | OSHA PEL-TWA ACGIH PEL-TWA | None None |
| 'Water-based Primer or Sealer CAS# None | 1-2 | OSHA PEL-TWA ACGIH PEL-TWA | None None |
| 'Wax CAS# 8002-74-2 | 0-2 | OSHA PEL-TWA ACGIH TLV-TWA | 2 mg/m3 (h 2 mg/m3 (h |
| 4Top Spray (melamine urea formaldehyde) CAS# None | <1 | OSHA PEL-TWA OSHA PEL-STEL ACGIH TLV Ceiling | 0.75 ppm (g 2 ppm (g 0.3 ppm (g) |

- softwood or hardwood total dust

- softwood total dust

- selected hardwood total dust (beech, oak, others)

- Western red cedar total dust

- total dust

(!) respirable dust

(g) free gaseous formaldehyde

(h) paraffin wax fume

'Based on 1989 OSHA Permissible Exposure Limits (PEL)

Try Veneer

This product contains less than 0.05% free formaldehyde

'Hardboard Siding

Appearance and Odor:

Plywood is a 3-9 ply-veneer product with a slightly aromatic resinous odor and natural wood color. Hardboard siding consists of a ligno cellulosic matrix of interlocking wood fibers, having a slightly aromatic odor and painted surface. Ply Veneer is wood veneer laminated with kraft paper with a slightly aromatic odor. The wood component of these products may consist of alder, aspen, beech, birch, cottonwood, fir, gum, hemlock, hickory, maple, oak, pecan, pine, poplar, spruce, walnut and/or Western red cedar.

El Physical/Chemical Characteristics

BOILING POINT (1 atm):	NAP

VAPOR PRESSURE (mm Hg):	NAP

VAPOR DENSITY (Air=1; 1 atm):	NAP

SPECIFIC GRAVITY (H2O=1):	0.40-0.90

MELTING POINT:	NAP

EVAPORATION RATE (Butyl Acetate=1):	NAP

SOLUBILITY IN WATER (% by Weight;! atm);	;<0.1% .;.-;;>.;: x:x:-.

& VOLATILE BY VOLUME @ 7O-F (216Cj:	6

Q Fire and Explosion Hazard Data

Rash Point (Method Used): NAP Flammable Limits:

LEL: See below under "Unusual Fire and Explosion Hazards" UEL: NAP

Extinguishing Media: Water, carbon dioxide, sand.

Autoignition Temperature (F° or C°): 400°-500°F (204°C-260°C)

Special Rrefighting Procedures: None

Unusual Fire and Explosion Hazards: Depending on moisture content and more importantly particle diameter, wood dust may explode in the presence of an ignition source. An airborne concentration of 40 grams (40,000 mg) of dust per cubic meter of air is often used as the LEL for wood dusts.

0 Reactivity Data

Stability:

() Unstable	(x) Stable

Conditions to Avoid:   NAP Incompatibility (Materials to Avoid):

Avoid contact with oxidizing agents. Avoid open flame. Product may ignite at temperatures in excess of 400°F (204°C).

Hazardous Decomposition or By-products: Thermal decomposition products include carbon monoxide, carbon dioxide, aliphatic aldehydes, rosin acids, terpenes, and polycyclic aromatic hydrocarbons.

Hazardous Polymerization:
() May Occur	(x) Will Not Occur

|J Precautions for Safe Handling and Use

Steps to be Taken In Case Material Is Released or Spilled:

Not applicable for product in purchased form. Wood dust generated from sawing, sanding, drilling or routing of this product may be vacuumed or shoveled for recovery or disposal. Avoid dusty conditions and provide good ventilation.  Use NIOSH/MSHA-approved respirator and goggles where ventilation is not possible.

Waste Disposal Method:

If disposed of or discarded in its purchased form, incineration is preferable. Dry land disposal is acceptable in most states. It is, however, the user's responsibility to determine at the time of disposal whether your product meets RCRA criteria for hazardous waste. Follow applicable federal, state and local regulations.

Precautions to be Taken In Handling and Storage: No special handling precautions are required. This product may

: • release very small .quantities of fonrjaldehyde in gaseous form.: -.-";!:• •    Under foreseeable conditions of use, these products release less than 0.050 ppm in standard large chamber test conditions. Store " in well-ventilated, cool, dry place away from open flame.

Other Precautions:

A NIOSH/MSHA-approved respirator and goggles should be worn when the allowable exposure limits may be exceeded.

Q Health Hazard Data

Primary Health Hazards:

The primary health hazards posed by this product are thought to be due to exposure to wood dust.

Primary Route(s) of Exposure: () Ingestion:

(x) Skin:	Dust

(x) Inhalation:	Dust	:     .-.•,'-X.•:..'••.;•'.'•

Acute Health Hazards — Signs and Symptoms of Exposure/

Emergency and First-Aid Procedures: INGESTION: Not applicable with normal use. EYE CONTACT: Wood dust may cause mechanical irritation. Treat dust in eye as foreign object. Flush with water to remove dust particles. Get medical help if irritation persists. SKIN CONTACT: Wood dust(s) of certain species can elicit allergic contact dermatitis in sensitized individuals, as well as mechanical irritation resulting in erythema and hives. Get medical help if rash, irritation or dermatitis persists. SKIN ABSORPTION: Not known to occur with normal use. INHALATION: Wood dust may cause unpleasant deposit/ obstruction in the nasal passages, resulting in dryness of nose, dry cough, sneezing and headaches. Remove to fresh air. Get medical help if persistent irritation, severe coughing or breathing difficulty occurs.

Medical Conditions Generally Aggravated by Exposure: Wood dust may aggravate preexisting respiratory conditions or allergies.

Chronic Health Hazards:

Wood dust(s), depending on the species may cause allergic contact dermatitis with prolonged, repetitive contact, and respiratory sensitization after prolonged exposure to elevated dust levels.

Carcinogenicity Listing:
() NTP:	Not listed

() (ARC Monographs:	Not listed

() OSHA Regulated:	Not listed

Wood dust has been alleged to cause nasal/paranasal sinus cancer (certain European hardwoods: oak and beech).

n Control Measures

Personal Protective Equipment:

RESPIRATORY PROTECTION - Not applicable for product in purchased form. A NIOSH/MSHA-approved respirator is recommended when allowable exposure limits may be exceeded. PROTECTIVE GLOVES ~ Not required.  However, cloth, canvas or leather gloves are recommended to minimize potential mechanical irritation from handling product.

EYE PROTECTION - Not applicable for product in purchased form. Goggles or safety glasses are recommended when machining this product.

OTHER PROTECTIVE CLOTHING OR EQUIPMENT -- Not applicable for product in purchased form. Outer garments may be desirable in extremely dusty areas.

WQRK/HYGIENE PRACTICES - Follow good hygienic and   ;...;•: housekeeping practices. ;Cleari up areas where wood dust settles to avoid excessive accumulation of this combustible material. Minimize blowdown or other practices that generate high airborne-dust concentrations. Ventilation:

LOCAL EXHAUST - Provide local exhaust as needed so that exposure limits are met.

MECHANICAL (GENERAL) - Provide general ventilation in processing and storage areas as needed so that exposure limits are met.

SPECIAL - Self-contained breathing apparatus (SCBA) recommended when fighting fire. OTHER - NAP

El Transportation Data

DOT Proper Shipping Name:	Not Regulated

Hazard Class/Division Number:

ID Number:

Packing Group:

Label/Placard Required:

DOT Hazardous Substance:

JJ User's Responsibility

The information contained in this Material Safety Data Sheet is based on the experience of occupational health and safety professionals and comes from sources believed to be accurate or otherwise technically correct. It is the user's responsibility to determine if this information is suitable for their applications and to follow safety precautions as may be necessary. The user has the responsibility to make sure that this sheet is the most up-to-date issue.

Additional Information

Definition of Common Terms:

ACGIH	= American Conference of Governmental Industrial

Hygienists

C	=  Ceiling Limit

CAS#	=  Chemical Abstracts System Number

IARC	=  International Agency for Research on Cancer

MSHA	=  Mining Safety and Health Administration

NAP	=   Not Applicable

NAV	=   Not Available

NIOSH	=  National Institute for Occupational Safety and Health

NTP	=  National Toxicology Program

OSHA	=  Occupational Safety and Health Administration

PEL	=  Permissible Exposure Limit

STEL	=  Short-Term Exposure Limit (15 minutes)

TLV	= Threshold Limit Value

TWA	=  Time-Weighted Average (8 hours)

~

MSDS WC183-08 06/94

Weyerhaeuser