-1

OS/IS/04

D42263502

ACCT:      11182S007 CAT WO:  S712014

PAGE:  1 PO WER:  X30O0012202

.

DATE;     OS/16/04 INDEX:1 . ,042263^0:2

ACCT:      1 11 82SQ.O'7' CAT NO:  S7120X4

PO KTBR:  13000012202

MATERIAL SAFETY DATA SHEET

Sodium Silicate Solution 40181

*•* SECTION 1 - CHEMICAL PRODUCT AND COMPANY IDENTIFICATION * *

MSDS Name: Sodium. Silicate Solution Catalog Numbers :

S712O13,  S712O14,  S712015,  SS338-1,  SS338--5 Synonyms:

XXSODSIL22GM

None Company Identification:

Fisher Scientific

1 Reagent Lane

Fairlawn, Na   07410 For- information,  call:   201-796-7100 Emergency Number-:         201-796-7100 For CHEMTREC assistance,  call:  800-424-9300 For International CHEMTREC assistance,  call:  703-527-3887

SECTION 2  - COMPOSITION,  INFORMATION ON INGREDIENTS

| CAS# | Chemical Name | % | EINECS# |
| --- | --- | --- | --- |
| 1313-59-3 | Sodium oxide | 9.1 | 215-208-9 |
| 7631-86-9 | Silica, amorphous | 29 . 2 | 231-545-4 |
| 7732-18-5 | Water | 61 .7 | 231-791-2 |

Hazard Symbols: C Risk Phrases:  34

* * * *  SECTION 3  - HAZARDS IDENTIFICATION * * * *

EMERGENCY OVERVIEW Appearance: not available liquid. Danger! Causes skin burns. Causes eye burns. Corrosive. May cause

evere respiratory tract irritation with, possible burns, May cause severe digestive tract irritation with possible burns. Target Organs: Respiratory system,  eyes,  skin.

Potential Health Effects Eye:

Causes severe eye burns.  May cause chemical conjunctivitis and

cornea.1 damage . Skin:

Causes skin burns - May cause deep, penetrating ulcers of the skin. Ingestion:

May cause severe and permanent damage to the digestive tract. Causes

gastrointestinal tract burns. May cause perforation of the digestive

tract. Causes severe pain, nausea, vomiting, diarrhea, and shock.

May cause systemic effects. Inhalation:

Irritation may lead to chemical pneumonitis and pulmonary edema.

Causes chemical burns to the respiratory tract. May cause systemic

effects. Chronic:

Prolonged or repeated skin contact may cause dermatitis. Effects may

be delayed.

**** SECTION 4 - FIRST AID MEASURES ****

Eyes :

Get medical aid immediately, Do NOT allow victim to rub or keep eyes closed. Extensive irrigation with water is required (at least 3 0 minutes).

Skin :

Get medical aid immediately. Immediately flush skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes. Wash clothing before reuse. Destroy contaminated shoes.

Ingestion:

Do NOT induce vomiting.  If victim is conscious and alert,  give 2-4 cupfuls of milk or water. Never give anything by mouth to an unconscious person. Get medical aid immediately.

Inhalation:

Get medical aid. immediately. Remove from exposure and move to fresh air immediately. If breathing is dif ficult, give oxygen. Do NOT use mouth-to-mouth resuscitation. if breathing has ceased, apply artificial respiration using oxygen and a suitable mechanical device such as a bag and a mask.

Notes to Physician:

Treat symptomatically and

* * * * SECTION 5  -  FIRE FIGHTING MEASURES * * * *	' f

General Information:

As in any fire , wear a self -contained, breathing apparatus in pressure -demand, MSHA/NIOSH ( approved or equivalent ) , and full protective gear . During a fire , irritating and highly toxic gases may be generated by thermal decomposition or combustion . Wear appropriate protective clothing to prevent contact with skin and eyes . Wear a self-contained breathing apparatus ( SCBA) to prevent contact with thermal decomposition products . Runoff from fire control or dilution water may cause pollution .

Extinguishing Media :

For small fires , use dry chemical , carbon dioxide , water spray or alcohol -res is tant foam .

Autoignition Temperature : Not applicable .

Flash. Point :	KTon- flammable .

Explosion Limits , lower :Not available .

Explosion Limits , upper : Not available .

KTFPA Rating :  (estimated) Health:  3 ; Flammabili ty :  O ;  Instability :  0

SECTION 6 - ACCIDENTAL RELEASE MEASURES * *

Gen

l Information : Use proper personal protective equipment as indicated

in Section 8 . Spills /Leaks :

Absorb spill with ineirt material (e.g. vermiculi te , sand or earth) , then place in suitable container . Avoid runof f into storm sewers and ditches which lead to waterways . Clean up spills immediately, observing precautions in the Protective Equipment section _ Provide ven t i 1 a t i on .

* * * * SECTION 7  - HANDLING and STORAGE * * * *

Handling :

Wash thoroughly after handling .	Use with adequate ventilation . Do

not breathe dust , vapor , mist ,	or gas . Do not get in eyes ,  on skin ,

or on clothing . Keep container	tightly closed . Do not ingest or
inhale . Discard contaminated shoes .

Storage :

Keep container closed when not	in use . Store in a cool ,  dry,

well -ventilated area away from	incompatible substances . Corrosives

area -

**** SECTION 8 - EXPOSURE CONTROLS,  PERSONAL PROTECTION ****

Engineering Controls :

Facilities storing or utilizing this material should be equipped with an eyewash facility and a safety shower . Use adequate general or local exhaust ventilation to keep airborne concentrations below the permissible exposure limits .

Exposure Liirni ts

| C'hemi ca 1 Name | ACGIH | NIOSH | OSHA - Final PELs |
| --- | --- | --- | --- |
| Sodium oxide | none listed | none listed | none listed |
| Silica , amorphous | none listed | 6 mg/m3 TWA 3000 rng/m3 IDLH | none listed |
| Water | none listed | none listed | none listed |

OSHA Vacated PELs: Sodium oxide:

No OSHA Vacated PELs are listed for this chemical. Silica, amorphous:

No OSHA Vacated pels are listed for this chemical. Water:

No OSHA Vacated PELs are listed for this chemical.

Personal Protective Equipment

Eyes :

Wear appropriate protective eyeglasses or chemical safety goggles as described by OSHA's eye and face protection regulations in 29 CFR 1910.133 or European Standard EN166.

Wear appropriate protective gloves to prevent skin exposure.

Wear appropriate protective clothing to prevent skin exposure.

Clothing:

Follow the OSHA respirator regulations found in 29 CFR 1910.134 or European Standard EN 149. Always use a NIOSH or European Standard EN 149 approved respirator when necessary.

Respirators:

SECTION 9  -  PHYSICAL AND CHEMICAL PROPERTIES ****

PAGE:  3 PO NBR;  13000012202

DATE;     OS/16/04 :.-'"-.. . .INDEX:  r>422€3S02 :•"..:•:

111825007 .- S7'12D1.4

Physical State:

Liquid.

not available

none reported.

11 . 2

14 rnm Hg

0. 7

>1 (ether=l)

Not available .

213 deg F

3 1 deg F

Not available .

Soluble in water

1 . 4

Mixture

0

Color:

Odor :

pH:

Vapor Pressure:

Vapor Density:

Evaporation Rate:

Viscosity:

Boiling- Point;

Freezing/Melting Point:

Decomposition Temperature:

Solubility in water:

Specific Gravi ty/Density:

Molecular Formula:

Molecular Weight:

* *** SECTION 10 - STABILITY AND REACTIVITY * ***

Chemical stability:

Stable at room temperature in closed containers under normal storage

and handling conditions.  Sodium oxide reacts on contact with water

forming Sodium Hydroxide. Conditions to Avoid:

None reported. Incompatibilities with Other Materials:

Acids, Corrosive to metals such as aluminum, tin, and zinc. These

reactions may cause the formation of hydrogen gas., Sodium oxide is

reactive with water.. Hazardous Decomposition Products:

Sodium hydroxide.. Hazardous polymeri zation: Has not been reported.

* * ** SECTION 11 - TOXICOLOGICAL INFORMATION * * **

RTECStt:

CAS# 1313-59-3 unlisted.

CAS# 7631-86-9: EU8655000 W7565000

CAS# 7732-18-5:  ZC0110000 LD50/LC50:

Not available.

CAS# 7631-86-9: Draize test,  rabbit,  eye: 25 mg/24H Mild.

CAS# 7732-18-5: Oral,  rat: LD50 = >90 mL/kg. Carcinogen!city: Sodium oxide -

Not listed by ACGIH, IARC, NIOSH, ntp, or OSHA. Silica, amorphous -

IARC: Group 3 carcinogen Water -

Not listed by ACGIH,  IARC,  NIOSH,  NTP,  or OSHA. Epidemiology:

No information found. Teratogenicity:

No information found. Reproductive Effects:

No information found. Neiarotoxici ty :

No information found. Mutagenicity:

No information found. Other Studies:

The toxicological properties have not been fully investigated.

** ** SECTION 12 - ECOLOGICAL INFORMATION ** * *

Ecotoxicity:

Fish: Pseudomonas putida:

* ** * SECTION 13  - DISPOSAL CONSIDERATIONS ** * *

Chemical waste generators must determine whether a discarded chemical is classified as a hazardous waste.

US EPA guidelines for the classification determination are listed in 40 CFR Parts 261 . 3 . Additionally, waste generators must consult state .nd local hazardous waste regulations to ensure complete and accurate classification.

RCRA P—Series: None listed. RCRA U-Series: None listed.

**** SECTION 14 - TRANSPORT INFORMATION ****

US DOT

Shipping Name:	CORROSIVE LIQUID, BASIC,  INORGANIC,

N.O.S.

Hazard Class:	8

UN Number:	UN3266

Packing Group:	II
Canadian TDG

No information	available.

PAGE:  4 ' .

08/16/04 ,i>42263S02

ACC T:      111825007 CAT NO;  S712Q14':;-

PO NBR:  13000012202 SECTION 15  - REGULATORY INFORMATION * * * *

US FEDERAL TSCA

CAS# 1313-59-3 is listed on the TSCA inventory. CAS# 7631-86-9 is listed on the TSCA inventory. CAS# 7732-18-5 is listed on the TSCA inventory. Health & Safety Reporting List

None of the chemicals are on the Health & Safety Reporting List . Chemical Test Rules

None of the chemicals in this product are under a Chemical Test Rule . Section 12b

None of the chemicals are listed under TSCA Section 12b . TSCA Significant New use Rule

None of the chemicals in this material have a SNUR under TSCA . SARA

CERCL.A Hazardous Substances and corresponding RQs

None of the chemicals in this material have an RQ . SARA Section 302 Extremely Hazardous Substances

None of the chemicals in this product have a TPQ . Section 313

No chemicals are report able under Section 313. Clean Air Act :

This material does not contain any hazardous air pollutants . This material does not contain any Class 1 Ozone depletors . This material does not contain any Class 2 Ozone depletors . Clean water Act :

None of the chemicals in this product are listed as Hazardous Substances under the CWA .

None of the chemicals in this product are listed as Priority Pollutants under the CWA.

None of the chemicals in this product are listed as Toxic Pollutants under the CWA . OSHA:

None of the chemicals in this product are considered highly hazardous by OSHA. STATE

Sodium oxide is not present on state lists from CA, PA, MN, MA , FL , or NJ.

Silica , amorphous can be found on the f ol lowing state right to know lists : California , New Jersey,  Pennsylvania , Minnesota , Massachusetts .

Water is not present on state lists from CA, PA, MN, MA, FL, or NJ . California No Sign if icant Risk Level : None of the chemicals in this product are listed . European /International Regulations

European Labeling in Accordance with EC Directives Hazard Symbols : C Risk Phrases :

R 34  Causes burns . Safety Phrases :

S 26  In case of contact with eyes , rinse immediately wi th plenty of water and seek medical advice . S 37/39  Wear suitable gloves and eye /face protection ,

S 45   In case of accident or if you feel unwell ,  seek medical advice immediately ( show the label where possible) .

WGK (Water Danger /Protection) CAS# 1313-59-3 :  1 CAS# 7631-86-9:  0

CAS# 7732-18-5 : No information available . United Kingdom Occupational Exposure Limits

CAS# 7631-86-9: OES-United Kingdom, TWA 6 mg/m3 TWA (total inhalable dust) ; 2.4 mg/m3 TWA (respirable dust)

Uni ted Kingdom Maximum Exposure Limi ts

Canada

CAS#	1313-59-3	is	listed on Canada ' s DSL List .

CAS#	7631-86-9	is	listed on Canada ' s DSL List .

CAS#	7732-18-5	is	listed on Canada ' s DSL List .

This	product has a WHMIS classification of E .

CAS#	1313-59-3	is	not listed on Canada ' s Ingredient Disclosure Lis t .

CAS#	7631-86-9	is	listed on Canada ' s Ingredient Disclosure Lis t .

CAS#	7732-18-5	is	not listed on Canada ' s Ingredient Disclosure List .

Exposure Limits

** SECTION 16  - ADDITIONAL INFORMATION

MSDS Creation Date:   1/05/1999  Revision #3 Date: 10/09/2002

The information above	is believed to be accurate and represents the best

information currently	available to us . However , we make no warranty of
merchantabili ty or any other warranty, express or implied, with respect to

such information, and	we assume no liability resulting from its use . Users

should make their own	investigations to determine the suitability of the

information for their	particular purposes - In no way shall the company be
liable for any claims , losses , or damages of any third party or for lost

profits or any special, indirect, incidental, consequential or exemplary damages, howsoever arising, even if the company has been advised of the possibility of such damages.