CUSTOMER:    30257564

BATCH #: 020420009

ZONE   #*:   RW01

BARCODE  #:   9611019046201611109584

PRODUCT  NAME:   ONE-STROKE  AEROSOL

ORDER  # : STICKER  #:

0464@

020420082

MATERIAL   SAFETY  DATA  SHEET:   ONE-STROKE  AEROSOL

Page:   1

000-000000-

-5010

DATE  OF  ISSUE 9/11/2001

SUPERSEDES 1/19/1999

SECTION   I   -   GENERAL   INFORMATION

Chemical Name £ Synonyms N/A

Trade Name £ Synonyms

ONE-STROKE AEROSOL

Chemical Family:

METHYLENE CHLORIDE/ALCOHOL/SOLVENT SOLUTION

Formula Mixture —> X

Manufacturer's Name:

MANTEK, DIVISION OF NCH CORP.

Address:

BOX 152170 IRVING,

TEXAS 75015

Prepared By:

K Dickinson/Chemist

Product Code Number 5010

Emergency Phone Number 800-424-9300

SECTION II - HAZARDOUS INGREDIENTS

THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS

Chemical Name (Ingredients)

ETHYLENE GLYCOL MONOBUTYL ETHER

ALIPHATIC PETROLEUM DISTILLATE

METHANOL

METHYLENE CHLORIDE

N-METHYLPYRROLIDONE

PETROLEUM WAX

PROPANE

N-BUTANE

S DENOTES EXPOSURE LIMIT FOR FUMES

Hazard

IRR/COMB

IRR/COMB

FLAM/TOX

IRR/CARC

IRR

IRR

FLAM/AS PHY

FLAM/AS PHY

TLV

20 PPM 1 100 PPM 1 200 PPM 1 50 PPM 1 N/E 1 2 MG/M3 $1 2500 PPM 1 800 PPM 1

PEL

25 PPM 2 500 PPM 2 200 PPM 2 25 PPM 2 N/E 2 N/E 2 1000 PPM 2 N/E 2

STEL

N/E

N/E

250 PPM 1

125 PPM 1

N/E

N/E

N/E

N/E

CAS *

111-76-2

64742-88-7

67-56-1

75-09-2

872-50-4

64742-43-4

74-98-6

106-97-6

SECTION   Ila   -   NON-HAZARDOUS   INGREDIENTS

(NON-HAZARDOUS   INGREDIENT  NAMES  AND  CAS   NUMBERS  ARE   PROTECTED  UNDER NJ  TRADE)

Secret Registry #:   409363-5073P

SECTION   III   -   PHYSICAL  DATA

Boiling Point (F) :	105	Specific Gravity	(H20=l):	1.2

Vapor Pressure (MM HG) :	260	Color:	COLORLESS

Vapor Density (Air-1):	2	Odor:	METHYLENE CHLORIDE

PH t 100% :	N/A	Clarity:	HAZY

% Volatile by Volume:	>95	Evaporation Rate	(BU A/C=1) :	14

H2O Solubility:	EMULSIFIABLE	Viscosity:	SEMI-VISCOUS

SECTION IV - FIRE AND EXPLOSION HAZARD

Flash Point	Flammable Limits	LEL	UEL

100 F    / SETAFLASH	AL. PET. DIST./METHANOL	1%	36%

Extinguishing Media

X <--Foant         X <—Alcohol Foam       X <—CO2      X <—Dry Chemical       X <—Water Spray     <—Other

Special Fire Fighting Procedures:

FIREFIGHTERS SHOULD WEAR A SELF-CONTAINED BREATHING APPARATUS AND FULL PROTECTIVE GEAR. EXTINGUISHING MEDIA SHOULD BE CHOSEN BASED ON THE NATURE OF THE SURROUNDING FIRE. COOL FIRE-EXPOSED CONTAINERS WITH WATER SPRAY TO PREVENT BURSTING.

Unusual Fire and Explosion Hazards:

VAPORS ARE HEAVIER THAN AIR AND MAY TRAVEL TO DISTANT SOURCES OF IGNITION AND FLASHBACK. THERMAL DECOMPOSITION PRODUCES PHOSGENE, HYDROGEN CHLORIDE AND CHLORINE. FLAME EXTENSION IS 30 INCHES, BURNBACK IS 0 INCHES.

Aerosol Level (NFPA 30B) :    3

NFPA 704 Hazard Rating      (0=Insignificant l=Slight 2=Moderate  3-High  4-Extreme) 3  <--hedith      4  <—Flammability  0  <—Instability     <—Special

MATERIAL SAFETY DATA SHEET: ONE-STROKE AEROSOL	Page: 2

,
	SECTION V - HEALTH HAZARD DATA (Continued)

SKIN CONTACT:  CAUSES IRRITATION SEEN AS ITCHING, REDNESS, BURNING SENSATION, DEFATTING OF THE SKIN AND MAY CAUSE DERMATITIS. PRODUCT MAY BE ABSORBED

THROUGH THE SKIN IN HARMFUL AMOUNTS WITH EFFECTS SIMILAR TO INGESTION.

EYE CONTACT: CAUSES SEVERE IRRITATION SEEN STINGING, TEARING, REDNESS, SWELLING, EXCESS BLINKING AND TEAR PRODUCTION. PROLONGED CONTACT MAY CAUSE

TRANSIENT CORNEAL INJURY.

INHALATION: CAUSES IRRITATION, HEADACHE, NAUSEA, VOMITING, DROWSINESS, WEAKNESS, APATHY, SLURRED SPEECH, TREMOR, BLURRED VISION, STUTTERING AND

STAGGERING GAIT. WITH HIGH EXPOSURE LEVELS, DIZZINESS, LOSS OF CONCENTRATION, CENTRAL NERVOUS SYSTEM DEPRESSION (DRUNKENNESS), UNCONSCIOUSNESS,

CARDIAC ARRHYTHMIAS (IRREGULAR HEARTBEATS) AND POSSIBLY DEATH COULD OCCUR.  VAPORS CAN DISPLACE AIR AND CAN CAUSE VOMITING, PULMONARY IRRITATION,

DISORIENTATION, EXCITATION, ANESTHESIA AND ASPHYXIATION.

INGESTION: INGESTION AND SUBSEQUENT VOMITING OF THIS PRODUCT CAN LEAD TO ASPIRATION OF THE PRODUCT INTO THE LUNGS WHICH CAN CAUSE DAMAGE AND MAY BE

FATAL. CAUSES HEADACHE, DIZZINESS AND NAUSEA.  ALCOHOL MAY EXACERBATE THE EFFECTS OF OVEREXPOSURE. AVOID ALCOHOL CONSUMPTION. CONTAINS METHANOL WHICH

MAY CAUSE BLINDNESS IF INGESTED. MAY CAUSE CENTRAL NERVOUS SYSTEM EFFECTS SUCH WEAKNESS, LEG CRAMPS, BLURRED VISION, PAIN IN ABDOMEN AND LOWER BACK,

BLUE COLORING OF SKIN AND NAILS, AND IN EXTREME CASES COMA OR DEATH.

-Chronic (Long Term Exposure)

EXCESSIVE EXPOSURE OF METHYLENE CHLORIDE MAY CAUSE CARBOXYHEMOGLOBINEMIA, THEREBY IMPAIRING THE BLOOD'S ABILITY TO TRANSPORT SUFFICIENT OXYGEN. IARC

AND NTP HAVE LISTED METHYLENE CHLORIDE AS A POSSIBLE HUMAN CARCINOGEN BECAUSE STUDIES HAVE SHOWN AN INCREASE IN MALIGNANT TUMORS IN MICE AND BENIGN

TUMORS IN RATS. OTHER ANIMAL STUDIES AND SEVERAL EPIDEMIOLOGICAL HUMAN STUDIES HAVE FAILED TO SHOW ANY TUMORIGENIC RESPONSES. UPON REPEATED OR

PROLONGED CONTACT OF ETHYLENE GLYCOL MONBUTYL ETHER, THIS PRODUCT MAY CAUSE SEVERE IRRITATION AND MAY BE ABSORBED IN TOXIC AMOUNTS. EXCESSIVE

INHALATION OF VAPORS MAY CAUSE NASAL AND RESPIRATORY IRRITATION AND CENTRAL NERVOUS SYSTEM EFFECTS AND ALSO THE PRESENCE OF BLOOD IN THE URINE.

INGESTION MAY BE HARMFUL WITH SYMPTOMS SIMILAR TO INTOXICATION, THE SEVERITY OF WHICH DEPENDS ON THE AMOUNT INGESTED. ABSORPTION THROUGH THE SKIN OR

LUNGS FROM REPEATED/PROLONGED SKIN CONTACT OR INHALATION MAY CAUSE LIVER ABNORMALITIES, KIDNEY OR LUNG DAMAGE, SPLEEN DAMAGE, BLOOD ABNORMALITIES,

WEIGHT LOSS, ANOREXIA AND TESTIS DAMAGE MAY OCCUR. METHANOL CAN CAUSE BLINDNESS AS A RESULT OF EXPOSURE THROUGH INGESTION.

TARGET ORGANS: CENTRAL NERVOUS SYSTEM, HEART, LIVER, LUNGS, KIDNEYS, SPLEEN, BLOOD-FORMING ORGANS, TESTES, EYES, BRAIN AND PANCREAS.

MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE ARE PRE-EXISTING RESPIRATORY AND SKIN CONDITIONS SUCH AS ASTHMA, EMPHYSEMA AND DERMATITIS AND PRE-EXISTING

LIVER, KIDNEY, BLOOD, SPLEEN, TESTES, PANCREAS, EYE, HEART, BRAIN AND CENTRAL NSRVOUS SYSTEM DISEASES.

Primary Routes of Entry:    X <—Inhalation    X <—Ingestion     X <—Absorption

Emergency and First Aid Procedures: -Inhalation:

REMOVE FROM THE AREA TO FRESH AIR. IF NOT BREATHING, CLEAR THE AIRWAY AND START MOUTH TO MOUTH ARTIFICIAL RESPIRATION.  GET IMMEDIATE MEDICAL ATTENTION.

-Eye Contact:

IMMEDIATELY RINSE THE EYES WITH WATER. REMOVE ANY CONTACT LENSES AND CONTINUE FLUSHING FOR AT LEAST 15 MINUTES.  HOLD THE EYELIDS APART TO ENSURE RINSING OF THE ENTIRE SURFACE OF THE EYES AND LIDS WITH WATER. GET IMMEDIATE MEDICAL ATTENTION.

-Skin Contact:

WASH AFFECTED AREAS WITH LARGE AMOUNTS OF SOAP AND WATER FOR 15 MINUTES.  REMOVE CONTAMINATED CLOTHING AND SHOES. GET IMMEDIATE MEDICAL ATTENTION. WASH CLOTHING AND CLEAN SHOES BEFORE REUSE.

-Ingeation:

GIVE 3 TO 4 GLASSES OF WATER, BUT DO NOT INDUCE VOMITING.  IF VOMITING OCCURS, GIVE FLUIDS AGAIN. GET IMMEDIATE MEDICAL ATTENTION.  DO NOT GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS OR CONVULSING PERSON.

-Notes to Physician:

INGESTION AND SUBSEQUENT VOMITING OF THIS PRODUCT CAN LEAD TO ASPIRATION OF THE PRODUCT INTO THE LUNGS WHICH CAN CAUSE DAMAGE AND MAY BE FATAL. DEPENDING ON THE AMOUNT INGESTED AND RETAINED AS WELL AS THE TOXICITY OF THE PRODUCT, GASTRIC LAVAGE SHOULD BE CONSIDERED. KEEP PATIENT'S HEAD BELOW HIPS TO PREVENT PULMONARY ASPIRATION. IF COMATOSE, A CUFFED ENDOTRACHAEL TUBE WILL PREVENT ASPIRATION. CHLORINATED HYDROCARBONS MAY SENSITIZE THE HEART TO EPINEPHRINE AND OTHER CIRCULATING CATECHOLAMINES SO THAT ARRHYTHMIAS MAY OCCUR.  CAREFUL CONSIDERATION OF THIS POTENTIAL ADVERSE EFFECT SHOULD PRECEDE ADMINISTRATION OF EPINEPHRINE OR OTHER CARDIAC STIMULANTS AND THE SELECTION OF BRONCHODILATORS. METHANOL IS METABOLIZED TO FORMALDEHYDE AND FORMIC ACID.  THIS IN TURN, MAY CAUSE METABOLIC ACIDOSIS, VISUAL DISTURBANCES AND BLINDNESS.  BECAUSE METABOLISM MUST OCCUR BEFORE THE TOXIC EFFECTS, TOXIC SYMPTOMS MAY BE DELAYED FROM 6 TO 30 HOURS FOLLOWING INGESTION.  ETHANOL COMPETES FOR THE SAME METABOLIC PATHWAY AND HAS BEEN USED AS AN ANTIDOTE.  METHANOL IS EFFECTIVELY REMOVED BY HEMODIALYSIS. ORGANIC ABNORMALITIES RESULTING FROM OVEREXPOSURE TO ETHYLENE GLYCOL MONOBUTYL ETHER BY ANY ROUTE LIKELY WOULD BE AN ABNORMAL BLOOD PICTURE CHARACTERIZED BY ERYTHROPENIA, RETICULOCYTOSIS, GRANULOCYTOSIS AND LEUKOCYTOSIS.

SECTION VI - TOXICITY INFORMATION

Product Contains Chemicals Listed as Carcinogen or Potential Carcinogen By:

IARC—> Yes       NTP—> Yes	OSHA—> No	ACGIH—> No         OTHER—> No

ETHYLENE GLYCOL MONOBUTYL ETHER

ORL-RAT LDSO:  470 MG/KG     3.

SKN-RBT LDSO:  220 MG/KG     3.

IHL-RAT LC50:  450 PPM/4H    3.

IHL-RAT TCLO:  100 PPM/6H (6-18D PREG) :  TER  3.

ALTHOUGH THE LISTED EFFECTS WERE OBSERVED IN LABORATORY ANIMALS UPON INHLATION OF THIS MATERIAL, STUDIES HAVE ALSO SUGGESTED THAT THERE IS LITTLE POSSIBILITY OF SIGNIFICANT ADVERSE HEALTH EFFECTS IN HUMANS EXPOSED IN THE WORKPLACE TO LEVELS THAT COMPLY WITH THE OSHA PERMISSIBLE EXPOSURE LIMIT OF 25 PPM.  AT 100 PPM SOME SUBJECTIVE COMPLAINTS WERE SEEN IN SENSITIVE INDIVIDUALS, AND EVEN AT 200 PPM THE PRIMARY COMPLAINT WAS IRRITATION.  THIS APPEARS TO BE ONE OF THE FEW MATERIALS TO WHICH THE HUMAN IS MORE RESISTANT THAN THE USUAL EXPERIMENTAL ANIMALS.    4. INHALATION EXPOSURE TO 2-BUTOXYETHANOL PRODUCED SOME EVIDENCE OF CANCER IN MICE IN A LONG-TERM BIOASSAY, NO EVIDENCE OF CANCER IN MALE RATS AND EQUIVOCAL EVIDENCE IN FEMALE RATS.   5.

ALIPHATIC PETROLEUM DISTILLATE (PRODUCT MIXTURE): IHL-RAT LDSO: >700 PPM   6. SKN-RBT LDSO: >4 G/KG    6. ORL-RAT LDSO: >25 GM/KG  6.

METHYLENE CHLORIDE

ORL-HMN LDLO: 357 MG/KG	3.

ORL-RAT LDSO: 1600 MG/KG 3.
SKN-RBT SDT: 100 MG/24H MODERATE 3.
EYE-RBT SDT: 162 MG MODERATE 3.
IHL-RAT LC50: 52 G/M3	3.

IHL-HMN LDLO: 357 MG/KG	3.

IHL-HMN TCLO: 500 PPM/8H         3. TUMORIGENIC DATA

IHL-RAT TCLO: 3500 PPM/6H/2Y-I   3.

METHYLENE CHLORIDE HAS BEEN EVALUATED FOR POSSIBLE CANCER CAUSING EFFECTS IN LABORATORY ANIMALS.  INHALATION STUDIES AT CONCENTRATIONS OF 2000 AND 4000 PPM INCREASED THE INCIDENCE OF MALIGNANT LIVER AND LUNG TUMORS IN MICE.  THREE INHALATION STUDIES OF RATS HAVE SHOWN INCREASED INCIDENCE OF BENIGN MAMMARY GLAND TUMORS IN FEMALE RATS AT CONCENTRATIONS OF 500 PPM AND ABOVE AND INCREASES IN BENIGN MAMMARY GLAND TUMORS IN MALES AT CONCENTRATIONS OF 1SOO PPM AND ABOVE.  RATS EXPOSED TO 50 AND 200 PPM VIA INHALATION SHOWED NO INCREASED INCIDENCE OF TUMORS.  MICE AND RATS EXPOSED BY INGESTION AT LEVELS UP TO 250 MG/KG/DAY LIFETIME AND HAMSTERS EXPOSED VIA INHALATION TO CONCENTRATIONS UP TO 35OO PPM LIFETIME DID NOT SHOW AN INCREASED INCIDENCE OF TUMORS.  6.

EPIDEMIOLOGY STUDIES OF 751 HUMANS CHRONICALLY EXPOSED TO METHYLENE CHLORIDE IN THE WORKPLACE OF WHICH 252 WERE EXPOSED FOR A MINIMUM OF 20 YEARS DID NOT DEMONSTRATE ANY INCREASE IN DEATHS CAUSED BY CANCER OR CARDIAC PROBLEMS.  A SECOND STUDY OF 2227 WORKERS CONFIRMED THESE RESULTS.  6. MUTAGENICITY- METHYLENE CHLORIDE HAS BEEN EVALUATED FOR ITS POTENTIAL TO INDUCE GENOTOXIC EFFECTS BOTH IN VIVO AND IN VITRO SYSTEMS, WITH MIXED RESULTS.  BASED ON THIS EVIDENCE, METHYLENE CHLORIDE MAY BE CONSIDERED A WEAK MUTAGEN IN MAMMALIAN SYSTEMS.  6.

REPRODUCTIVE TOXICITY- LABORATORY ANIMAL STUDIES ON MICE, RATS AND RABBITS HAVE BEEN CONDUCTED TO EVALUATE THE POTENTIAL REPRODUCTIVE AND DEVELOPMENTAL EFFECTS OF METHYLENE CHLORIDE EXPOSURES.  METHLYENE CHLORIDE EXPOSURE HAS NOT BEEN SHOWN TO CAUSE TERATOGENIC EFFECTS (BIRTH DEFECTS) IN EXPERIMENTAL ANIMALS.  6.

MATERIAL SAFETY DATA SHEET: ONE-STROKE AEROSOL	Page: 3

SECTION VI - TOXICITY INFORMATION (Continued)

METHANOL

ORL-RAT-LD50: 5628 MG/KG     3.

IHL-RAT-LC50: 64,000 PPM/4H  3.

SKN-RBT-LD50: 15,800 MG/KG   3.

SKN-RBT: 20MG/24H MOD         3.

EYE-RBT: 100 MG/24H  MOD     3.

OVEREXPOSURE TO THIS MATERIAL (OR TO ITS COMPONENTS) HAS BEEN SUGGESTED AS
A CAUSE OF CENTRAL NERVOUS SYSTEM DAMAGE IN LABORATORY ANIMALS AND VISUAL
IMPAIRMENT IN HUMANS.	6.

METHANOL HAS CAUSED BIRTH DEFECTS IN LABORATORY ANIMALS, BUT ONLY WHEN INHALED AT EXTREMELY HIGH VAPOR CONCENTRATIONS. THE RELEVANCE OF THIS FINDING TO HUMANS IS UNCERTAIN. 6.

N-METHLPYRROLIDONE

ORL-RAT LD50: 4990 MG/KG     6.

IHL-RAT LC50: >5.1 MG/L/4HR  6.

SKN-RBT: MARKEDLY IRRITATING 6.

EYE-RBT! MARKEDLY IRRITATING 6.

IN ANIMAL STUDIES, N-METHYL PYRROLIDONE WAS EMBRYOTOXIC BY THE ORAL, DERMAL AND INTRAPERITONEAL ROUTES, BUT ONLY AFTER REPEATED HIGH DOSES THAT APPROACHED THE LD50 OR WERE MATERNALLY TOXIC. EMBRYOTOXICITY WITHOUT MATERNAL TOXICITY WAS OBSERVED AT A HIGH CONCENTRATION IN ONE RAT INHALATION STUDY, BUT NOT IN OTHERS. TESTICULAR EFFECTS IN RATS WERE NOTED AFTER REPEATED, HIGH-DOSE ORAL AND INHALATION EXPOSURES.   6.

PETROLEUM WAX

ALTHOUGH THIS SPECIFIC PRODUT HAS NOT BEEN TESTED IN LABORATORY ANIMALS,

NO ADVERSE HEALTH EFFECTS HAVE BEEN IDENTIFIED IN TOXICOLOGICAL TESTS USING SIMILAR HIGHLY REFINED PETROLEUM WAXES.  6.

PROPANE NO TOXICITY DATA AVAILABLE.

N-BUTANE

IHL-RAT   LC50:   658   G/M3/4H     3.

IHL-MUS   LC50:    680   G/M3/4H      3.

HUMAN  VOLUNTEERS   EXPOSED  REPEATEDLY   TO   GASES   OF   SIMILAR  HYDROCARON  MIXTURES   RANGING   FROM   250   TO   1000   PPM  EXHIBITED   NO   CARDIAC   OR   PULMONARY   FUNCTION ABNORMALITIES.       6.

SECTION VII - REACTIVITY DATA

Stability:        X <--Stable     <—Unstable

Conditions to Avoid:

AVOID HEAT, HOT SURFACES, SPARKS AND OPEN FLAMES.

Incompatibility (Materials to Avoid) :

STRONG OXIDIZING AGENTS SUCH AS CHLORINE BLEACH AND CONCENTRATED HYDROGEN PEROXIDE, ACIDS, ALKALIES, AMINES, AND WATER.  METALS SUCH AS ALUMINUM, MAGNESIUM, POTASSIUM, LITHIUM AND ZINC AND PLASTICS AND RUBBERS.

Hazardous Decomposition Products:

OXIDES OF CARBON AND NITROGEN AND OTHER UNIDENTIFIED ORGANIC COMPOUNDS.  PHOSGENE, HYDROGEN CHLORIDE AND CHLORINE.

Hazardous Polymerization:	<—May Occur     X <—Will Not Occur

Conditions to Avoid: N/A.

SECTION VIII - SPILL OR LEAK PROCEDURES

Steps to be Taken if Material is Released or Spilled:

DUE TO THE NATURE OF THE AEROSOL PACKAGING, A LARGE SPILL IS UNLIKELY. FOR A SMALL SPILL, VENTILATE THE AREA AND ABSORB WITH AN INERT MATERIAL AND TRANSFER ALL MATERIAL  INTO A PROPERLY LABELED CONTAINER FOR DISPOSAL. WEAR APPROPRIATE PROTECTIVE CLOTHING.

Waste Disposal Method(s):

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL, STATE, AND LOCAL REGULATIONS.  TYPICAL DISPOSAL IS TO WRAP THE EMPTY AEROSOL CONTAINER IN SERVERAL LAYERS OF NEWSPAPER AND DISPOSE OF IN THE TRASH. AEROSOL RECYCLING PROGRAMS ARE AVAILABLE IN MANY AREAS.  DO NOT PUNCTURE OR INCINERATE THIS CONTAINER.

Neutralizing Agent:

NONE KNOWN.

SECTION IX - SPECIAL PROTECTION INFORMATION

Required Ventilation:

LOCAL VENTILATION IS RECOMMENDED TO CONTROL EXPOSURE FROM OPERATIONS THAT CAN GENERATE MISTS OR VAPORS.

Respiratory Protection:

A NIOSH/MSHA APPROVED RESPIRATOR IN POORLY VENTILATED AREAS AND/OR FOR EXPOSURE ABOVE THE ACGIH TLV OR OSHA PEL OR WHERE MISTING EXISTS.

Glove Protection:

NEOPRENE OR NITRILE RUBBER GLOVES SHOULD BE WORN.

Eye Protection:

CHEMICAL GOGGLES AND A FACE SHIELD SHOULD BE WORN.

Other Protection:

WEAR PROTECTIVE CLOTHING WHEN HANDLING.

SECTION X - STORAGE AND HANDLING INFORMATION

Storage Temperature:    Indoors—> X	Outdoors—>	Heated—>	Refrigerated—>

Minimum Temperature: 32 F   Maximum Temperature: 100 F

Precautions to be Taken in Handling and Storing:

USE WITH CAUTION AROUND HEAT, SPARKS, PILOT  LIGHTS, STATIC ELECTRICITY AND OPEN FLAME. USE WITH CAUTION AROUND HEAT, SPARKS, PILOT LIGHTS, STATIC ELECTRICITY AND OPEN FLAME.

Other Precautions:

KEEP OUT OF REACH OF CHILDREN. READ THE ENTIRE LABEL BEFORE USING THE PRODUCT.

MATERIAL SAFETY DATA SHEET: ONE-STROKE AEROSOL	Page: 4

SECTION XI - REGULATORY INFORMATION (Continued)

| Chemical Name | CAS Number | Upper | % Limit |
| --- | --- | --- | --- |
| METHYLENE CHLORIDE METHANOL | 75-09-2 67-56-1 | 70 5 |  |
| GLYCOL ETHERS | N/A | 5 |  |
| N-METHYLPYRROLIDONE | 872-50-4 | 5 |  |

ingredients listed above are subject to the reporting requirements of 313 of Title III of the Superfund Amendments and Reauthorization Act o: i    and 40 CFR part 372.

Please call 1-800-527-9919 for additional information if you are a California customer. This MSDS is not intended for users in the state of California.

SECTION XII - REFERENCES

- THRESHOLD LIMIT VALUES FOR CHEMICAL SUBSTANCES AND PHYSICAL AGENTS AND BIOLOGICAL EXPOSURE INDICES, ACGIH, 2001.

- OSHA PEL.

- REGISTRY OF TOXIC EFFECTS OF CHEMICAL SUBSTANCES, CCINFQpisc, 2001.

- DISSOLVING THE MYTHS ABOUT EGBE AND HEALTH, CMA, GLYCOL ETHER PANEL,
1989.

- CHEMICAL MANUFACTURERS ASSOCIATION, NOVEMBER 9, 1998.

- VENDOR'S MSDS.

ALL COMPONENTS IN THIS PRODUCT CAN BE FOUND IN THE CURRENT TSCA INVENTORY.

_	 IRR:IRRITANT, FLAM/FLAMM:FLAMMABLE, COMB:COMBUSTIBLE,

CORR: CORROSIVE  CARC:CARCINOGENIC, TOX:TOXIC, N/A:NOT APPLICABLE, N/E:NOT ESTABLISHED,  COC: CLEVELAND OPEN CUP, PMCC: PENSKY-MARTIN CLOSED CUP, TCC.'TAGLIABOE CLOSED CUP, LEL:LOWER EXPLOSION LIMIT, UEL:UPPER EXPLOSION LIMIT,  NFPA:NATIONAL FIRE PROTECTION ASSOCIATION, IARC: INTERNATIONAL AGE1 FOR THE RESEARCH ON CANCER, NTPlNATIONAL TOXICOLOGY PROGRAM, OSHA:OCCUPATIONAL SAFETY S HEALTH ADMINISTRATION, ACGIH:AMERICAN CONFERENCE OF GOVERNMENTAL INDUSTRIAL HYGIENISTS, TLV:THRESHOLD LIMIT VALUE, PEL:PERMISSIBLE EXPOSURE LIMIT, STEL:SHORT-TERM EXPOSURE LIMIT, MLD:MILD, MOD:MODERATE, SEV:SEVERE, MUT:MUTAGENIC, ASPHYX:ASPHYXIANT, PNOS: PARTICULATES (INSOLUBLE) NOT OTHERWISE SPECIFIED, SDT:STANDARD DRAIZE TEST, ORL: ORAL, HMN: HUMAN, IHL: INHALATION

THE INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORMULATION.  HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLII REGARDING THE ACCURACY OF THESE DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF.

MANTEK, DIVISION OF NCH CORP. assumes no responsibility for personal injury or property damage caused by the use, storage, or disposal of the prodi in a manner not recommended on the product label.  Users assume all risks associated with such unrecommended use, storage, or disposal of the product.

/"—	SECTION V - HEALTH HAZARD DATA

Ti.__ohold Limit Value:

NOT ESTABLISHED FOR MIXTURE.  SEE SECTION II.

Effects of Overexposure:

-Acute (Short Term Exposure)