TERRAGREEN STRIPPER

Product number: 361002 / 361052

MATERIAL SAFETY DATA SHEET

1. Product and Company Identification

Material name Product number Revision date Company information

Emergency General information

2. Hazards Identification

Emergency overview

Potential short term health effects Routes of exposure Eyes Skin

Inhalation Ingestion

Target organs

TERRAGREEN STRIPPER

361002/361052 23-Jul-2007

Brulin & Company, Inc. P.O. Box 270 Indianapolis, IN 46206 US www.Brulin.com

CHEMTREC 1-800-424-9300

Phone: 317-923-3211 Fax:317-925-4596

WARNING

Irritating to eyes, respiratory system and skin. Prolonged exposure may cause chronic

effects. This product is not considered to be a carcinogen by IARC, ACGIH, NTP, or

OSHA.

Inhalation.

Irritating to eyes. Avoid contact with eyes.

Health injuries are not known or expected under normal use. Prolonged and/or repeated skin contact may result in mild irritation or redness.

Prolonged or excessive inhalation may cause respiratory tract irritation.

Harmful if swallowed. Ingestion may produce burns to the lips, oral cavity, upper airway, esophagus and possibly the digestive tract. Eyes. Respiratory system. Skin.

3. Composition / Information on Ingredients

| Hazardous component(s) | CAS# | Percent |
| --- | --- | --- |
| Propanol, 1-(2-butoxy-1-methylethoxy)- | 29911-28-2 | <20 |
| Isopropyl alcohol | 67-63-0 | <2.5 |
| Benzyl alcohol | 100-51-6 | < 10 |
| Ethanolamine | 141-43-5 | <10 |

4. First Aid Measures

First aid procedures Eye contact

Skin contact

Inhalation Ingestion General advice

Immediately flush eyes with plenty of water for at least 20 minutes. Get medical attention immediately.

Immediately flush skin with running water for at least 20 minutes. Remove and isolate contaminated clothing and shoes. Launder contaminated clothing before reuse. Get medical attention if irritation develops or persists.

Move to fresh air. If breathing is difficult, give oxygen. Call a physician if symptoms develop or persist.

Give several glasses of water to dilute contents of stomach and call a physician. Have victim rinse mouth thoroughly with water. Do not induce vomiting without medical advice.

If you feel unwell, seek medical advice (show the label where possible).

5. Fire Fighting Measures

Extinguishing media

Suitable extinguishing media        Small Fires: Dry chemical, CO2, water spray or regular foam.

Large Fires: Water spray, fog or regular foam.

6. Accidental Release Measures

Personal precautions	Do not touch or walk through spilled material. Do not touch damaged containers or spilled

material unless wearing appropriate protective clothing.

Methods for containment	Stop the flow of material, if this is without risk. Dike the spilled material, where this is

possible. Prevent entry into waterways, sewers, basements or confined areas.

Methods for cleaning up	Never return spills in original containers for re-use.

Large Spills: Absorb with earth, sand or other non-combustible material and transfer to containers for later disposal. Dike far ahead of liquid spill for later disposal.

Small Spills: Wipe up with absorbent material (e.g. cloth, fleece). Clean contaminated surface thoroughly. Absorb with earth, sand or other non-combustible material and transfer to containers for later disposal.

7. Handling and Storage

Handling	Handle and open container with care. Avoid contact with skin and eyes.

Storage	Keep out of the reach of children. Keep this material away from food, drink and animal

feed. Keep at temperatures between 4 and 43°C.

8.	Exposure Controls / Personal Protection

Exposure guidelines

ACGIH - Threshold Limits Values - Short Term Exposure Limits (TLV-STEL)

Ethanolamine	141-43-5	6 Ppm STEL

Isopropyl alcohol	67-63-0	400 Ppm STEL

ACGIH - Threshold Limits Values - Time Weighted Averages (TLV-TWA)

Ethanolamine	141-43-5	3 Ppm TWA

Isopropyl alcohol	67-63-0	200 Ppm TWA

ACGIH - Threshold Limits Values - TLV Basis - Critical Effects

Ethanolamine	141-43-5	eye and skin irritation

Isopropyl alcohol	67-63-0	CMS impairment; eye and upper respiratory tract irritation

OSHA - Final PELs - Time Weighted Averages (TWAs)

Ethanolamine	141-43-5	3 Ppm TWA; 6 mg/m3 TWA

Isopropyl alcohol	67-63-0	400 Ppm TWA; 980 mg/m3 TWA

Personal protective equipment

Eye / face protection	Wear chemical goggles.

Skin protection	Protective gloves.

Respiratory protection	Not normally needed. In case of mist, spray or aerosol exposure wear suitable personal

respiratory protection and protective suit.

General hygeine considerations   Handle in accordance with good industrial hygiene and safety practice.

9.	Physical & Chemical Properties

Appearance	Not available

Color	green

Form	Liquid.

Odor	Mild organic

Odor threshold	Not available

pH	10.8

Freezing point	Not available

Boiling point	212°F

Flash point	> 212 °F Pensky-Martens Closed Cup

Flammability limits in air, lower, %      Not available by volume

Flammability limits in air, upper, %      Not available by volume

Vapor pressure	Not available

Vapor density	Not available

Specific gravity	1.014

Solubility (H2O)	100%

VOC (Weight %)	7.25 % estimated

10.	Chemical Stability & Reactivity Information

Chemical stability	Stable at normal conditions.

Conditions to avoid	Do not freeze.

Incompatible materials	Strong oxidizing agents. Strong acids.

Possibility of hazardous reactions	Will not occur.

11.	Toxicological Information

Acute effects	Acute LD50: 9726 mg/kg estimated, Rat, Oral

Acute LD50: 32212 mg/kg estimated, Rat, Dermal irritation of mucous membranes

Local effects	Irritating to eyes. Irritating to respiratory system.

Chronic effects	Repeated absorption may cause disorder of central nervous system, liver, kidneys and

blood. Prolonged exposure may cause chronic effects.

Further information	Symptoms may be delayed.

12. Ecological Information

Ecotoxicity	Components of this product have been identified as having potential environmental

concerns.

13. Disposal Considerations

Disposal instructions	Dispose in accordance with all applicable regulations. This product, in its present state,

when discarded or disposed of, is not a hazardous waste according to Federal regulations (40 CFR 261.4 (b)(4)). Under RCRA, it is the responsibility of the user of the product to determine, at the time of disposal, whether the product meets RCRA criteria for hazardous waste.

14. Transport Information

Department of Transportation (DOT) Requirements

Not regulated as dangerous goods.

Department of Transportation (DOT) Requirements NON-BULK

Not regulated as dangerous goods.

IATA

Not regulated as dangerous goods.

IMDG

Not regulated as dangerous goods.

15. Regulatory Information

US federal regulations	All components are on the U.S. EPA TSCA Inventory List.

CERCLA/SARA Hazardous Substances - Not applicable. CERCLA/SARA - Section 313 - Emission Reporting

Isopropyl alcohol	67-63-0	1.0 % de minimis concentration (only if manufactured by the strong acid process, no

supplier notification)

Occupational Safety and Health Administration (OSHA)

29 CFR 1910.1200 hazardous        Yes chemical

CERCLA (Superfund) reportable quantity

None Superfund Amendments and Reauthorization Act of 1986 (SARA)

Hazard categories	Immediate Hazard - Yes

Delayed Hazard - Yes Fire Hazard - No Pressure Hazard - No Reactivity Hazard - No

Section 302 extremely	No

hazardous substance

Section 311 hazardous chemical Yes

International regulations State regulations

16. Other Information

The product is classified and labelled in accordance with EC directives or respective national laws.

This product does not contain a chemical known to the State of California to cause cancer, birth defects or other reproductive harm.

HMIS rating

/      2 0

Physical Hazard       0

Personal Protection

B

NFPA codes

Prepared date Superseded date

Disclaimer

23-Jul-2007 09:48:36 08-May-2007 07:28:05

The information provided in this Safety Data Sheet is correct to the best of our knowledge, information and belief at the date of its publication. The information relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any process, unless specified in the text. The information given is designed only as a guidance for safe handling, use, processing, storage, transportation, disposal and release.