From: Dawn Infotrac 1-352-323-8160

Date: 1/20/100 Time: 3:50:40 PM

Page 3 of 6

MATERIAL SAFETY DATA SHEET

(SIMILAR TO OSHA FORM 174}

*^f \ff wBCTr^Dolicibl.

Not Esl -Not Established P«^> -Proprietary

oduct Name: Break Thru Floor Slripper pplier's Name: National Chemical Laboratories

SECTION 1-MANUFACTURER/DISTRIBUTOR INFORMATION

of PA, Inc. Supplier's Address: 401 N. 10th Street

Philadelphia, PA 19123

Chemical Name: Blended Synthetic Detergents and Solvents Chemical Family: Floor Finish Stripper Formula: N.A. Blended Product Emergency Phone #:    1 (215) 922-1200

HMIS Hazard Code

| Health | 2 |
| --- | --- |
| Flammability | 0 |
| Reactivity | 0 |
| Other | 0 |

SECTION 2-HAZARDOUS INGREDIENTS AND FIVE MAJOR COMPONENTS

| CHEMCAL NAME | CAS NO. %C0nc.,*«. |  |  | R«>9» HuOTl FMIng« (TLV/PEL) |
| --- | --- | --- | --- | --- |
| Ethylene Diamine Tetraacetic Acid, Tetra Sodium Salt 2 (2 Butoxy Ethanol) 1 -<2-Butoxyelhoxy)-2-Propanol Monoetnanolamine Water | 64-02-8 1 1 1 -76-2 124-16-3 ' 141-43-5 7732-18-5 |  | PROP. PROP. PROP. PROP. PROP. | Not Est. 25ppm Not Est. 3ppm None |
| SECTION 3-PHYSICAL DATA |  |  | SECTION | 4-FIRE AND EXPLOSION DATA |

Boil ing Point (F):  212

Vapor Pressure: = To Water

Vapor Density: = To Water

Solubility In Water: Complete

Appearance: Clear, Thin, Blue

Liquid

Odor: Sassafras Type

Specific Gravity: 1.017 % Volatiles By Weight: 91 Evaporation Rate: - To Water pH: 12.5

Flash Point (F): None To Boiling (TOC) Flammable Limits: Not Est. Unusual Fire & Explosion Hazards: Heated product may liberate toxic fumes. Wear self contained breathing apparatus.

SECTION 5-HEALTH HAZARD.DATA

Route(s) of Entry: lnhalation:No   Skin:Yes   lngestion:Yes	Threshold Limit Value: Not Est.

*ects of Overexposure: Causes skin and eye irritation. Harmful i( swallowed. May cause systemic intoxication, .ergency & First Aid Procedures: For Eyes: Flush with water for at least fifteen minutes. Get medical aid. For Skin: Flush .thoroughly with water, if irritation persists get medical aid. If Ingested: Do not induce vomiting. Give milk, egg white, gelatin, or if ne of these are available give large quantities rjf water. Get medical aid. jnditions Aggravated by Exposure:   May aggravate existing skin and bronchial conditions.

Listed Carcinogen, Mutagen, Teratogen (Ingredient, Source, Amount): None

SECTION 6-REACTIVITY DATA

SECTION 7-SPILL OR LEAK PROCEDURES

Stability: Stable

Conditions to Avoid: None

Incompatibility: Acids and Oxidizers

Hazardous Decomposition Products: Carbon Oxides and

Amines

Hazardous Polymerization: Will not occur.

Steps to be taken if material is released or spilled:

Recontainerize by mopping, wet vacuum, or using a suitable

absorbent. Rinse remaining material to drains. Be cautious of

slippery floors.

Waste disposal method: Follow local, state, and federal

regulations.

SECTION 8-SPECIAL PROTECTION INFORMATION

SECTION 9-SPECIAL PRECAUTIONS

Respiratory Protection: Generally not required.

Ventilation: Local.

Protective Gloves: Rubber or latex.

Eye Protection: Goggles or safety glasses with shields

Other Protective Equipment: Rubber boots should be worn if

working in standing solutions

Precautions to be taken in handling and storage: Store

between 40°and 120 ° F.

Other Precautions:   Wash all contaminated clothing before reuse. Keep out of reach of children. For sale to, use and storage by service personnel only.

C

"" i inlormation contained in Ihis MSDS was obtained from current and reliable sources, however the data is provided without any warranty, expressed

•iplied, regarding its correctness or accuracy. Sbca the conditions or handling, storage and disposal of this product are beyond the control of this
Ljmpany, it is not responsible tor toss, injury and expanse arising out ot ttw products improper use. No warranty, expressed or interred, regarding the
product described in this MSDS shall be created or inferred by any statement in this MSDS. Various government agencies- may have specific regula
tions regarding the transportation, handling, storage, use or disposal of this product which may not be covered by Ihis MSDS. The user is responsible
lor full compliance.