Reckitt Benc-^iser

Page 7 of 10

Tue May 25 2004 12:04:13 EDT

Material Safety Data Sheet

~

| Section 1. Product and Company Identification |  |  |  |
| --- | --- | --- | --- |
| Product Name | EASY-OFF® FUME FREE MAX™ OVEN CLEANER (AEROSOL) |  |  |
|  |  | MSDS# Not available. |  |
| Product Description | Non-corrosive, no caustic fumes. | Validation Date 3/18/2004 |  |
|  |  | Print Date 3/23/2004 |  |
| Manufacturer | Reckitt Benckiser North America, Inc. Morris Corporate Center IV 399 Interpace Parkway (P.O. Box 225) Parsippany, N.J. 07054-0225 |  |  |
|  |  | to case of 7p¦ppnnni=- Rnri-228-47 | 22 300 Canada |
|  |  | Emergency: |  |
| Product Identifier | Not available. | Transportation p.hpmtrf3^' 1 -80n-4?4-c |  |
| Item Number | 363915 | Emergencies: /i i o o pan^al |  |
|  |  | Outside the U.S & (North America), call: 703-527-3887 |  |
| Formula Number | F/F 363915 (Cone. 110394) Lemon Scent. |  |  |
| UPC Number | 62338-00178 (16 oz. / 453 gm.) Aerosol. |  |  |

1

| Section 2. | Composition and Information on Ingredients |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name |  | CAS# | % by Weight | Exposure Limits : TLV/PEL |  |
| 1) DIETHYLENE GLYCOL MONOBUTYL ETHER 2) MONOETHANOLAMINE 3) ISOBUTANE |  | 112-34-5 141-43-5 75-28-5 | 7-11 1-5 1-5 | Not available. TWA: 7.5 STEL: 15 (mg/m3) from ACGIH (TLV) TWA: 3 STEL: 6 (ppm) from ACGIH (TLV) TWA: 6 STEL: 15 (mg/m3) from OSHA TWA: 3 (ppm) from OSHA TWA: 1900 (mg/m3) from ACGIH (TLV) TWA: 800 (ppm) from ACGIH (TLV) |  |
| Section 3. | Hazards Identification |  |  |  |  |
| Emergency Overview CAUTION: CONTENTS UNDER PRESSURE. Causes eye irritation. Contains monoethanolamine OUT OF REACH OF CHILDREN. |  |  |  |  | KEEP |
| Section 4. | First Aid Measures |  |  |  |  |
| Eye Contact | In case of eye contact, immediately rinse eyes with plenty of water, remove any contact lenses and continue to rinse eyes with water for at least 15 minutes. If irritation persists, get medical attention. |  |  |  |  |
| Skin Contact | In keeping with good hygienic practices, wash exposed areas thoroughly with soap and water. If persists, get medical attention. |  |  |  | irritation |
| Inhalation | Remove person to fresh air. |  |  |  |  |

| Ingestion | Rinse mouth with water. Contact a physician or poison control center if symptoms develop. NEVER unconscious person anything to ingest. |  |  | give an |
| --- | --- | --- | --- | --- |
| EASY-OFF® FUME FREE MAX™ OVEN CLEANER (AEROSOL) |  |  | Page Number: 2 |  |
| Section 5. Fire and Explosion Data |  |  |  |  |
| Fl;im inability |  | See Section 14 for any Shipping Classifications. |  |  |
| Flash Point |  | Not applicable. Flame extension less than 18 inches or 45 cm. |  |  |
| Explosive Limits in Air |  | Not available. |  |  |
| Products of Combustion |  | Not available. |  |  |
| Fire and Explosion Hazards |  | CONTENTS UNDER PRESSURE. Do not use or store near hear or open flame. Do not puncture or incinerate. Exposure to temperatures above 120° F (50° C) may cause bursting. |  |  |
| Fire Fighting Media and Instructions Special Fire Fighting Instructions |  | Not a fire hazard. Select media suitable for surrounding fire. Use water spray to Wear full protective equipment: eye/face (shield), body (full bunker gear), shielding for venting, rupturing or bursting containers. | cool fire-exposed containers, respiratory (SCBA). Provide |  |

J

| Section 6. Accidental Release Measures |  |
| --- | --- |
| Accidental Spill Small spill: Soak up with liquid absorbent material. Rinse area well to avoid slippery condition. Large spills from ruptured or leaking containers: Eliminate all sources of ignition and ventilate equipment rated for use in flammable environments. Spill should be diked, contained, collected metal containers for later disposal according to local, state or federal regulations. | the area with and stored in |
| 1 Section 7. Handling and Storage |  |

CAUTION: Contents under pressure.  Causes eye irritation.  Contains monoethanolamine.    Do not puncture or incinerate container.

Do not expose to heat or store at temperatures above 120 °F (50° C).    Avoid freezing.    Store in an area inaccessable to children and pets.  KEEP OUT OF REACH OF CHILDREN.

Handling and Storage

|  | Section 8. Exposure | Controls/Personal Protection |  |
| --- | --- | --- | --- |
|  | Ventilation Requirements | No specific controls are needed. If user operations generate fume, vapor or mist, such as a operation, use ventilation to keep exposure to airborne contaminants below the exposure limit. | fire or clean-up |
|  | Eye Protection | None required under normal use conditions. Emergency responders should wear full eye and face protection. |  |
|  | Skin Protection | None required. Emergency responders should wear impermeable gloves. |  |
|  | Respiratory Protection | None required under normal use conditions. Emergency responders should wear suitable respiratory protection where long term exposure to large areas of spilled liquid is possible such as a major spill, a fire or a clean-up operation. |  |
|  | Other Protection | None required. Emergency responders should wear impermeable clothing and footwear when situation where contact with the liquid is possible. | responding to a |
|  | Work/Hygienic Practices | Wash thoroughly with soap and water after handling. |  |
|  | Section 9. Physical and Chemical Properties |  |  |
|  | Description | Liquid. (White foam when sprayed.) odor Lemon ¦jke |  |
|  | pH |  |  |
|  |  | 12.25 [Basic.] Color White foam when sprayed. |  |
| ' | Boiling/Condensation Point |  |  |
|  |  | Not applicable. |  |
|  | Specific Gravity | 1.03-1. 06 @25/25°C(conc) |  |
|  | Continued on Next | Page |  |

EASY-OFF® FUME FREE MAX™ OVEN	Page Number: 3

CLEANER (AEROSOL)

I Vapor Pressure	Not determined.

Vapor Density	Not determined.

Viscosity	Not available.

Solubility	> 95% @ 70 °F

Physical Chemical Comments  Not available.

Section 10. Stability and Reactivity Data

Chemical Stability	The product is stable.

Conditions of Instability	None known.

Incompatibility with Various    Incompatible with acids. Substances

Hazardous Decomposition         None known. Products

Hazardous Polymerization        Will not occur.

Section 11. Toxicological Information

Exposure effects

Eye Contact	Causes eye irritation.

"

Skin Contact	Not a primary skin irritant.  May be irritating to sensitive skin. Avoid prolonged contact, wear rubber gloves.

Inhalation	Not toxic by inhalation.

Ingestion	Not orally toxic.

Carcinogenicity	Not listed as carcinogenic by OSHA, NTP or IARC.

Section 12. Ecological Information

Ecotoxicity	Not available.

Section 13. Disposal Considerations

Waste Disposal	Discard individual empty container in trash.

Large quantity disposal: Degas cans and discharge liquid contents to sanitary sewer or accumulate intact cans for transport to shredder, approved incinerator, or other facility in accord with local, state and federal regulations.

Section 14. Transport Information

DOT Classification	ORM-D

'

Proper Shipping Name	Consumer Commodity

DOT Identification Number      Not applicable.

Reckitt Benc'luser

Page 10 of 10

Tue Hay 25 2004 12:06:35 EDT

Reckitt Benc'luser

Page 10 of 10

Tue Hay 25 2004 12:06:35 EDT

| EASY-OFF® FUME FREE MAX™ OVEN Page Number: 4 CLEANER (AEROSOL) |  |  |
| --- | --- | --- |
| * | 1 Maritime Transportation | Not applicable. |
|  | Hazardous Substances Reportable Quantity | Not applicable. |
|  | Special Provisions for Transport | ORM-D: For US Domestic Shipments Only |
|  | TDG Classification | Consumer Commodity - Transborder Consignment |
|  | ADR Classification | Not applicable. |
|  | IMDG Classification | Aerosols, class 2.2 UN1950 - LTD QTY. |
|  | IATA Classification | Consumer Commodity, 9 ID8000 |
|  | Section 15. Regulatory Information |  |
|  | Federal and State SARA Title III, Section 313 Toxic Chemical Notification & Release Reporting: Regulations 1)DIETHYLENEGLYCOL MONOBUTYL ETHER CAS #: 1 1 2-34-5 5 % California Proposition 65: This product contains the following ingredients which require a warning under the Safe Drinking Water & Toxic Enforcement Act: None |  |
|  | Other Classifications | WHMIS (Canada) CLASS E: Corrosive liquid. |
| ' ¦ | \ |  |
|  | Section 16. Other Information |  |
|  | HMIS (U.S.A.) | ¦2 National Fire Protection /4\ FireHaiar<l |
|  |  | ° $ Speciflc hazard |
|  |  | Personal Protection ; D \/ |
|  |  | NFPA Aerosol Level 1 . per NFPA 30B |
|  | Validated by Product Safety on | 3/18/2004. Printed 3/23/2004. |
|  | Notice to Reader |  |
|  | To the best of our knowledge, the information contained herein is accurate. However, neither the above named supplier nor any of its subsidiaries assumes any liability whatsoever for the accuracy or completeness of the information contained herein. Final determination of suitability of any material is the sole responsibility of the user. All materials may present unknown hazards and should be used with caution. Although certain hazards are described herein, we cannot guarantee thai these are the only hazards that exist. |  |

| EASY-OFF® FUME FREE MAX™ OVEN Page Number: 4 CLEANER (AEROSOL) |  |  |
| --- | --- | --- |
| * | 1 Maritime Transportation | Not applicable. |
|  | Hazardous Substances Reportable Quantity | Not applicable. |
|  | Special Provisions for Transport | ORM-D: For US Domestic Shipments Only |
|  | TDG Classification | Consumer Commodity - Transborder Consignment |
|  | ADR Classification | Not applicable. |
|  | IMDG Classification | Aerosols, class 2.2 UN1950 - LTD QTY. |
|  | IATA Classification | Consumer Commodity, 9 ID8000 |
|  | Section 15. Regulatory Information |  |
|  | Federal and State SARA Title III, Section 313 Toxic Chemical Notification & Release Reporting: Regulations 1)DIETHYLENEGLYCOL MONOBUTYL ETHER CAS #: 1 1 2-34-5 5 % California Proposition 65: This product contains the following ingredients which require a warning under the Safe Drinking Water & Toxic Enforcement Act: None |  |
|  | Other Classifications | WHMIS (Canada) CLASS E: Corrosive liquid. |
| ' ¦ | \ |  |
|  | Section 16. Other Information |  |
|  | HMIS (U.S.A.) | ¦2 National Fire Protection /4\ FireHaiar<l |
|  |  | ° $ Speciflc hazard |
|  |  | Personal Protection ; D \/ |
|  |  | NFPA Aerosol Level 1 . per NFPA 30B |
|  | Validated by Product Safety on | 3/18/2004. Printed 3/23/2004. |
|  | Notice to Reader |  |
|  | To the best of our knowledge, the information contained herein is accurate. However, neither the above named supplier nor any of its subsidiaries assumes any liability whatsoever for the accuracy or completeness of the information contained herein. Final determination of suitability of any material is the sole responsibility of the user. All materials may present unknown hazards and should be used with caution. Although certain hazards are described herein, we cannot guarantee thai these are the only hazards that exist. |  |

Packing Group	. Not applicable.